| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, Sweden, 11 July – 20 July, 2012 | Document: JCTVC-J0305 WG11 Number: m25633 |
| --- | --- |

| Title: | On Gradual Temporal Layer Access |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes gradual temporal layer access (GTLA) pictures. It is asserted that the GTLA pictures provide more flexibility in selection of reference pictures while providing temporal layer switching functionality. It is asserted that gradual temporal layer access functionality is useful in allowing selection of desired frame rate in a step-by-step manner. This document proposes to keep TLA pictures functionality of the HEVC as it is and to add GTLA pictures in addition.

# Introduction

HEVC Draft 7 [1] includes the temporal layer access (TLA) picture type. A corresponding NAL unit type (3) is also included. The TLA picture type can be used to indicate temporal layer switching points.

The Temporal Layer Access Picture is defined as follows [1]:

  - temporal layer access (TLA) unit: An access unit in which the coded picture is a TLA picture.

  - temporal layer access (TLA) picture: A coded picture for which each slice has nal_unit_type equal to 3.

When the value of nal_unit_type is equal to 3 for all VCL NAL units of a particular picture, that particular picture is referred to as a TLA picture. A TLA picture and all coded pictures with temporal_id greater than or equal to the temporal_id of the TLA picture that follow the TLA picture in decoding order shall not use inter prediction from any picture with temporal_id greater than or equal to the temporal_id of the TLA picture that precedes the TLA picture in decoding order. A TLA picture shall not be TFD picture; hence TLA picture is also referred to as non-TFD TLA picture.

We define a new type of picture Gradual Temporal Layer Access Picture as follows.

  - Gradual temporal layer access (GTLA) unit: An access unit in which the coded picture is a GTLA picture.

  - Gradual temporal layer access (GTLA) picture: A coded picture for which each slice has nal_unit_type equal to 9.

When the value of nal_unit_type is equal to 9 for all VCL NAL units of a particular picture, that particular picture is referred to as a GTLA picture. A GTLA picture and all coded pictures with temporal_id equal to the temporal_id of the GTLA picture that follow the GTLA picture in decoding order shall not use inter prediction from any picture with temporal_id greater than or equal to the temporal_id of the GTLA picture that precedes the GTLA picture in decoding order.

We propose to keep the TLA picture functionality of HEVC as it is and to add the GTLA pictures in addition.

The main advantages of GTLA pictures are:

- The GTLA pictures provide more flexibility in selection of reference pictures while providing temporal layer switching functionality. This can provide better compression efficiency compared to use of only TLA pictures.

- The gradual temporal layer access functionality is useful in allowing selection of desired frame rate in a step-by-step manner.

Figure 1 shows an example with coding structure when using a temporal layer access (TLA) picture. Figure 2 shows an example with coding structure when using the proposed gradual temporal layer access picture. In the figures the horizontal axis represents the output order. The vertical axis represents the temporal layers.

Figure 1: An example coding structure with a Temporal Layer Access (TLA) picture

Figure 2: An example coding structure with a Gradual Temporal Layer Access (GTLA) picture.

# Proposed Change

We propose following syntax and semantics. The proposed changes are highlighted below compared to HEVC CD [2].

## Syntax and Semantics:

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

NAL units that use nal_unit_type equal to 0 or in the range of 48..63, inclusive, shall not affect the decoding process specified in this Recommendation | International Standard.

NOTE 2 – NAL unit types 0 and 48..63 may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Recommendation | International Standard. Since different applications might use NAL unit types 0 and 48..63 for different purposes, particular care must be exercised in the design of encoders that generate NAL units with nal_unit_type equal to 0 or in the range of 48 to 63, inclusive, and in the design of decoders that interpret the content of NAL units with nal_unit_type equal to 0 or in the range of 48 to 63, inclusive.

Decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of nal_unit_type.

NOTE 3 – This requirement allows future definition of compatible extensions to this Recommendation | International Standard.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-RAP, non-TFD and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Coded slice of a TFD picture slice_layer_rbsp( ) | VCL |
| 3 | Coded slice of a non-TFD TLA picture slice_layer_rbsp( ) | VCL |
| 4, 5 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 6, 7 | Coded slice of a BLA picture slice_layer_rbsp( ) | VCL |
| 8 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 9 | Coded slice of a non-TFD GTLA picture slice_layer_rbsp( ) | VCL |
| 10..24 | Reserved | n/a |
| 25 | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 26 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 27 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 28 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 29 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 30 | Filler data filler_data_rbsp( ) | non-VCL |
| 31 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 32..47 | Reserved | n/a |
| 48..63 | Unspecified | non-VCL |

- NOTE 4 - A CRA picture having nal_unit_type equal to 4 may have associated TFD pictures present in the bitstream. A CRA picture having nal_unit_type equal to 5 does not have associated TFD pictures present in the bitstream. A BLA picture having nal_unit_type equal to 6 may have associated TFD pictures present in the bitstream. A BLA picture having nal_unit_type equal to 7 does not have associated TFD pictures present in the bitstream.

Coded slice NAL unit collectively refers to a coded slice with nal_unit_type in the range of 1 to 8, inclusive. The variable IdrPicFlag is specified as

IdrPicFlag = ( ( nal_unit_type  = =  8 )  ?  1  :  0 )		(7-1)

The variable RapPicFlag is specified as

RapPicFlag = ( ( nal_unit_type  >=  4  &&  nal_unit_type  <=  8  )  ?  1  :  0 )	(7-2)

When the value of nal_unit_type is equal to any particular value in the range of 1 to 8, inclusive, for a NAL unit of a particular picture, all VCL NAL units of that particular picture shall have nal_unit_type equal to that particular value.

When the value of nal_unit_type is equal to 4 or 5 for all VCL NAL units of a particular picture, the particular picture is referred to as a CRA picture.

When the value of nal_unit_type is equal to 6 or 7 for all VCL NAL units of a particular picture, the particular picture is referred to as a BLA picture.

All coded pictures that follow a CRA or BLA picture both in decoding order and output order shall not use inter prediction from any picture that precedes the CRA or BLA picture either in decoding order or output order; and any picture that precedes the CRA or BLA picture in decoding order shall also precede the CRA or BLA picture in output order.

It is a requirement of bitstream conformance that no TFD pictures shall be present in the bitstream that are associated with a CRA picture having nal_unit_type equal to 5 or a BLA picture having nal_unit_type equal to 7.

When the value of nal_unit_type is equal to 8 for all VCL NAL units of a particular picture, that particular picture is referred to as an IDR picture. All coded pictures that follow an IDR picture in decoding order shall not use inter prediction from any picture that precedes the IDR picture in decoding order, and any picture that precedes the IDR picture in decoding order shall also precede the IDR picture in output order.

RAP picture collectively refers to a coded picture that is a CRA picture, a BLA picture or an IDR picture, and RAP access unit collectively refers to an access unit that is a CRA access unit, a BLA access unit or an IDR access unit.

- NOTE 4 - Any parameter set (video parameter set, sequence parameter set, picture parameter set, or adaptation parameter set) must be available before the activation of the parameter set. To be able to perform random access from any particular RAP picture by discarding all access units before the particular RAP access unit (and to correctly decode the particular RAP access unit and all the subsequent access units in both decoding and output order), the following condition must be satisfied: each parameter set that is activated during the decoding of the particular RAP access unit or during the decoding of any subsequent access unit in decoding order is either present or provided through external means at or subsequent to that particular RAP access unit and prior to any NAL unit activating that parameter set.

When the value of nal_unit_type is equal to 3 for all VCL NAL units of a particular picture, that particular picture is referred to as a TLA picture. A TLA picture and all coded pictures with temporal_id greater than or equal to the temporal_id of the TLA picture that follow the TLA picture in decoding order shall not use inter prediction from any picture with temporal_id greater than or equal to the temporal_id of the TLA picture that precedes the TLA picture in decoding order. A TLA picture shall not be TFD picture; hence TLA picture is also referred to as non-TFD TLA picture.

When the value of nal_unit_type is equal to 9 for all VCL NAL units of a particular picture, that particular picture is referred to as a GTLA picture. A GTLA picture and all coded pictures with temporal_id equal to the temporal_id of the GTLA picture that follow the GTLA picture in decoding order shall not use inter prediction from any picture with temporal_id greater than or equal to the temporal_id of the GTLA picture that precedes the GTLA picture in decoding order.

A GTLA picture shall not be TFD picture; hence GTLA picture is also referred to as non-TFD GTLA picture.

A GTLA picture could be a TLA picture. In this case it shall be marked as a TLA picture.

When temporal_id_nesting_flag is equal to 1 and temporal_id is greater than 0, nal_unit_type shall be equal to 3 or 9.

Additionally a restriction is put on the RPS of a GTLA picture as follows:

Decoding process for reference picture set

It is a requirement of bitstream conformance that the reference picture set is restricted as follows:

- There shall be no reference picture with temporal_id greater than that of the current picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr.

- When the current picture is a TLA picture, there shall be no reference picture included in the reference picture set with temporal_id greater than or equal to the temporal_id of the current picture.

- When the current picture is a GTLA picture, there shall be no reference picture with temporal_id greater than or equal to that of the current picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr of the GTLA picture and in those sets (RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr) of all subsequent pictures in decoding order with temporal id equal to the temporal_id of the GTLA picture.

- There shall be no reference picture included There shall be no reference picture included in the reference picture set that precedes, in output order, any CRA picture that precedes the current picture both in decoding order and output order.

- When the first coded picture in the bitstream is an IDR picture or the current coded picture is not a leading picture of the first coded picture in the bitstream, there shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that is equal to "no reference picture".

NOTE 5 – A reference picture cannot be included in more than one of the five reference picture set lists.

# Conclusion

This document proposes gradual temporal layer access (GTLA) pictures. It is asserted that the GTLA pictures provide more flexibility in selection of reference pictures while providing temporal layer switching functionality. We propose to adopt the GTLA pictures in HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” JCTVC-I1003_dK, Geneva, May, 2012.