| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11-20 July, 2012 | Document: JCTVC-J0307 |
| --- | --- |

| Title: | Inclusion of adaptive loop filtering (ALF) in Main profile of the HEVC standard |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Yu-Wen Huang2 In Suk Chong3 |  | tomoo.yamakage@toshiba.co.jp yuwen.huang@mediatek.com ichong@qualcomm.com |
| Source: | Toshiba1, Qualcomm2, MediaTek3 |  |  |

_____________________________

# Introduction

Adaptive loop filtering (ALF) was adopted in TMuC (Test Model under Consideration) and it has been part of WD (Working Draft) and HM (HEVC Test Model) from the beginning. ALF has been studied by the committee and making progress in terms of coding efficiency improvement and complexity reduction.

At the last meeting (the 9th JCT-VC meeting) in Geneva, ALF was significantly simplified, and showed a certain level of evidence of subjective picture quality enhancement. During this meeting cycle between the 9th and the 10th JCT-VC meeting, non-normative optimization of ALF encoding (i.e., filter design/reuse and RDO) has been made, which is reported in JCTVC-J0048 and JCTVC-J0144. By JCTVC-J0048, even though ALF in HM7 is significantly simplified, BD-rate gain of ALF is improved. In addition, specification text for ALF is integrated into HEVC text specification draft 7 (JCTVC-I1003 version 6) under the consultation with the text editors.

This contribution proposes to include ALF in Main profile of the HEVC standard.

# Summary of simplification/modification of ALF in WD7/HM7 from WD6/HM6

- A unified syntax to support both low latency application and high efficiency application.

- Region Adaptive (RA16) mode is adopted instead of Block Adaptive (BA) mode, which reduces additional complexity (computation for block classification and switching of filter by 4x4 block).

- Simplification of ALF coefficient signalling such as no switch between predictive coding and direct coding (direct coding is adopted), no run-length coding and Qp independent coefficient quantization, which makes syntax parsing simpler.

- LCU level ALF on/off control instead of CU level control, which makes syntax parsing simpler.

- Independent ALF on/off control (in slice level and LCU level) for each component.

- HM source code cleanup.

- Non-normative optimization of ALF encoding at this meeting (JCTVC-J0048 and JCTVC-J0144)

In addition, all tickets related to ALF are closed (as of July 3), which means ALF has become more stable than the previous version.

## Discussion of simulation results

The coding efficiency improvement by ALF in HM 7.0 with JCTVC-J0048 on the common test conditions is 1.6% (AI Main CURR), 3.1% (RA Main CURR), 2.3% (LDB Main CURR) and 3.6% (LDP Main CURR). In HE cases, additional 0.2 to 0.8% gains are achieved, with 12% decoding time increase on average. It was reported in JCTVC-G923 that there is affinity for ALF with SIMD implementation. ALF provides higher BD-rate gain in high resolution sequences which will be widely deployed in the near future. Since the lifetime of the HEVC standard is expected to be longer than a decade, the benefit to support ALF in the HEVC standard becomes bigger in the future. Furthermore, since ALF does not require any additional external memory access and suitable for parallel processing, the computational burden would be compensated by the progress of LSI design process (e.g. Moore’s Law). In applications that power consumption is serious issue, parallel processing capability is very important, since it can reduce the operating frequency which results in reducing the power voltage to the circuit (i.e., power consumption is proportional of a square of voltage).

Since television monitors that adopt 4K panel have been launched, entertainment applications will support 4K contents through multiple channels (i.e., internet, broadcasting, cable, mobile, and etc) in a couple of years.

In applications that store compressed video in limited storage capacity such as DVD, Blu-Ray, SD card, etc., variable bit rate encoding provides better (or robust) picture quality throughout a sequence, which is realized by saving bits from relatively simple scenes and assigning the saved bits to complex scenes within the maximum bitrate allowed. In typical sequences, the duration of complex scenes is not so long. Therefore, saving 3 to 5% of bits from the other scenes would help improving the picture quality of the complex scenes. If ALF is used, it is not necessary to sacrifice the picture quality of the other scenes by increasing Qp to save bits.

In JCTVC-J0048, it is also reported that the coding efficiency improvement by HM 7.0 with JCTVC-J0048 for KTA 720p and 1080p sequences is 2.5% (AI Main CURR), 4.3% (RA Main CURR), 3.8% (LDB Main CURR) and 9.0% (LDP Main CURR). Therefore, ALF is a promising tool.

In JCTVC-J0049, a comparison between LDP with ALF and LDB without ALF is reported. This reports a trade-off between coding efficiency and (the worst case) memory bandwidth. For application where power consumption is critical, LDP with ALF would be an alternative of LDB without ALF.

Considering the above situation, ALF is one of the essential tools in Main profile of the HEVC standard.

# Conclusion

This contribution presents the benefit to support ALF in Main profile of the HEVC standard and proposes to include ALF in Main profile of the HEVC standard.

# Patent rights declarations

Toshiba Corporation, MediaTek Inc. and Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).