| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0308-r1 |
| --- | --- |

| Title: | Context index derivation method for level 0 significance map coding in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone Wei-Ying Kung Xue Fang Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3618 krit@motorola.com +1-858-404-2350 viviankung@motorola.com +1-858-404-2723 X.Fang@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to share contexts for level 0 significance map coding between 4x4 and 8x8 TUs.  To reduce complexity, the proposal uses common logic to derive context index instead of using mapping table. The simulation results show that overall performance loss is less than 0.1%.

# Introduction

Significance map represents significance value of each element within a TU. Significance value of 1 means that the corresponding element contains at least one significance (non-zero) quantized coefficient(s). Significance value of 0 means that the corresponding element contains no significance (non-zero) quantized coefficient(s).

HEVC [1] employs multi level significance map coding. Level 1 significance map is used to code significance element at subblock level within a TU and level 0 significance map is used to code significance element at pixel level. Level 0 signicance map coding is encoded according to TU size, one for small TUs (4x4, 8x8) and another for large TUs (16x16, 32x32, 16x4, 4x16, 32x8, 8x32).

In HM-7.0, level 0 significance map coding for 4x4 and 8x8 luma and chroma TUs is coded using context and context indices are determined using mapping tables as shown in Tables 1 and 2. A total of 18 contexts for luma TU are used in this process. Two features are employed in these mapping tables [2] as follows:

- Context for (0,0) position as shared for all TU sizes.

- Context for each index of 8x8 TU (except (0,0)) can be derived by (1)

Map_8x8(x,y) = Map_4x4(x<<1 , y<<1) + 9 if (x+y) > 0, 			(1)

Map_8x8(x,y) = Map_4x4(x , y) if (x+y) = 0, where x and y are coordinate within 8x8 TU

Table 1. Context index mapping table for 4x4 TU

| 0 | 1 | 4 | 5 |
| --- | --- | --- | --- |
| 2 | 3 | 4 | 5 |
| 6 | 6 | 8 | 8 |
| 7 | 7 | 8 | 8 |

Table 2. Context index mapping table for 8x8 TU

| 0 | 9 | 10 | 10 | 13 | 13 | 14 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 9 | 10 | 10 | 13 | 13 | 14 | 14 |
| 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 |
| 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 |
| 15 | 15 | 15 | 15 | 17 | 17 | 17 | 17 |
| 15 | 15 | 15 | 15 | 17 | 17 | 17 | 17 |
| 16 | 16 | 16 | 16 | 17 | 17 | 17 | 17 |
| 16 | 16 | 16 | 16 | 17 | 17 | 17 | 17 |

# Proposed Method

We propose to share the contexts between 4x4 and 8x8 TUs. These arrangements enable simple logics to derive context index for level 0 significance map coding for 4x4 and 8x8 TUs.  Our proposed logic to derive the context index of significance map is as below:

Idx = x>>shift + 4 * (y>>shift) + 1 - !(x+y)				(2)

Shift = log2TrafoWidth - 2

Note that log2TrafoWidth is log base2 of TU width.

Table 3 and Table 4 are the context index mapping tables with the proposed logic for 4x4 and 8x8 TUs, respectively.

Table 3. Context index mapping table with the first proposed logic for 4x4 TU

| 0 | 2 | 3 | 4 |
| --- | --- | --- | --- |
| 5 | 6 | 7 | 8 |
| 9 | 10 | 11 | 12 |
| 13 | 14 | 15 | 16 |

Table 4. Context index mapping table with the first proposed logic for 8x8 TU

| 0 | 1 | 2 | 2 | 3 | 3 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 2 | 3 | 3 | 4 | 4 |
| 5 | 5 | 6 | 6 | 7 | 7 | 8 | 8 |
| 5 | 5 | 6 | 6 | 7 | 7 | 8 | 8 |
| 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 |
| 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 |
| 13 | 13 | 14 | 14 | 15 | 15 | 16 | 16 |
| 13 | 13 | 14 | 14 | 15 | 15 | 16 | 16 |

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 3. Detailed results are included in the attachment (JCTVC-J0308.xls).

Table 3 QP (22 27 32 37)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 78% |  |  | 80% |  |  |
| Dec Time[%] | 97% |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | 0.0% | 0.1% | 0.3% | 0.0% |
| Class B | 0.0% | 0.3% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.1% | 0.3% | 0.2% | 0.0% | 0.3% | 0.1% |
| Class D | 0.2% | 0.3% | 0.2% | 0.1% | 0.1% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.0% |
| Class F | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% | -0.2% |
| Enc Time[%] | 75% |  |  | 76% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.6% | 0.3% | 0.0% | 0.2% | 0.5% |
| Class C | 0.1% | 0.4% | 0.4% | 0.0% | 0.3% | 0.3% |
| Class D | 0.2% | 0.4% | 0.4% | 0.1% | 0.3% | 0.1% |
| Class E | 0.1% | 0.0% | -0.6% | 0.1% | 0.6% | -0.9% |
| Overall | 0.1% | 0.4% | 0.2% | 0.0% | 0.3% | 0.1% |
|  | 0.1% | 0.3% | 0.2% | 0.1% | 0.4% | 0.1% |
| Class F | 0.1% | 0.2% | 0.3% | 0.3% | 0.8% | 0.5% |
| Enc Time[%] | 75% |  |  | 75% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7,” JCTVC-I1003.

[2] J. Chen, J. Sole, W.-J. Chien, V. Seregin, M. Karczewicz, “AHG5: Context derivation clean-up for significance map,” JCTVC-I0373.

# Text for Working Draft

##### 9.2.3.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  0		(9-16)

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived as follows..

sigCtx  =  4*yC + xC + 1		(9-17)

- Otherwise, if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using as follows.

sigCtx  =  4*(yC>>1) + xC>>1 + 1		(9-18)

- Otherwise, sigCtx is derived using previously decoded bins of the syntax element significant_coeff_group_flag as follows.

- The variable xCG is derived as follows.

xCG  =  xC >> 2		(9-19)

- The variable yCG is derived as follows.

yCG  =  yC >> 2		(9-20)

- The variable cntCG is initialized as follows.

cntCG  =  0		(9-21)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

cntCG  =  cntCG  +  significant_coeff_group_flag[ xCG + 1 ][ yCG ]	(9-22)

- When yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

cntCG  =  cntCG  +  significant_coeff_group_flag[ xCG ][ yCG + 1 ] << 1	(9-23)

- If cntCG is equal to 0, the the following applies.

sigCtx  =   ( xC - (xCG << 2) ) + ( yC - (yCG << 2) ) <= 2 ? 1 : 0	(9-24)

- Otherwise, if cntCG is equal to 1, the following applies.

sigCtx  =   ( yC - (yCG << 2) ) <= 1 ? 1 : 0		(9-25)

- Otherwise, if cntCG is equal to 2, the following applies.

sigCtx  =   ( xC - (xCG << 2) ) <= 1 ? 1 : 0		(9-26)

- Otherwise ( cntCG is equal to 3 ), the following applies.

sigCtx  =   ( xC - (xCG << 2) ) + ( yC - (yCG << 2) ) <= 4 ? 2 : 1    	(9-27)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = sigCtx + 20		(9-28)

  - Otherwise, the following applies.

sigCtx = sigCtx + 17		(9-29)

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-30)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  23  +  sigCtx		(9-31)