| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0309-r3 |
| --- | --- |

| Title: | Removal of last coefficient coding in transform skipping mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone Yue Yu Xue Fang Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3618 krit@motorola.com +1-858-404-2251 yyu@motorola.com +1-858-404-2723 X.Fang@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to remove last coefficient coding in transform skipping mode. The simulation results show that overall performance loss is less than 0.03%.

# Introduction

HM-7.0 [1] uses last coefficient coding to signal the location of the last significance coefficient position. Last significance position indicates the last non-zero transform coefficient in a coding order within a TU. Hence, the remaining coefficients after the last significance coefficient position must be zero and do not need to be coded.

In HM-7.0, input is likely to be natural video dominated by low frequency component. Last coefficient coding takes advantage of this behavior in transform domain to improve coding efficiency and reduce complexity. Specifically, last coefficient coding assumes that most non-zero transform coefficients locate in the low frequency region and most high frequency coefficients are zero. It is hence beneficial to skip the consecutive zero coefficients in high frequency at the end of the block.

The assumed statistic may not be true when transform skipping mode is used. In transform skipping mode [2], a prediction residual block after prediction process is directly quantized without going through any transform operation. Benefits brought by last coefficient coding no longer exists for such residual blocks.

# Proposed Method

This contribution proposes to bypass last coefficient coding process for a TU when transform skipping mode is used. This change reduces complexity and may improve coding efficiency. Specifically, this contribution proposes coding all 16 residuals within a 4x4 intra TU using transform skipping mode; i.e., last significance coefficient is set to position (3,3). No other change is required in entropy coding process.

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. Detailed results are included in the attachment (JCTVC-J0309.xls). In this experiment, no change is made to the RD optimization. Experimental results with appropriate changes to RD optimization is provided in Table 2. Detailed results are included in the attachment (JCTVC-J0309-RDO.xls).

Table 1 Results without RD optimization QP (22 27 32 37)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.01% | -0.01% | -0.03% |
| Class B | 0.03% | 0.02% | -0.01% |
| Class C | 0.03% | -0.04% | -0.04% |
| Class D | 0.06% | 0.06% | 0.00% |
| Class E | 0.02% | 0.01% | -0.04% |
| Overall | 0.03% | 0.01% | -0.02% |
|  | 0.03% | 0.00% | -0.02% |
| Class F | -0.25% | -0.47% | -0.24% |
| Enc Time[%] | 98.45% |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.01% | 0.16% | 0.08% |
| Class B | 0.01% | -0.06% | -0.11% |
| Class C | -0.01% | -0.11% | -0.42% |
| Class D | 0.02% | -0.12% | -0.44% |
| Class E |  |  |  |
| Overall | 0.01% | -0.04% | -0.22% |
|  | 0.01% | -0.06% | -0.24% |
| Class F | -0.14% | -0.03% | -0.35% |
| Enc Time[%] | 80.06% |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.01% | 0.14% | 0.11% |
| Class C | -0.01% | -0.10% | 0.20% |
| Class D | 0.00% | 0.42% | 0.22% |
| Class E | 0.10% | -0.70% | -0.85% |
| Overall | 0.01% | -0.01% | -0.02% |
|  | 0.01% | 0.02% | 0.03% |
| Class F | 0.13% | 0.31% | -0.02% |
| Enc Time[%] | 77.84% |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 2 Results with RD optimization QP (22 27 32 37)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.01% | -0.02% | -0.02% |
| Class B | 0.02% | 0.00% | 0.00% |
| Class C | 0.02% | -0.02% | -0.02% |
| Class D | 0.06% | 0.07% | -0.02% |
| Class E | 0.01% | 0.01% | 0.04% |
| Overall | 0.02% | 0.00% | -0.01% |
|  | 0.03% | 0.00% | 0.00% |
| Class F | -0.28% | -0.50% | -0.34% |
| Enc Time[%] | 104.87% |  |  |
| Dec Time[%] | 105.00% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.07% | 0.10% | -0.14% |
| Class B | 0.01% | 0.01% | -0.04% |
| Class C | 0.00% | 0.07% | -0.26% |
| Class D | -0.01% | -0.20% | -0.31% |
| Class E |  |  |  |
| Overall | 0.01% | 0.00% | -0.18% |
|  | 0.02% | -0.03% | -0.18% |
| Class F | -0.11% | -0.19% | -0.30% |
| Enc Time[%] | 86.57% |  |  |
| Dec Time[%] | 91.09% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.00% | 0.08% | 0.11% |
| Class C | -0.01% | -0.15% | 0.10% |
| Class D | -0.01% | 0.51% | -0.08% |
| Class E | 0.11% | -0.67% | -0.32% |
| Overall | 0.02% | -0.01% | -0.02% |
|  | 0.01% | -0.01% | 0.04% |
| Class F | -0.03% | 0.28% | 0.76% |
| Enc Time[%] | 86.27% |  |  |
| Dec Time[%] | 84.98% |  |  |

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7,” JCTVC-I1003.

[2] . Lan, J. Xu, G. J. Sullivan, F. Wu, “Intra Transform Skipping,” JCTVC-I0408.

# Text for Working Draft

### Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| If( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if (!transform_skip_flag[ x0 ][ y0 ][ cIdx ]){ |  |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| } |  |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 0 ] >> 2 |  |
| yCG = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ i << 4 ][ 1 ] >> 2 |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − !transform_skip_flag[ x0 ][ y0 ][ cIdx ]) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| } |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold && !cu_ transquant_bypass _flag) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| TransCoeffLevel[x0][y0][cIdx][xC][yC] = − TransCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

last_significant_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_x_prefix shall be in the range from 0 to ( log2TrafoWidth << 1 ) − 1, inclusive. If last_significant_coeff_x_prefix is not present, last_significant_coeff_x_prefix is inferred to 3.

last_significant_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_y_prefix shall be in the range from 0 to ( log2TrafoHeight << 1 ) − 1, inclusive. If last_significant_coeff_x_prefix is not present, last_significant_coeff_x_prefix is inferred to 3.