| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0310 |
| --- | --- |

| Title: | Revival of decodable backward predicted pictures that are output preceding a RAP picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez, A. K Katti, and H-Y Hwang 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

When certain sequences are encoded using random access configuration, some of the currently defined leading pictures after a RAP picture in decode order may be decodable because they are solely backward predicted from the RAP picture or other decodable pictures after the RAP.  Some applications produce such backward predicted pictures when replacing existing coded video sequences with new content to manage the constant-bit-rate (CBR) bitstream emissions while operating with reasonable buffer CPB delay.  Some bitstreams are coded with hierarchical inter-prediction structures that are anchored by every pair of successive Intra pictures with a significant number of coded pictures between the Intra pictures. This contribution proposes not to mark backward predicted pictures after a RAP picture as TFD ("tagged for discard") pictures.

# Introduction

A coded picture that follows a RAP picture in decoding order and precedes it in output order is referred to as a leading picture of that RAP picture. While it was previously observed that leading pictures associated with a RAP picture need not be decoded when random access is initiated at that RAP picture, there are applications that do benefit from knowing that the pictures after the RAP picture are decodable, although their output times are prior to the RAP picture’s output time.

Leading pictures are identified with a different NAL unit type (nal_unit_type = 2). Their identification is useful so that they can be discarded when a decoder enters the bitstream at the RAP picture. Such leading pictures have been called TFD ("tagged for discard") pictures. Note that some of these leading pictures could be backward predicted solely from the RAP picture and/or other decodable pictures after the RAP in the decode order. This contribution aims to distinguish such backward predicted decodable pictures that are transmitted after a RAP picture and that have output time prior to the RAP picture from non-decodable leading pictures associated with the given RAP picture that are not decodable because they truly depend on reference pictures that precede the RAP picture in decode order.

Some applications produce backward predicted pictures when replacing existing coded video sequences with new content to manage the constant-bit-rate (CBR) bitstream emissions while operating with reasonable buffer CPB delay.  Some bitstreams are coded with hierarchical inter-prediction structures that are anchored by every pair of successive Intra pictures in the bitstream with a significant number of coded pictures between them.

# Proposal

A new definition for TFD pictures is proposed along with another type of NAL unit to identify leading pictures that are backward predicted and decodable from the associated RAP and/or other decodable pictures after the RAP in the decode order. A TFD picture should be a picture that depends on a picture or information preceding the RAP picture in the decode order, directly or indirectly.

- tagged for discard (TFD) picture: A coded picture for which each slice has nal_unit_type equal to 2. When the decoding of a bitstream starts at a particular RAP, a picture that follows this RAP picture in decode order and precede the same RAP picture in output order is considered a TFD picture if it is either inter-predicted from a picture that precedes this RAP picture in both decode and output order or inter-predicted from another TFD picture. In such cases, a TFD picture is not fully decodable.

- decodable with prior output (DWPO) access unit: An access unit in which the coded picture is a DWPO picture.

- decodable with prior output (DWPO) picture: A coded picture for which each slice has nal_unit_type equal to 9. When the decoding of a bitstream starts at a particular RAP, a picture that follows this RAP picture in decode order and precede the same RAP picture in output order is considered a DWPO picture if it is not a TFD picture. In such cases, a DWPO picture is fully decodable.

The following table indicates change in nal_unit_type for the proposed changes.

- Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-RAP, non-TFD and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Coded slice of a TFD picture slice_layer_rbsp( ) | VCL |
| 3 | Coded slice of a non-TFD TLA picture slice_layer_rbsp( ) | VCL |
| 4, 5 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 6, 7 | Coded slice of a BLA picture slice_layer_rbsp( ) | VCL |
| 8 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 9 | Coded slice of an DWPO picture slice_layer_rbsp() | VCL |
| 10..24 | Reserved | n/a |
| 25 | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 26 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 27 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 28 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 29 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 30 | Filler data filler_data_rbsp( ) | non-VCL |
| 31 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 32..47 | Reserved | n/a |
| 48..63 | Unspecified | non-VCL |

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).