| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0311 |
| --- | --- |

| Title: | Cross check of memory bandwidth reduction for higher level (JCTVC-J0175) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check result of JCTVC-J0175, which introduces SPS signaling and bi-pred to uni-pred conversion to achieve memory bandwidth reduction for higher level. The algorithms and source codes are studied and verified. In addition, it is confirmed that the experimental results match with those provided by the proponents.

# Cross check

In the cross check, following two configurations are tested.

- Config 1: Restrict 8x8 bi-pred with bi-pred to uni-pred conversion

- Config 2: Restrict 8x8 bi-pred (encoder technique)

Algorithms and macros for each configuration are summarized in Table 1. In Config 1, 8x8 bi-pred is converted to uni-pred, which means merge candidate can be always used. On the other hand, in Config 2, 8x8 bi-pred is not converted, thus merge candidate sometimes cannot be used. Other difference is SPS signal. In Config 1, a flag is added to signal whether 8x8 bi-pred is converted to uni-pred.

Table 1: Algorithms and macros for each configuration

|  |  | Config 1 | Config 2 |
| --- | --- | --- | --- |
| Prediction | > 8x8 | uni-pred & bi-pred | uni-pred & bi-pred |
|  | 8x8 | uni-pred (with conversion) | uni-pred (without conversion) |
|  | 8x4/4x8 | uni-pred (with conversion) | uni-pred (with conversion) |
| SPS signal |  | flag is added | as in HM-7.0 |
| Macros |  | AVOID_BIP_SMALL_MRG 1 AVOID_BIP_SMALL_MRG_DEFAULT 1 | AVOID_BIP_SMALL_MRG 0 AVOID_BIP_SMALL_MRG_DEFAULT 0 |

Using the software provided by the proponents, simulations were carried out based on the Common test condition. Random access and low delay B are tested both with MAIN and HE10. Linux cluster were used for running the simulation, thus encoding and decoding time may not be accurate.

Table 2 and 3 show the experimental results for Config 1 and Config 2 respectively. It is confirmed that BD-rate values match with those provided by the proponents.

Table 2: Results for Config 1 (Restrict 8x8 bi-pred with bi-pred to uni-pred conversion)

Table 3: Results for Config 2 (Restrict 8x8 bi-pred (encoder technique))

# Conclusion

The algorithms and experimental results of bi-pred to uni-pred conversion (JCTVC-J0175) are checked and confirmed. Considering that 8x8 bi-pred restriction, which significantly reduces memory bandwidth, is useful for decoding high resolution sequences, the gain provided by the proposed method is beneficial. Therefore, it is suggested to consider adopting the proposed method.