| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0312 |
| --- | --- |

| Title: | Redundancy removal on InterDir syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Kim Tammy Lee Elena Alshina Jaehyun Kim | Tel: Email: | +82-31-279-2281 dionism@samsung.com tammy.lee@samsung.com elena_a.alshina@samsung.com jhgim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the previous meeting, the restriction of bi-prediction by 4x8/8x4 PU was adopted in order to reduce memory bandwidth. However, the encoder still signals the bi-prediction of 4x8/8x4 PU, the decoder ignores it. It cause additional condition check, that is bi-prediction checking . The redundancy removal of syntax for bi-prediction of small PU gives -0.1% gain for LB cases. This proposal is the same of JCT-VC0089

# Introduction

In [1], the small size PUs such as 4x8 and 8x4 were not allowed use bi-prediction to reduce memory bandwidth. However, the syntax for small Pus on bi-prediction is still signalled. It cause necessity of checking for pcCU->getInterDir(uiAbsPartIdx)==3 which does not occurred if the redundancy removal is performed although it requires one checking condition this PU has restriction or not. The removal of redundancy gives a coding gain by 0.1% for LB cases and reduce all the condition check for pcCU->getInterDir(uiAbsPartIdx)==3 in HM software as follows.

Case 1 :

if(pcCU->getSlice()->getMvdL1ZeroFlag() && eRefList == REF_PIC_LIST_1

Case 2 :

if (pcSubCU->isBipredRestriction(uiPartIdx) )

Whales, adding one checking

if(!pcCU->isBipredRestriction(uiAbsPartIdx))

m_pcBinIf->encodeBin( uiInterDir == 2 ? 1 : 0, *( pCtx + uiCtx ) );

# Simulation Result

Table. 1  Common condition test

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | -0.1% | 0.1% | -0.2% | -0.1% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.2% |
| Class C | -0.1% | 0.1% | -0.1% | -0.1% | -0.2% | 0.2% |
| Class D | -0.1% | -0.2% | 0.3% | -0.2% | -0.2% | -0.2% |
| Class E | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% | 0.4% |
| Overall | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% | 0.1% |
|  | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% |
| Class F | -0.1% | 0.7% | 0.0% | 0.0% | -0.1% | 0.9% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

# Conclusion

This contribution presents the redundancy removal of syntax for restriction bi-prediction of small PU. Redundancy removal gives 0.1% gain for LB cases and no loss for others. Therefore, it is recommend that removal of redundancy on syntax be considered in the current HM .

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S. Fukushima, M. Ueda, H.Takehara  “AHG7:Bi-pred restriction for small PUs,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0297, April, 2012

# Draft Change

In Table 9-1 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| inter_pred_idc | Error: Reference source not found | 0 | If (nPSW + nPSH != 12) ctDepth Otherwise 4 | If (nPSW + nPSH != 12) 4 Otherwise na | na | na | na |

#### 9.2.2.x Binarization process for inter_pred_idc

Input to this process is a request for a binarization for the syntax element inter_pred_idc, slice_type and variables specifying the width and the height of the prediction unit, nPSW and nPSH.

Output of this process is the binarization of the syntax element.

The binarization for inter_pred_idc is given by Table 9-xx depending on inter_pred_idc, slice_type and prediction unit size nPSW and nPSH.

Table 9-xx – Binarization for inter_pred_idc

| slice_type | inter_pred_idc | Name of inter_pred_idc | Bin String |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | nPSW + nPSH != 12 | nPSW + nPSH == 12 |  |
| P | inferred | Pred_L0 | - | - |  |
| B | 0 | Pred_L0 | 00 |  | 0 |
|  | 1 | Pred_L1 | 01 |  | 1 |
|  | 2 | Pred_BI | 1 |  | - |

- – Name association to inter prediction mode

- -

- -

- -

- -

- -

- -