| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0313_r2 WG11 Number: m25641 |
| --- | --- |

| Title: | Modified coefficient scan order mapping for transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Cohen Anthony Vetro Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

The coefficient scan order mapping in the current HEVC text specifications map Intra prediction modes 6-14 to ScanType 2 (vertical) and Intra prediction modes 22-30 to ScanType 1 (horizontal). The purpose of this proposal is to improve the coding performance when transform skip mode is used, without introducing new scan orders into the standard. Method 1 of this proposal swaps the horizontal and vertical mapping when transform skip mode flag is true for a transform block. Average BD-Rate changes for Class F sequences are –0.9% for AI-HE10, –0.7% for RA-HE10, and –0.3% for LB-HE10 configurations. Method 2 expands the range of swapped scans. Average BD-Rate changes for Class F sequences for Method 2 are –1.0% for AI-HE10, –0.7% for RA-HE10, and –0.4% for LB-HE10 configurations. Changes to the HEVC text specification and results in combination with JCTVC-J0105 are provided as well.

# Introduction

Table 7-12 of the High Efficiency Video Coding (HEVC) text specification draft 7 [1] defines a mapping between the Intra prediction mode and the coefficient scan order ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]. It maps Intra prediction modes 6-14 to ScanType 2 (vertical) and Intra prediction modes 22-30 to ScanType 1 (horizontal). The rest of the modes are mapped to ScanType 0 (up-right diagonal). This mapping is shown in Figure 1.

Figure 1. Current HEVC scan order mapping for Intra prediction modes

When transform skip mode is used on a TU, the distribution of data in the block is no longer compacted toward the DC and low-frequency corner normally associated with a transformed block. The existing scan mapping is therefore suboptimal in this case. The purpose of this proposal is to propose a mapping that improves the performance on skipped blocks, without introducing new scan orders into the standard.

# Scan order map modifications

## Method 1: Swapping

The modification simply swaps the horizontal and vertical scan orders for when transform skip mode is used. The modification of the scan order mapping is illustrated in Figure 2. Intra prediction modes 6-14 are mapped to ScanType 1 (horizontal), and Intra prediction modes 22-30 are mapped to ScanType 2 (vertical).

Figure 2. Modified scan order mapping for Method 1

The mapping table used if transform_skip_flag for the current TU is 0 is the unmodified table currently in the specifications. If transform_skip_flag is 1, then two values in the table are swapped as follows:

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 5 | 0 | 0 | 0 | 0 |
| 6 – 14 | 2 | 2 | 0 | 0 |
| 15 – 21 | 0 | 0 | 0 | 0 |
| 22 – 30 | 1 | 1 | 0 | 0 |
| 31 – 35 | 0 | 0 | 0 | 0 |

swap these values when transform_skip_flag is 1

Because the swap is done only when transform_skip_flag is 1, it only affects the column associated with the 4x4 transform.

## Method 2: Swapping and expansion

In addition to swapping the horizontal and vertical scan ranges as in Method 1, this method also expands these scans to cover a wider range of Intra prediction modes. This mapping is illustrated in Figure 3.

Figure 3. Modified scan order mapping for Method 2

The mapping table in this case is as follows:

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 5 | 1 | 0 | 0 | 0 |
| 6 – 14 | 1 | 2 | 0 | 0 |
| 15 – 17 | 1 | 0 | 0 | 0 |
| 18 – 21 | 2 | 0 | 0 | 0 |
| 22 – 30 | 2 | 1 | 0 | 0 |
| 31 – 34 | 2 | 0 | 0 | 0 |
| 35 | 0 | 0 | 0 | 0 |

# Experimental results

The proposed modification was tested using the HM-7.0 software and common test conditions [2]. Transform skip mode is only used with the HE10 configurations, so this proposal does not affect the Main configurations.

## Method 1: Swapping

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.2% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% |
| Class F | -0.9% | -0.8% | -0.8% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.2% | -0.6% |
| Class E |  |  |  |
| Overall | 0.0% | -0.1% | -0.3% |
|  | 0.0% | -0.1% | -0.3% |
| Class F | -0.7% | -0.5% | -0.6% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.5% | 0.0% |
| Class E | 0.1% | -0.9% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | -0.3% | 0.3% | 0.3% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 103% |  |  |

## Method 2: Swapping and expansion

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.2% | -0.2% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% |
| Class F | -1.0% | -1.0% | -0.9% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | -0.1% |
| Class D | -0.1% | -0.1% | -0.4% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | -0.2% |
|  | 0.0% | 0.0% | -0.2% |
| Class F | -0.7% | -0.5% | -0.7% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.2% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | -0.1% | 0.2% | -0.2% |
| Class E | 0.0% | -0.2% | -0.3% |
| Overall | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.1% | 0.0% |
| Class F | -0.4% | 1.0% | 0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

# Additional results when combined with JCTVC-J0105

To see if this proposal were compatible with JCTVC-J0105 [3], we combined Simplification 2 from JCTVC-J0105, which reduces the number of Intra prediction modes mapped to horizontal and vertical scans from 9 to 8, with Method 1 of this proposal. The results show that JCTVC-J0105 is compatible with Method 1.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.2% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | 0.0% |
| Class F | -0.9% | -0.8% | -0.7% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | -0.1% |
| Class D | -0.1% | 0.0% | -0.3% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.1% |
| Class F | -0.6% | -0.4% | -0.7% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.3% |
| Class D | -0.1% | 0.5% | 0.0% |
| Class E | 0.1% | -0.7% | -0.5% |
| Overall | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.1% | 0.0% |
| Class F | -0.3% | 0.5% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

# Conclusions

The objective of this proposal was to improve the coding performance on transform blocks whose transform was skipped, without introducing new scan orders or reordering data in the block. In Method 1, by swapping the horizontal and vertical scan mapping for when transform skip mode is used, the average BD-Rate changes for Class F sequences are –0.9% for AI-HE10, –0.7% for RA-HE10, and –0.3% for LB-HE10 configurations. Method 2 expands the range of the swapped scans. Average BD-Rate changes for Class F sequences for Method 2 are –1.0% for AI-HE10, –0.7% for RA-HE10, and –0.4% for LB-HE10 configurations. We propose either of these modifications for inclusion in the HEVC text specification.

Method 1 of this proposal JCTVC-J0313 is identical to Method 2 of JCTVC-J0202 [4] and JCTVC-J0212 [5]. A common set of presentation slides from all three proponents of these proposals is included with the uploaded set of files associated with each of these proposals.

# References

- B. Bross et al., “High Efficiency Video Coding (HEVC) text specification draft 7,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1003_d5, Geneva, CH, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1100, Geneva, CH, May 2012.

- R. Cohen and A. Vetro, “Simplification of coefficient scanning order,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0105, Stockholm, SE, July 2012

- H. Y. Kim et al., “Coefficient scan for transform skip mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0202, Stockholm, SE, July 2012

- T. Sugio and T. Nishi, “Modified scan method for intra transform skipping,”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0212, Stockholm, SE, July 2012

- Changes to the HEVC text specification

There are several ways to effect these changes in the text. This section describes two approaches. Approach 1 minimizes changes to lines of text in the semantics, while Approach 2 puts the changes in a separate table. Additions are highlighted in yellow.

## Method 1: Swapping

### Approach 1: transform_skip_flag is used inside the table

    - Transform coefficient semantics

The transform coefficient levels are parsed into the arrays TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices xC, yC specify the transform coefficient location ( xC, yC ) within the current transform block.

When PredMode is equal to MODE_INTRA, different scanning orders are used. The array ScanType[ log2TrafoSize − 2 ][ IntraPredMode ], specifying the scanning order for various luma transform block sizes and intra prediction modes, is derived as specified in Table 7-2.

Table 7-1 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 5 | 0 | 0 | 0 | 0 |
| 6 – 14 | 2 >> transform_skip_flag[x0][y0][cIdx] | 2 | 0 | 0 |
| 15 – 21 | 0 | 0 | 0 | 0 |
| 22 – 30 | 1 << transform_skip_flag[x0][y0][cIdx] | 1 | 0 | 0 |
| 31 – 35 | 0 | 0 | 0 | 0 |

### Approach 2: transform_skip_flag is not used inside the table

    - Transform coefficient semantics

The transform coefficient levels are parsed into the arrays TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices xC, yC specify the transform coefficient location ( xC, yC ) within the current transform block.

When PredMode is equal to MODE_INTRA, different scanning orders are used. The array ScanType[ log2TrafoSize − 2 ][ IntraPredMode ], specifying the scanning order for various luma transform block sizes and intra prediction modes, is derived as specified in Table 7-2, except when log2TrafoSize is equal to 2 and transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1.

Table 7-2 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 5 | 0 | 0 | 0 | 0 |
| 6 – 14 | 2 | 2 | 0 | 0 |
| 15 – 21 | 0 | 0 | 0 | 0 |
| 22 – 30 | 1 | 1 | 0 | 0 |
| 31 – 35 | 0 | 0 | 0 | 0 |

When log2TrafoSize is equal to 2 and transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1, the scanning order is derived as specified in Table 7-13.

Table 7-13 – Specification of ScanType[ 0][ IntraPredMode ]

| IntraPredMode | ScanType[ 0][ IntraPredMode ] |
| --- | --- |
| 0 | 0 |
| 1 | 0 |
| 2 – 5 | 0 |
| 6 – 14 | 1 |
| 15 – 21 | 0 |
| 22 – 30 | 2 |
| 31 – 35 | 0 |

## Method 2: Swapping and expansion

Additions are highlighted in yellow. Deletions are highlighted in blue.

Table 7-3 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 – 5 | transform_skip_flag[x0][y0][cIdx] | 0 | 0 | 0 |
| 6 – 14 | 2 >> transform_skip_flag[x0][y0][cIdx] | 2 | 0 | 0 |
| 15 – 2117 | transform_skip_flag[x0][y0][cIdx] | 0 | 0 | 0 |
| 18 – 21 | transform_skip_flag[x0][y0][cIdx] << 1 | 0 | 0 | 0 |
| 22 – 30 | transform_skip_flag[x0][y0][cIdx] << 1 | 1 | 0 | 0 |
| 31 – 3534 | 0 | 0 | 0 | 0 |
| 35 | transform_skip_flag[x0][y0][cIdx] << 1 | 0 | 0 | 0 |

# Patent rights declaration

Mitsubishi Electric Research laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).