| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0315 |
| --- | --- |

| Title: | AHG5: Context reduction for MVD coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Kim Jaehyun Kim JeungHun Park | Tel: Email: | +82-31-279-2281 dionism@samsung.com jhgim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current HM, the first two bins of motion vector difference (MVD) are coded with context. The remaining bits are coded with bypass mode. The two flags,  abs_mvd_greater0_flag, and abs_mvd_greater1_flag, are used for context at both flags. From the statistical analysis, the distribution between ‘0’ and ‘1’ is similar, so, the use of context is not much benefit for separating ‘0’ and ‘1’. Therefore, we perform abs_mvd_greater1_flag rather than abs_mvd_greater0_flag  where abs_mvd_greater0_flag is coded with bypass mode. The proposed method reduces context from 2 to 1. No loss and complexity is observed both normal QP and lowQP (1,5,9,13). Moreover, total bins (context + bypass) is reduced by 0.1% and contexts is also reduced 1.5% average for RA and LD

# Introduction

In the current HM, the first two bins of motion vector difference(MVD) syntax are coded with two contexts respectively and remaining bins are processed through bypass mode. From the statistical analysis, the occurrence between ‘0’ and ‘1’ shows the similar probability. However, the large value than ‘2’ has the different statistic. Therefore, the combined treatment between ‘0’ and ‘1’ is more reasonable than separate handling. The number of context is reduced from 2 to 1. Fig.1 show the current HM scheme for MVD coding. The propose method denoted in Fig. 2 only can use rearrange existing function module. Therefore, no additional function and complexity are not introduced. The changed meaning of abs_mvd_greater1_flag is following.

In current HM : abs_mvd_greater1_flag is used after abs_mvd_greater0_flag, therefore, abs_mvd_greater1_flag can be used for checking ‘1’ and ‘larger than 1’.

The proposed method : abs_mvd_greater1_flag is used at first. That means that conditions are ‘less than ‘1’ including ‘0’ and ‘larger than 1’.

Fig 1.  HM 7.0 MVD coding

Fig 2. The proposed method

# Simulation Result

Table. 1  Common condition test

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.05% | -0.15% | -0.09% | 0.04% | -0.14% | 0.23% |
| Class B | 0.00% | 0.06% | 0.04% | 0.03% | 0.03% | 0.03% |
| Class C | 0.00% | 0.06% | 0.06% | 0.05% | 0.06% | 0.05% |
| Class D | 0.00% | -0.01% | -0.02% | -0.01% | -0.03% | -0.13% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.00% | 0.03% | -0.02% | 0.04% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.15% | 0.16% | 0.20% | 0.13% | 0.05% | 0.12% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.02% | 0.14% | 0.16% | 0.01% | 0.07% | 0.19% |
| Class C | -0.02% | 0.03% | 0.03% | -0.05% | -0.08% | -0.14% |
| Class D | -0.02% | 0.28% | -0.06% | -0.05% | -0.04% | 0.27% |
| Class E | -0.12% | -0.20% | -0.07% | 0.10% | -0.42% | 0.36% |
| Overall | -0.02% | 0.08% | 0.03% | 0.00% | -0.09% | 0.16% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.2% |
| Class F | 0.03% | -0.04% | 0.20% | 0.09% | -0.04% | 0.86% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |

Table 2. LowQP test (1,5,9,13)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | -0.01% | -0.01% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.04% | -0.03% | -0.04% | 0.02% | 0.02% | 0.03% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | -0.01% | -0.01% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | -0.01% | -0.02% | 0.00% | -0.01% | -0.02% | 0.00% |
| Class E | 0.01% | 0.01% | 0.00% | 0.00% | 0.00% | 0.02% |
| Overall | 0.00% | -0.01% | 0.00% | 0.00% | -0.01% | 0.00% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.03% | 0.02% | 0.00% | -0.11% | 0.04% | 0.01% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3. AHG5 total context reduction

|  | Bins relative to total bins |  |  |  | Total bins reduction ration |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test |  |  |  |  |  |
|  | ctx | bp |  |  |  |  |
| AI Main |  |  |  |  |  |  |
| RA Main | 98.57% | 103.39% |  |  |  | -0.10% |
| LB Main | 98.51% | 105.44% |  |  |  | -0.09% |
| LP Main |  |  |  |  |  |  |
| Average | 98.54% | 104.42% |  |  |  | -0.09% |
|  | Bins relative to total bins |  |  |  | Total bins reduction ration |  |
|  | Test |  |  |  |  |  |
|  | ctx | bp |  |  |  |  |
| AI HE10 |  |  |  |  |  |  |
| RA HE10 | 98.56% | 103.40% |  |  |  | -0.11% |
| LB HE10 | 98.50% | 105.40% |  |  |  | -0.07% |
| LP HE10 |  |  |  |  |  |  |
| Average | 98.53% | 104.40% |  |  |  | -0.09% |

# Conclusion

One context is removed for MVD coding. From the statistical analysis, it is desirable to code between ‘0’ and ‘1’ with bypass mode. Proposed method rearrange existing functions, no complexity is introduced. We recommend adopting it in the next HM.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Change

#### 7.3.7.1   Motion vector difference coding syntax

| mvd_coding( mvd_x, mvd_y ) { | Descriptor |
| --- | --- |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater1_flag[ 0 ] ) { |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } else { |  |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) |  |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if(abs_mvd_greater1_flag[1]) { |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } else { |  |
| abs_mvd_greater0_flag[1] | ae(v) |
| if(abs_mvd_greater0_flag[ 1 ] ) |  |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| mvd_x = abs_mvd_greater0_flag[ 0 ] * ( abs_mvd_minus2[ 0 ] + 2 ) * ( 1 − 2 * mvd_sign_flag[ 0 ] ) |  |
| mvd_y = abs_mvd_greater0_flag[ 1 ] * ( abs_mvd_minus2[ 1 ] + 2 ) * ( 1 − 2 * mvd_sign_flag[ 1 ] ) |  |
| } |  |

- 7.4.7.1 Motion vector difference coding semantics

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 0. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

abs_mvd_greater1_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 1. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

abs_mvd_minus2[ compIdx ] is the absolute value of a motion vector component difference minus 2. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows.

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

Table 9-1 – Values of variable initValue for abs_mvd_greater1_flag ctxIdx (Should modifying)

| Initialization variable | abs_mvd_greater1_flag ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 |  |  |
| initValue | 120 | 166 |  |  |

Table 9-34 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset (Should modifying)

| abs_mvd_greater0_flag[ ] | 1 | FL, cMax = 1 | na | Table 9-1na | na,(uses Decode Bypass) |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | na | Table 9-1na | na,(uses Decode Bypass) |
| abs_mvd_greater1_flag[ ] | 1 | FL, cMax = 1 | 0 | Table 9-1 | 0 |
|  | 2 |  | 0 | Table 9-1 | 1 |

Table 9-39– Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

|  | Table 9-1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| abs_mvd_greater1_flag[ ] | Table 9-1 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |