| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0318_r1 |
| --- | --- |

| Title: | Consideration on Chroma QP Range Extension for HEVC version 1 and 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Electronics / Sony Corp. |  |  |

_____________________________

# Abstract

When CPB is likely to overflow, one straightforward way to avoid it is to increase the value of QP. However, under the HEVC CD, upper bound of chroma QP is limited to 39, whereas luma QP can reach the value 51. This will become bottleneck for rate-control, especially if the sequence is in 4:2:2 or 4:4:4 format, although it happens in 4:2:0 cases.

To solve this problem, at the 8th JCTVC meeting the authors proposed to extend chroma QP range in JCTVC-H0400 by removing the luma-chroma QP mapping table, which is inherited from AVC specification. It was not adopted, as one expert claimed that the proposed method will break the default balance of luma-chroma quality in the common test conditions.

Therefore we submitted another proposal, JCTVC-I0265 at the 9th JCTVC meeting. In this proposal one more solution has been introduced, that does not break luma-chroma quality balance at the default cb/cr_qp_offset (=0). It was claimed that generated bits for chromaQP=39 may already be small and cannot be decreased even if higher value chromaQP is set. However we already showed the data in JCTVC-H0400 that this is not true.

This contribution requests to consider chroma QP range extension as one of the study items under AHG or CE for HEVC version 2, and items to be adopted for HEVC version 1. It will be preferable that chroma QP extraction process remains same with version-1 and version-2 specifications for the reason of compatibility.

# Discussions

When CPB is likely to overflow, one straightforward way to avoid it is to increase the value of QP. However, under the HEVC CD, upper bound of chroma QP is limited to 39, whereas luma QP can reach the value 51. This will become bottleneck for rate-control, especially if the sequence is in 4:2:2 or 4:4:4 format, although it happens in 4:2:0 cases.

To solve this problem, the authors proposed 2 methods in JCTVC-H0400 and in JCTVC-I0265 as follow:

Method A:

QPCb = Clip(0, 51, QPY+Cb_QP_offset)

QPCr = Clip(0, 51, QPY+Cr_QP_offset)

Method B:

QPCb = Clip(0, 51, g_aucChromaScale (QPY)+Cb_QP_offset)

QPCr = Clip(0, 51, g_aucChromaScale(QPY)+Cr_QP_offset)

Comparison of HM Method, Method A and Method B are shown in Fig 1.

- HM method

- Method A

- Method B

Fig 1.

Both with Method A and B, chroma QP can reach 51. Pros and cons of each methods are as follow:

Method A:

Pros: YtoC Table can be removed

Cons: Cb/Cr_QP_Offset values should be adjusted to remain luma-chroma quality balance with the common test conditions

Method B:

Pros: Luma-chroma quality balance remains same with Cb/Cr_QP_Offset=0

Cons: YtoC Table cannot be removed

One expert claimed that generated bits for chroma may be saturated at chroma QP=39 and even if its value is increased further bit-saving cannot be expected.

We already showed data in JCTVC-H0400 that this is not true as shown in Fig 2. This is the RD-curve with “Method A” at QP=38, 42, 46 and 50. With HM-5.0 bitrate is around 200kbps at LumaQP=50 and ChromaQP=39. With Method A bitrate is around 100kbps at LumaQP=50 and ChromaQP=50. It means that further bit reduction can be possible if ChromaQP goes beyond 39. Even with 4:2:0 this functionality is useful, and it will be still more useful for 4:2:2 and 4:4:4 videos.

LumaQP=(38, 42, 46, 50)

Fig 2.

There are also some contributions [4][5][6] addressed chroma QP problems in deblocking filter of chroma. In JCTVC-J0091[4], visual improvements can be observed by set chroma_qp_offset =12. The reason is that large QP is generated to obtain larger deblocking threshold tc for chroma deblocking. By removing the YtoC table as in Method A, chroma QP is actually increased and therefore deblocking threshold tc is increased. So we expect to have better quality in the scenario of deblocking. What’s more, Method A can also extend the QP range to [0, 51]. Thus more options on the values for tc are available in deblocking for chroma to address blocky artefacts appearing in chroma.

# Conclusion

This contribution requests to consider chroma QP range extension proposed by JCTVC-H0400 and JCTVC-I0265 as one of the study items under AHG or CE for HEVC version 2, and items to be adopted for HEVC version 1.

The proposed method will enables more flexibility especially to avoid CPB overflow when the buffer constraint is tight. It will be more useful for 4:2:2 / 4:4:4 videos but also useful with 4:2:0 videos.

# References

[1] J. Xu, A. Tourapis, E. Manni and K. Sato, “Chroma QP extension”, JCTVC-H0400, Feb 2012

[2] J. Xu, A. Tourapis, A. Tabatabai and K. Sato, “Chroma QP extension and signalling enhancement”, JCTVC-I0265, Apr 2012

[3] P. Silcock, K. Sharman, N. Saunders, and J. Gamei, “AHG12: Extension of HM7 to Support Additional Chroma Formats”, JCTVC-J0191, Jul 2012

[4] G. Van der Auwera, X. Wang, and M. Karczewicz, “AHG6: Chroma QP Offset and Deblocking”, JCTVC-J0091, July 2012.

[5] G. Van der Auwera, X. Wang, and M. Karczewicz, “AHG6: Chroma QP Offsets and Chroma Deblocking Filtering”, JCTVC-I0283, Apr 2012.

[6] S. Kanumuri and G. J. Sullivan, “Use of Chroma QP offsets in Deblocking”, JCTVC-J0343, July 2012.

# Patent rights declaration(s)

Sony Electronics / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).