| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0319 |
| --- | --- |

| Title: | Further Unification of Luma/Chroma Context Derivation for Last Position Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz Wei-Jung Chien 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In HM7, equation-based context derivation is used for last significant position coding where Luma and Chroma components share the same equation but with different parameters (offset and shift). This contribution proposes to further unify last coding context derivation by using the same shift parameter for Luma and Chroma components. Experimental results reportedly show there is 0.0% Y-BD rate change under common test conditions.

# Introduction

In HM 7.0 software, position of last significant coefficient (LAST) is encoded as last_significant_coeff_x and last_significant_coeff_y. Each element is encoded as two syntax elements as last_significant_coeff_X_prefix and last_significant_coeff_X_suffix, where X={x, y}. Bins in the prefix are encoded using contexts. Bins in the suffix are encoded using bypass mode..

The context derivation of last_significant_coeff_X_prefix is based on the following equation [1]

ctx_idx = m+ (bin_idx>>n)                                                  (1)

where m and n are offset and shift parameters.

The parameter values for Luma and Chroma components are:

|  | Luma | Chroma |
| --- | --- | --- |
| m | 3* k + ((k+1)>>2) | 15 |
| n | (k+3)>>2 | k |

where k =  log2TUSizeMinus2

The derived context index for last_significant_coeff_X_prefix  bins are shown in Table I and II.

Table I Context assignment for last_significant_coeff_X_prefix in HM7 (Luma)

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 3 | 4 | 4 | 5 |  |  |  |  |
| TU 16x16 | 6 | 6 | 7 | 7 | 8 | 8 | 9 |  |  |
| TU 32x32 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 |

Table II Context assignment for last_significant_coeff_X_prefix in HM7 (Chroma)

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 15 | 16 | 17 |  |  |  |  |  |  |
| TU 8x8 | 15 | 15 | 16 | 16 | 17 |  |  |  |  |
| TU 16x16 | 15 | 15 | 15 | 15 | 16 | 16 | 16 |  |  |

# Proposed solution

To further unify the context derivation process, this contribution proposes that chroma uses the same shift parameter  value as Luma.

|  | Luma | Chroma |
| --- | --- | --- |
| m | 3* k + ((k+1)>>2) | 15 |
| n | (k+3)>>2 | (k+3)>>2 |

The mapping between bin indices and context indices for Chroma components is shown in  Table III. Compared to HM7, the number of contexts is increased by 2 (1 for X and 1 for Y).

Table III Context assignment for last_significant_coeff_X_prefix for the proposed method (Chroma)

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 15 | 16 | 17 |  |  |  |  |  |  |
| TU 8x8 | 15 | 15 | 16 | 16 | 17 |  |  |  |  |
| TU 16x16 | 15 | 15 | 16 | 16 | 17 | 17 | 18 |  |  |

# Experimental results

The proposed method is implemented on top of HM 7.0. The performance evaluation is evaluated under common conditions as specified in [2]. The coding efficiency is measured using 4-point BD-Rate. The encoding time and decoding time are obtained from running on cluster with same CPU type.

Table IV a show the results of the proposed method under common test conditions. It can be seen essentially there is no change in BD-rate or encoding/decoding time.

# Conclusions

This contribution further unifies last coding context derivation process.  With the proposed method, Luma and Chroma use the same shift parameter in the context index calculation and only the offset parameter is different. It is recommended the proposed method integrated into HM.

Table IV BD-rate results of the proposed method (Anchor: HM7.0)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.01% | 0.01% | -0.01% | 0.01% | -0.02% | 0.01% |
| Class B | 0.01% | 0.09% | 0.05% | 0.00% | 0.10% | 0.02% |
| Class C | 0.00% | 0.06% | 0.05% | 0.00% | 0.07% | 0.04% |
| Class D | -0.01% | 0.14% | 0.09% | 0.00% | 0.09% | 0.00% |
| Class E | 0.00% | 0.05% | 0.05% | 0.00% | 0.00% | 0.07% |
| Class F | -0.01% | 0.08% | 0.02% | -0.02% | 0.04% | 0.03% |
| Overall | 0.00% | 0.07% | 0.04% | 0.00% | 0.05% | 0.03% |
|  | 0.00% | 0.07% | 0.04% | 0.00% | 0.04% | 0.04% |
| Enc Time[%] | 98.30% |  |  | 99.25% |  |  |
| Dec Time[%] | 101.14% |  |  | 99.69% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.03% | -0.36% | -0.05% | 0.01% | 0.12% | 0.10% |
| Class B | -0.02% | 0.06% | -0.07% | 0.00% | 0.03% | -0.02% |
| Class C | -0.04% | 0.13% | -0.05% | 0.00% | 0.10% | -0.02% |
| Class D | 0.02% | 0.24% | 0.14% | -0.04% | 0.06% | -0.15% |
| Class E |  |  |  |  |  |  |
| Class F | -0.04% | -0.01% | -0.05% | -0.05% | 0.08% | 0.12% |
| Overall | -0.02% | 0.01% | -0.02% | -0.02% | 0.07% | 0.01% |
|  | -0.03% | 0.02% | -0.03% | -0.02% | 0.05% | 0.01% |
| Enc Time[%] | 101.17% |  |  | 99.67% |  |  |
| Dec Time[%] | 101.38% |  |  | 98.43% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.26% | 0.39% | -0.01% | 0.21% | 0.07% |
| Class C | -0.04% | 0.11% | -0.04% | 0.03% | 0.05% | 0.22% |
| Class D | 0.00% | 0.28% | 0.20% | -0.07% | 0.13% | -0.25% |
| Class E | -0.06% | 0.24% | 0.05% | 0.03% | -0.36% | -1.18% |
| Class F | 0.06% | -0.10% | 0.71% | 0.11% | 0.20% | 0.63% |
| Overall | 0.00% | 0.16% | 0.28% | 0.02% | 0.08% | -0.04% |
|  | 0.00% | 0.16% | 0.25% | 0.02% | 0.09% | 0.12% |
| Enc Time[%] | 99.83% |  |  | 99.62% |  |  |
| Dec Time[%] | 99.81% |  |  | 99.91% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.03% | 0.32% | -0.27% | 0.00% | 0.33% | -0.46% |
| Class C | -0.01% | -0.10% | -0.32% | -0.01% | -0.09% | 0.20% |
| Class D | 0.09% | 0.42% | 0.51% | -0.02% | 0.24% | -0.30% |
| Class E | -0.07% | -0.11% | 0.82% | 0.00% | 0.22% | -1.71% |
| Class F | -0.07% | 0.41% | 0.51% | -0.13% | 0.57% | 0.01% |
| Overall | -0.02% | 0.21% | 0.19% | -0.03% | 0.26% | -0.39% |
|  | -0.02% | 0.19% | 0.16% | -0.03% | 0.30% | -0.30% |
| Enc Time[%] | 99.27% |  |  | 100.16% |  |  |
| Dec Time[%] | 98.84% |  |  | 97.81% |  |  |

# Draft Text

##### 9.2.3.1.2 Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

Inputs to this process are the binIdx, the colour component index cIdx, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc,

ctxIdxInc = m + (binIdx>>n)                                                                               (9-8)

where n = (log2TrafoSizeMinus2+3)>>2

When cIdx is equal to 0, m and n are is derived as follows

m = 3* log2TrafoSizeMinus2 + ((log2TrafoSizeMinus2+1)>>2)                         (9-9)

n = (log2TrafoSizeMinus2+3)>>2                                                                        (9-10)

When cIdx is greater than 0, m and n are is are derived as follows.

m = 15                                                                                                                    (9-11)

n = log2TrafoSizeMinus2                                                                                    (9-12)

where

–	for last_significant_coeff_x_prefix, log2TrafoSizeMinus2 is derived as follows

log2TrafoSizeMinus2 = log2TrafoWidth – 2

–	for last_significant_coeff_y_prefix, log2TrafoSizeMinus2 is derived as follows

log2TrafoSizeMinus2 = log2TrafoHeight - 2

# References

- L. Guo, M. Karczewicz, and W.-J. Chien “Table removal in contexts assignment of last significant position (LAST) coding”, JCTVC-I0331, Geneva, CH, April/May, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, Geneva, CH, April/May, 2012.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).