| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0320 |
| --- | --- |

| Title: | Multicore-friendly ALF luma region coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehul Tikekar Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | mtikekar@ti.com +1-214-480-1861 madhukar@ti.com sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The region coding scheme for luma ALF in HM-7.0 does not allow regions 0 and 15 to share filters although they are positioned adjacent to each other. This contribution proposes the addition of an extra flag to allow that share for a more coherent design. It is observed that the encoder chooses to share a filter for regions 0 and 15 in 39% of the test cases. For large picture sizes, a typical encoder may break the picture into sub-pictures to parallelize encoding using multi-core processors. It is asserted that allowing the aforementioned share enables the encoder to determine luma ALF parameters for each sub-picture independently and combine regions within the sub-pictures. Addition of the flag is observed to have a 0.0% change in the BD-Rate for common test conditions.

# Introduction

Adaptive Loop Filter (ALF) is the final step in the reconstruction process in High-Efficiency Video Coding. It is a 10-tap FIR filter with picture-adaptive and region-adaptive filter coefficients which are computed by the video encoder and signaled to the decoder in the bitstream. Typically, the encoder uses the Wiener filter technique to choose coefficients that minimize the SSE (sum of square error) between the filter output and encoder input.

At present, each picture can use up to 18 filter sets – 16 for luma (Y) and one each for the chroma components (Cb and Cr). For filtering the luma component, the picture is divided into 16 filter regions as shown below. One filter set can be signaled for each region.

| 0 | 1 | 4 | 5 |
| --- | --- | --- | --- |
| 15 | 2 | 3 | 6 |
| 14 | 11 | 10 | 7 |
| 13 | 12 | 9 | 8 |

Furthermore, to reduce the overhead of signaling filter sets in the bitstream, multiple filter regions can share the same filter set. This is achieved by combining the filter regions. A group of regions can be combined only if they have consecutive region indices. For example, in the figure above regions 2, 3, 4, 5 can share a filter set. The sharing is signaled using a 15-bit vector alf_filter_pattern_flag[i] (i = 0 … 14). alf_filter_pattern_flag[i] being 0 implies regions i and i +1 use the same filter; 1 implies they are separate.

So, region 0 starts with a new filter set and each 1 in alf_filter_pattern_flag implies the start of the next filter set. The number of luma filter sets (AlfNumFilters) is one more than the number of 1’s alf_filter_pattern_flag. The mapping between filter region and filter index is generated as MapFiltFidx[i] (i = 0 … 15).

AlfNumFilters = i alf_filter_pattern_flag[ i ] + 1 for i = 1..15

MapFiltFidx[0] = 0
MapFiltFidx[i] = MapFiltFidx[i – 1] + alf_filter_pattern_flag[i - 1] for i = 1 .. 15

One inconsistency in this scheme is that regions 0 and 15 cannot share a filter although they are adjacent to each other. This also has an effect on the encoder when sub-picture based processing is employed. An encoder may choose to divide a large picture into sub-pictures are show below.

The sub-pictures have half the width of the original picture which reduces the size of line buffers. In both cases, subpic 0 cannot use a single filter as regions 0 and 15 cannot be merged.

# Proposal

The proposed solution introduces an extra flag alf_filter_pattern_flag[0] which signifies regions 15 and 0 are merged iff the flag is 0. The number of luma filter sets and filter map are modified as follows:

AlfNumFilters = i alf_filter_pattern_flag[ i ]  for i = 0..15

MapFiltFidx[0] = 0
MapFiltFidx[i] = (MapFiltFidx[i – 1] + alf_filter_pattern_flag[i]) % AlfNumFilters   for i = 1 .. 15

(where % is the modulus operation)

The semantics of alf_filter_pattern_flag remain unchanged except that for region 0, region 15 is considered as the previous region in a circular fashion. This can be compactly represented by using the formula (i + 15) % 16 for the previous region index of region i. The modulus operation in MapFiltFidx ensures that filter regions including and after the final 1 in alf_filter_pattern_flag get a MapFiltFidx of 0 if alf_filter_pattern_flag[0] is 0.

With the addition of the flag, regions 15-0-1-2 or 11-12-13-14-15-0-1-2 can share the same filter which enable subpic 0 to use and signal a single filter set. The addition of the extra flag has been observed to have negligible (0.0%) impact on the bit-rate. At the same time, about 39% pictures in the test patterns are found to use this flag to merge 15 and 0.

# Change in CD Text

7.3.2.7 Adaptive loop filter picture info syntax

| … |  |
| --- | --- |
| for( i = 0; i < 16; i++ ) |  |
| alf_filter_pattern_flag[ i ] | u(1) |

7.4.2.7  Adaptive loop filter picture info semantics

alf_no_filters_idc (no change)

alf_start_second_filter (no change)

The number of filters AlfNumFilters is derived as follows.

- If alf_no_filters_idc is less than 2,

AlfNumFilters = alf_no_filters_idc + 1		(7-1)

- Otherwise (alf_no_filters_idc is equal to 2)

AlfNumFilters = i alf_filter_pattern_flag[ i ] with i = 0..15	(7-2)

alf_filter_pattern_flag [ i ] equal to 1 specifies that the filter index for the i-th RA mode index is increased by 1 modulo AlfNumFilters: equal to 0 specifies that the filter index for the i-th RA mode index is the same as (i + 15) % 16-th RA mode index.

The mapping between the i-th RA mode index for luma samples and the filter index MapFiltFidx [ i ] specifying is derived as follows:

- If AlfNumFilters is equal to 1, (no change)

- Otherwise if AlfNumFilters is equal to 2, (no change)

- Otherwise (AlfNumFilters is greater than 2),

MapFiltFidx [ 0 ] = 0
MapFiltFidx [ i ] =  (MapFiltFidx [ i-1 ] + alf_filter_pattern_flag [ i]) % AlfNumFilters	(7-3)
	with i = 1..15

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).