| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, Sweden, July 11-20, 2012 | Document: JCTVC-J0328 |
| --- | --- |

| Title: | AHG5: Cross-check of reduction of context bins and bypass bins grouping for cu_qp_delta (JCTVC-J0226) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vivienne Sze | Email: | sze@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

This contribution reports cross-check results from Texas Instruments for J0226.  The BD-rate results under AHG5 test conditions match results provided by Panasonic.

# Summary based on Analysis of Source Code

Binarization and context selection for cu_qp_delta is modified.  Two variations of the proposal were cross-checked.

In “Step 1” sign was moved from binIdx=1 of cu_qp_delta, to the end of cu_qp_delta to group the bypass bin.  The binarization of I0204 was used.

- binIdx=0, context coded with ctxIdx=0 (indicates if cu_qp_delta is non-zero)

- binIdx=1 to 4, context coded (binIdx=1 has ctxIdx=1, binIdx=2 to 4 has ctxIdx=2)

- binIdx>4, exp-golomb bypass coded bins

- sign bin

In “Step 2c” sign was moved from binIdx=1 of cu_qp_delta, to the end of cu_qp_delta to group the bypass bin.  The binarization of I0204 was used. And only the first bin in cu_qp_delta is context coded.

- binIdx=0, context coded with ctxIdx=0 (indicates if cu_qp_delta is non-zero)

- binIdx=1 to 4, bypass coded

- binIdx>4, exp-golomb bypass coded bins

- sign bin

# Experiment Results

The HM-7.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations

Results for Step 1

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.4% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class F | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 97% |  |  |
| Dec Time[%] | 103% |  |  | 96% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | 0.1% | 0.0% | 0.1% | 0.5% |
| Class C | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.5% | 0.5% | 0.0% | 0.4% | -0.2% |
| Class E | 0.0% | -1.0% | -0.7% | 0.0% | -0.5% | 1.3% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.3% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class F | 0.2% | 0.6% | 0.6% | 0.0% | 0.1% | 1.3% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Results for Step 2c

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class B | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class E | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class F | 0.2% | 0.3% | 0.2% | 0.2% | 0.3% | 0.0% |
| Enc Time[%] | 93% |  |  | 87% |  |  |
| Dec Time[%] | 92% |  |  | 90% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.4% |
| Class B | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.2% | 0.4% | 0.1% | 0.2% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.0% |
|  | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class F | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 95% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.1% | 0.4% | 0.5% |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% |
| Class D | 0.1% | 0.1% | 0.6% | 0.0% | -0.2% | 0.1% |
| Class E | 0.1% | 0.0% | 0.1% | 0.1% | 0.5% | 0.7% |
| Overall | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.3% |
|  | 0.1% | 0.0% | 0.1% | 0.1% | 0.3% | 0.2% |
| Class F | 0.3% | 0.5% | 0.2% | 0.4% | 0.3% | 1.4% |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 93% |  |  | 100% |  |  |