| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0330r1 |
| --- | --- |

| Title: | Inclusion of ALF in Main profile and additional test results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Yukinobu Yasugi Tomoyuki Yamamoto Takeshi Tsukuba Tomoko Aono 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides additional test results of ALF using twenty 1080p materials outside of common test sequences and asserts ALF inclusion in Main profile. The test materials are provided by ITE [The Institute of Image Information and Television Engineers ] and widely used in broadcasting and video products related companies in Japan. The test results reportedly show BD-rate gain is 2.8 % , 2.8 %, x,x %, 3.1 %, 3.3 % , 6.1 % in HELB and HERA, HELP, HE10LB and HE10RA, HE10LP compared to HM70 ALF off. Considering the gain in a variety of materials, it is recommended to include the ALF in the Main profile.

# Introduction

The performance of ALF has been improved by non-normative encoder optimization in JCTVC-J0048, where the coding performance was tested in common sequences and KTA sequences. In this contribution, other test materials, which were provided by ITE [The Institute of Image Information and Television Engineers ], were used to test the performance of ALF in a variety of HD sequences.

# Sequences

The simulated number of frames is 300 frames (5 sec.) while the length of original sequences is 900 frames. The start frame is 0.

Table 1: Test sequences (All 1920x1080p@60fps)

|  | Number | Name | Notes |
| --- | --- | --- | --- |
| 1 | s201 | Ginkgo trees | Avenue 0-89: No move 90-299: Zoom in |
| 2 | s202 | Truck train | Truck train 0-299: Right (slightly down) pan |
| 3 | s203 | Cosmos flowers | Cosmos garden with many people with pan 0-299: pan down |
| 4 | s204 | Red leaves (pan up) | Walking Person in the still background 0-299: Still background |
| 5 | s205 | Sunlight through leaves | Random motion with sunlight and pan 0-179: Still 180-299: pan left |
| 6 | s206 | Red leaves (pan down) | Random motion with speedup pan 0-299: pan right and down |
| 7 | s207 | Woman at harbor (circle dolly) | Still centered person with pan right background 0-299: Pan right(background), Person(Still) |
| 8 | s208 | Fountain (follow) | Walking person in the still background 0-208: Still background 209-299: Pan background |
| 9 | s209 | Fountain (dolly) | Walking person in the randome move foundain 0-299: Still camera |
| 10 | s210 | Studio concert (confetti) | Band concert with many ticker tapes 0-149: Pan 150-299: Zoom in |
| 11 | s211 | Studio concert | Drum person with zoom out and glittering lighting 0-299: Zoom out |
| 12 | s212 | Drama set (day) | Walking person with pan 0-299: Pan right |
| 13 | s213 | Drama set (night) | Walking person with still and dark background 0-299: Still |
| 14 | s214 | Basketball | Many moving players 0-169: Pan right 170-299: Still |
| 15 | s215 | Evening scene (zoom out) | Glittering ferris wheel in the dark scnene 0-299: Zoom out |
| 16 | s216 | Evening scene (fixed) | Glittering ferris wheel and building in the dark scnene 0-299: Still |
| 17 | s217 | Night scene (fixed) | Glittering ferris wheel and building in the dark scnene 0-299: Still |
| 18 | s218 | Horse racing (dirt) | Many horses with pan 0-299: Pan right |
| 19 | s264 | Flash photography | Almost still person in black background 0-299: Still |
| 20 | s265 | Fountain (chromakey) | Swing person in the waterflow background 0-299: Still |

# Experimental results

Comparison ALF on vs ALF off was experimented. Test results are provided here as well as the attached excel file.

Table 2: Test results (Summary)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| s201-s204 | -2.9% | -0.9% | -2.1% | -3.2% | -1.3% | -2.6% |
| s205-s208 | -2.7% | -0.8% | -3.2% | -3.0% | -1.2% | -3.8% |
| s209-s212 | -2.1% | -1.1% | -1.8% | -2.4% | -1.5% | -1.6% |
| s213-s216 | -2.5% | -1.9% | -1.9% | -2.8% | -2.7% | -2.2% |
| s217-s265 | -3.7% | -5.6% | -1.5% | -3.9% | -8.6% | -2.0% |
| Overall | -2.8% | -2.1% | -2.1% | -3.1% | -3.1% | -2.4% |
|  | -2.8% | -2.0% | -2.1% | -3.1% | -3.1% | -2.5% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 118% |  |  | 116% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| s201-s204 | -2.9% | -0.8% | -2.1% | -3.6% | -2.3% | -3.9% |
| s205-s208 | -2.4% | -1.4% | -3.0% | -2.8% | -2.5% | -3.8% |
| s209-s212 | -2.0% | -1.1% | -1.7% | -2.3% | -5.5% | -1.8% |
| s213-s216 | -3.2% | -2.7% | -1.9% | -3.6% | -2.8% | -2.4% |
| s217-s265 | -3.7% | -2.8% | -1.7% | -4.1% | -6.8% | -2.8% |
| Overall | -2.8% | -1.8% | -2.1% | -3.3% | -4.0% | -2.9% |
|  | -2.8% | -1.8% | -2.3% | -3.3% | -4.0% | -2.9% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 113% |  |  | 116% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| s201-s204 | -5.4% | -1.6% | -2.3% | -6.7% | -4.0% | -2.9% |
| s205-s208 | -6.1% | -3.2% | -3.5% | -7.6% | 0.0% | 0.0% |
| s209-s212 | -3.5% | -1.5% | -1.9% | -4.6% | 0.0% | 0.0% |
| s213-s216 | -4.4% | -4.3% | -2.2% | -5.1% | 0.0% | 0.0% |
| s217-s265 | #VALUE! | #VALUE! | #VALUE! | -6.3% | 0.0% | 0.0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -6.1% | -6.2% | -4.0% |
|  | #VALUE! | #VALUE! | #VALUE! | -6.1% | -6.4% | -4.0% |
| Enc Time[%] | #NUM! |  |  | 101% |  |  |
| Dec Time[%] | 121% |  |  | 116% |  |  |

Table 3: Test results (Detail)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| s201 | -2.5% | -1.2% | -2.9% | -2.8% | -1.7% | -3.4% |
| s202 | -2.0% | -0.6% | -1.5% | -2.5% | -1.0% | -2.5% |
| s203 | -4.6% | -0.7% | -2.6% | -4.8% | -0.5% | -2.7% |
| s204 | -2.4% | -1.0% | -1.3% | -2.7% | -2.2% | -1.6% |
| s205 | -2.9% | -1.0% | -2.0% | -3.2% | -1.1% | -1.9% |
| s206 | -4.7% | -0.3% | -6.1% | -5.1% | -1.1% | -6.9% |
| s207 | -1.2% | -1.6% | -1.4% | -1.5% | -1.6% | -2.0% |
| s208 | -1.9% | -0.4% | -3.1% | -2.3% | -1.2% | -4.4% |
| s209 | -2.0% | -2.3% | -2.6% | -2.2% | -3.6% | -0.9% |
| s210 | -2.4% | -0.5% | -1.6% | -2.6% | -0.2% | -1.6% |
| s211 | -2.4% | 0.1% | -2.4% | -2.8% | -0.6% | -2.7% |
| s212 | -1.8% | -1.6% | -0.8% | -2.1% | -1.7% | -1.3% |
| s213 | -2.4% | -3.3% | -1.6% | -3.2% | -6.6% | -2.1% |
| s214 | -3.3% | -3.4% | -2.9% | -3.4% | -3.2% | -3.1% |
| s215 | -1.9% | 0.1% | -2.8% | -1.9% | 0.0% | -2.4% |
| s216 | -2.3% | -1.2% | -0.3% | -2.8% | -1.2% | -1.2% |
| s217 | -2.2% | -0.7% | -0.3% | -2.3% | -1.5% | -0.5% |
| s218 | -2.0% | -3.0% | -1.9% | -2.5% | -3.6% | -2.2% |
| s264 | -2.3% | -1.6% | -0.9% | -2.4% | -2.1% | -0.8% |
| s265 | -8.6% | -17.0% | -2.8% | -8.5% | -27.1% | -4.5% |
| Overall | -2.8% | -2.1% | -2.1% | -3.1% | -3.1% | -2.4% |
|  | -2.8% | -2.0% | -2.1% | -3.1% | -3.1% | -2.5% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 118% |  |  | 116% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| s201 | -1.4% | -0.7% | -1.4% | -1.8% | -1.4% | -2.0% |
| s202 | -5.1% | -1.9% | -3.5% | -6.4% | -4.6% | -7.6% |
| s203 | -2.9% | -0.1% | -1.0% | -3.2% | -2.0% | -2.0% |
| s204 | -2.4% | -0.7% | -2.5% | -2.9% | -1.3% | -3.9% |
| s205 | -2.6% | -1.0% | -1.3% | -2.9% | -1.6% | -1.4% |
| s206 | -4.1% | -1.2% | -5.4% | -4.3% | -3.1% | -6.2% |
| s207 | -1.3% | -1.6% | -1.8% | -1.6% | -2.9% | -2.3% |
| s208 | -1.6% | -1.7% | -3.7% | -2.3% | -2.6% | -5.5% |
| s209 | -1.9% | 0.3% | -2.7% | -1.9% | -13.3% | -1.1% |
| s210 | -2.2% | -1.3% | -1.2% | -2.5% | -2.1% | -1.6% |
| s211 | -1.9% | -1.3% | -1.8% | -2.8% | -4.5% | -2.9% |
| s212 | -2.0% | -2.3% | -1.0% | -1.9% | -2.1% | -1.5% |
| s213 | -4.6% | -3.7% | -3.3% | -5.6% | -4.6% | -3.9% |
| s214 | -4.1% | -4.6% | -3.7% | -4.1% | -4.4% | -3.5% |
| s215 | -1.4% | -0.9% | -0.7% | -1.7% | -1.4% | -1.1% |
| s216 | -2.8% | -1.7% | -0.1% | -3.1% | -0.8% | -1.1% |
| s217 | -2.7% | -1.9% | -0.8% | -3.3% | -5.2% | -2.2% |
| s218 | -2.2% | -4.3% | -2.2% | -2.8% | -5.9% | -3.0% |
| s264 | -1.6% | -1.8% | -0.9% | -1.8% | -3.2% | -0.7% |
| s265 | -8.3% | -3.1% | -2.7% | -8.6% | -13.0% | -5.3% |
| Overall | -2.8% | -1.8% | -2.1% | -3.3% | -4.0% | -2.9% |
|  | -2.8% | -1.8% | -2.3% | -3.3% | -4.0% | -2.9% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 113% |  |  | 116% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| s201 | -2.2% | -1.2% | -1.6% | -2.9% | -2.1% | -2.4% |
| s202 | -10.2% | -2.8% | -4.1% | -12.9% | -5.8% | -8.1% |
| s203 | -6.3% | -1.3% | -1.9% | -7.3% | -3.7% | -3.2% |
| s204 | -2.9% | -1.0% | -1.8% | -3.6% | -2.2% | -2.7% |
| s205 | -4.3% | -1.9% | -2.0% | -5.1% | -2.9% | -2.5% |
| s206 | -11.4% | -2.3% | -5.9% | -12.8% | -5.1% | -7.0% |
| s207 | -3.1% | -3.9% | -2.1% | -4.6% | -5.1% | -4.3% |
| s208 | -5.5% | -4.6% | -3.9% | -7.9% | -3.9% | -5.3% |
| s209 | -2.9% | 2.4% | -2.3% | -3.2% | -11.6% | -1.8% |
| s210 | -3.2% | -2.2% | -1.2% | -4.0% | -3.4% | -1.8% |
| s211 | -4.7% | -2.8% | -2.3% | -6.9% | -7.6% | -3.5% |
| s212 | -3.1% | -3.4% | -2.1% | -4.5% | -5.6% | -4.7% |
| s213 | -4.7% | -5.8% | -2.2% | -5.3% | -9.6% | -4.5% |
| s214 | -5.2% | -6.2% | -4.3% | -5.5% | -5.3% | -4.6% |
| s215 | -1.8% | -0.8% | 0.0% | -2.8% | -2.4% | -0.6% |
| s216 | -5.9% | -4.2% | -2.4% | -6.8% | -3.7% | -3.7% |
| s217 | -2.7% | -1.8% | -0.9% | -4.0% | -6.0% | -2.6% |
| s218 | -5.2% | -6.9% | -5.2% | -7.7% | -11.4% | -7.9% |
| s264 | -3.5% | -2.7% | -2.1% | -4.8% | -5.7% | -3.9% |
| s265 | #VALUE! | #VALUE! | #VALUE! | -8.8% | -21.8% | -5.3% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -6.1% | -6.2% | -4.0% |
|  | #VALUE! | #VALUE! | #VALUE! | -6.1% | -6.4% | -4.0% |
| Enc Time[%] | #NUM! |  |  | 101% |  |  |
| Dec Time[%] | 121% |  |  | 119% |  |  |

# Conclusion

The ALF mode shows significant gain in the additional HD test materials. It is recommended to include the ALF in the main profile.

# References

[1] C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, T. Yamakage, T. Itoh, T. Chujoh, I. S. Chong, M. Karczewicz: “AHG6: ALF with non-normative encoder-only improvements,” JCTVC-J0048, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 10th Meeting: Stockholm, SE, 11–20 July 2012

[2] T. Yamakage, Y.-W. Huang, I. S. Chong, “Inclusion of adaptive loop filtering (ALF) in Main profile of the HEVC standard”, JCTVC-J0307, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 10th Meeting: Stockholm, SE, 11–20 July 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).