| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0331 |
| --- | --- |

| Title: | AHG5: Cross check of Bin reduction for delta QP coding ( JCTVC-J0089 ) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba JAPAN | Tel: Email: | +81 43 299 8526 tsukuba.takeshi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides cross-check results for the proposal in JCTVC-J0089. In JCTVC-J0089, it is proposed to use truncated unary with cMax=5 as the prefix and 0th order Exp-Golomb coding as the suffix for cu_qp_delta coding. The verification was done successfully and the results exactly match those provided by the proponents.

# Verification

In JCTVC-J0089[1], it is proposed to use truncated unary with cMax=5 as the prefix and 0th order Exp-Golomb coding as the suffix for cu_qp_delta coding. As a result, regarding cu_qp_delta coding, the total context coded bins and the total bins in worst case reduces from 27 bins to 15 bins and from 27 bins to 15 bins respectively. In addition, the number of context for cu_qp_delta reduces from 3 to 2.

The proponents provided the HM source codes of their proposal described in [1]. A verification is carried by setting following macros in the provided code:

Test1:

#define CU_DQP_TU_EG       1

#if (CU_DQP_TU_EG)

#define CU_DQP_TU_CMAX    5         //max number bins for truncated unary

#define CU_DQP_EG_k         0         //expgolomb order

#endif

For cu_qp_delta coding, HM-7.0 was used as an anchor under the conditions set by AHG5 on entropy coding improvements as follows:

MaxCuDQPDepth        3   # Max depth of a minimum CuDQP for sub-LCU-level delta QP 
AdaptiveQP             1   # QP adaptation based on a psycho-visual model (0: None, 1: TM5Step3) 
MaxQPAdaptationRange  12  # dQP range of QP adaptation

# Experimental Results

Table 1 shows the summary of the experimental results on coding efficiency. Table 2 shows the AHG5 summary. It is confirmed that the results on exactly match those provided by the proponents.

Table 3 shows the summary of the experimental worst case bins per pixel ( informative ).

Table 1: Performance on common condition (QP=22, 27, 32, 37)

Table 2: AHG5 summary

Table 3: (informative) The Experimental Worst case Bins Per Pixel

# Conclusion

The cross-checkers results perfectly match those of the proponents.

# Reference

- V. Sze, M. Budagavi, V. Seregin, J. Sole, M. Karczewicz, “Bin reduction for delta QP coding,” JCTVC-J0089, Stockholm, July 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-H1100, San Jose, Feb. 2012.