| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0334 |
| --- | --- |

| Title: | Adding a Level Restriction on Coding Tree Block Size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA | Tel: Email: Tel: Email: | +1-949-926-6387 wwan@broadcom.com +1-978-719-1348 timhellman@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution recommends a level-specific limit on Coding Tree Block (CTB) size to facilitate high-performance decoder implementations.  Specifically, it proposes to limit the minimum CTB size to 32x32 for level 5 and higher.  It claims that this limit reduces line buffer storage requirements and increases decoder performance, and that such a limit has negliglble coding efficiency impact and only a minor impact on low-latency implementations.

# Problem Statement

The HEVC standard allows three CTB sizes: 16x16, 32x32 and 64x64.  These sizes represent a trade-off between  coding efficiency (larger CTB) and encoder/decoder latency (smaller CTB).  However, the small CTB size comes with a cost to pipelined decoders.  Specifically, every pipeline stage has a certain amount of overhead per CTB for initialization and setup.  The overhead time is fixed regardless of CTB size, so its impact is 4 times larger for a 16x16 CTB versus a 32x32 CTB.

Figure 1 - Processing Time for 16x16 CTB vs 32x32 CTB

The cumulative overhead for the 16x16 CTB can represent up to a 10% increase in overall worst-case decode time as compared to a 32x32 CTB.  This is manageable for the smaller frame sizes, but increases the worst-case processing requirements for large frame decoders, which must be guaranteed to work under all allowable stream conditions.

In addition, the small CTB size increases the amount of on-chip line buffering for the SAO filter implementation.  SAO parameters are sent per CTB, and a full line’s worth must be stored.  Eliminating the smallest CTB size from large pictures cuts the size of this line buffer in half, saving over 1K byte of memory for a 4K wide picture.

As noted above, the only advantage of the smaller CTB size is a reduced lower bound for encode and decode latency, since an entire CTB row must be captured before it can be encoded.  For large frame sizes, however, the advantage of the 16x16 CTB size is fairly minimal.  For a 4Kx2K stream @ 30 frames per second, the difference in the latency lower bound is 0.2 milliseconds, or 0.7% of a frame time.

# Conclusion

The performance advantages and line buffer savings offered by restricting minimum CTB size for large pictures are substantial, and the difference in best-case latency is tiny.  Therefore this contribution recommends adoption of a minimum 32x32 CTB size limit for level 5 and higher.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003_d4, June 2012.

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes

Section A.4.2

g)   In bitstreams conforming to the Main profile, the variable Log2CtbSize must be 5 or greater.