| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0335 |
| --- | --- |

| Title: | Explicit Specification of Bounds on Syntax Elements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: Tel: Email: | +1-949-926-5388 bheng@broadcom.com +1-978-719-1348 timhellman@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution identifies syntax elements in the current working draft that are not fixed-length or truncated codes and are believed to be missing explicit limits or there is a potentially a problem with the limits currently specified.  Constraints for these syntax elements are proposed in this contribution to facilitate discussion to appropriately bound the values of these syntax elements.

# Problem Statement

There are various syntax elements in the current working draft that are missing explicit limits on their potential values or have potential problems with the limits currently specified in the working draft text [1].

The following table provides a list of these syntax elements along with proposed limits to facilitate discussion and agreement on bounding the allowable values for these elements.

| Item | Syntax Element | Type | Min Value | Max Value | Proposed Min | Proposed Max | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | max_transform_hierarchy_depth_inter | ue(v) | 0 | ?? | 0 | Log2MaxCtbSize – Log2MinTrafoSize |  |
| 2 | max_transform_hierarchy_depth_intra | ue(v) | 0 | ?? | 0 | Log2MaxCtbSize – Log2MinTrafoSize |  |
| 3 | cb_qp_offset | se(v) | ?? | ?? | -12 | 12 | Consistent with AVC |
| 4 | cr_qp_offset | se(v) | ?? | ?? | -12 | 12 | Consistent with AVC |
| 5 | column_width | ue(v) | ?? | ?? | 1 | Indirectly bound by requiring sum of column widths adds up to picture width |  |
| 6 | row_height | ue(v) | ?? | ?? | 1 | Indirectly bound by requiring sum of row heights adds up to picture height |  |
| 7 | beta_offset_div2 | se(v) | ?? | ?? | -6 | 6 | Consistent with AVC |
| 8 | tc_offset_div2 | se(v) | ?? | ?? | -6 | 6 | Consistent with AVC |
| 9 | scaling_list_pred_matrix_id_delta | ue(v) | 0 | ?? | 0 | Indirectly bound by requiring RefMatrixID >= 0 |  |
| 10 | scaling_list_dc_coef_minus8 | se(v) | ?? | ?? | -8 | 247 | Prevents DC from going outside 0-255 range |
| 11 | scaling_list_delta_coef | se(v) | ?? | ?? | -128 | 127 | Consistent with AVC |
| 12 | alf_start_second_filter | ue(v) | ?? | ?? | 1 | 15 | Suggest to remove. Unclear whether using only two filters needs special signaling |
| 13 | five_minus_max_num_merge_cand | ue(v) | 0 | 5? | 0 | 4 | 5 implies zero merge candidates which is unclear |
| 14 | offset_len_minus1 | ue(v) | ?? | ?? | 0 | 31 |  |
| 15 | delta_idx_minus1 | ue(v) | 0 | ?? | 0 | Indirectly bound by requiring RIdx >= 0 |  |
| 16 | abs_mvd_minus2 | EGk(v) | 0 | ?? | 0 | Indirectly bound by requiring -both mvd_x and mvd_y be in the range [-215, 215-1] | Also need to bound motion vector (mvLX and mvLY) to range [-215, 215-1] |

Red text indicates proposed changes

# Draft Text Changes

## max_transform_hierarchy_depth_inter

Section 7.4.2.2 (Sequence parameter set RBSP semantics)

max_transform_hierarchy_depth_intra specifies the maximum hierarchy depth for transform blocks of coding blocks coded in intra prediction mode.  The value of max_transform_hierarchy_depth_intra shall be in the range of 0 to (Log2MaxCtbSize – Log2MinTrafoSize), inclusive.

## max_transform_hierarchy_depth_intra

Section 7.4.2.2 (Sequence parameter set RBSP semantics)

max_transform_hierarchy_depth_inter specifies the maximum hierarchy depth for transform units of coding units coded in inter prediction mode.  The value of max_transform_hierarchy_depth_inter shall be in the range of 0 to (Log2MaxCtbSize – Log2MinTrafoSize) , inclusive.

## cb_qp_offset

Section 7.4.2.3 (Picture parameter set RBSP semantics)

cb_qp_offset, cr_qp_offset are specifying the offset to the luma quantization parameter QP’Y used for deriving QP’Cb and QP’Cr , respectively.  The value of cb_qp_offset shall be in the range of -12 to 12, inclusive.

## cr_qp_offset

Section 7.4.2.3 (Picture parameter set RBSP semantics)

cb_qp_offset, cr_qp_offset are specifying the offset to the luma quantization parameter QP’Y used for deriving QP’Cb and QP’Cr , respectively.  The value of cr_qp_offset shall be in the range of -12 to 12, inclusive.

## column_width

Section 7.4.2.3 (Picture parameter set RBSP semantics)

column_width[ i ] specifies the width of the i-th tile column in units of coding tree blocks.  The value of column_width shall not be equal to 0. [TBD: how to indirectly bound max by requiring sum of column widths add up to picture width]

## row_height

Section 7.4.2.3 (Picture parameter set RBSP semantics)

row_height[ i ] specifies the height of the i-th tile row in units of coding tree blocks.  The value of row_height shall not be equal to 0. [TBD: how to indirectly bound max by requiring sum of row heights add up to picture height]

## beta_offset_div2

Section 7.4.2.3 (Picture parameter set RBSP semantics)

beta_offset_div2 and tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied for pictures referring to the picture parameter set unless the default deblocking parameter offsets are overriden by the deblocking parameter offsets present in the slice header for pictures referring to the picture parameter set.  The value of beta_offset_div2 shall be in the range of -6 to 6, inclusive.

Section 7.4.3 (Slice header semantics)

beta_offset_div2 and tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the current slice.    The value of beta_offset_div2 shall be in the range of -6 to 6, inclusive.

## tc_offset_div2

Section 7.4.2.3 (Picture parameter set RBSP semantics)

beta_offset_div2 and tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied for pictures referring to the picture parameter set unless the default deblocking parameter offsets are overriden by the deblocking parameter offsets present in the slice header for pictures referring to the picture parameter set.  The value of tc_offset_div2 shall be in the range of -6 to 6, inclusive.

Section 7.4.3 (Slice header semantics)

beta_offset_div2 and tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the current slice.    The value of tc_offset_div2 shall be in the range of -6 to 6, inclusive.

## scaling_list_pred_matrix_id_delta

Section 7.4.2.4 (Scaling list data semantics)

scaling_list_pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When scaling_list_pred_mode_flag is equal to 0, scaling_list_pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following:

RefMatrixID = MatrixID – scaling_list_pred_matrix_id_delta (7-23)

where MatrixID is specified in Table 7-3. The variable RefMatrixID must be greater than zero. When scaling_list_pred_matrix_id_delta is equal to 0, i.e., RefMatrixID is equal to MatrixID, scalingList is inferred from the default scaling list ScalingList[ SizeID ][ MatrixID ] as specified in Table 7-4 and Table 7-5.

## scaling_list_dc_coef_minus8

Section 7.4.2.5 (Scaling list semantics)

scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] plus 8 specifies the DC value of the scaling list for 16x16 size when sizeID is equal to 2 and specifies the DC value of the scaling list for 32x32 size when sizeID is equal to 3. The value of scaling_list_dc_coef_minus8 shall be in the range of -8 to 247, inclusive.

## scaling_list_delta_coef

Section 7.4.2.5 (Scaling list semantics)

scaling_list_delta_coef specifies the difference of the matrix coefficient from the previous matrix coefficient, when pred_mode is equal to 1. The value of scaling_list_delta_coef shall be in the range of -128 to 127, inclusive.

## alf_start_second_filter

Section 7.4.2.7 (Adaptive loop filter picture info semantics)

alf_start_second_filter specifies the region adaptive (RA) mode index of luma samples for which the second filter is applied.  The value of alf_start_second_filter shall be in the range of 1 to 15, inclusive.

## five_minus_max_num_merge_cand

Section 7.4.3 (Slice header semantics)

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5. The maximum number of merging MVP candidates, MaxNumMergeCand is computed as

MaxNumMergeCand = 5 − five_minus_max_num_merge_cand 				(7-45)

The value of five_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 0 to 54, inclusive.

## offset_len_minus1

Section 7.4.3 (Slice header semantics)

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset[ i ] syntax elements.  The value of offset_len_minus1 shall be in the range of 0 to 31, inclusive.

## delta_idx_minus1

Section 7.3.3.1 (Short-term reference picture set syntax)

delta_idx_minus1 plus 1 specifies the difference between the index of the reference picture set of the current picture and the index of the reference picture set used for inter reference picture set prediction.

The variable RIdx shall be derived as follows.

RIdx = idx − (delta_idx_minus1+1) (7-46)

The variable RIdx must be greater than zero.

## abs_mvd_minus2

Section 7.4.7 (Prediction unit semantics)

mvd_l0[ x0 ][ y0 ][ compIdx ], specifies the difference between a list 0 vector component to be used and its prediction. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1. The value of mvd_l0 shall be in the range of -215 to 215-1, inclusive.

When any of the two components is not present, the inferred value is 0.

mvd_l1[ x0 ][ y0 ][ compIdx ] has the same semantics as mvd_l0, with l0 and list 0 replaced by l1 and list 1, respectively.  The value of mvd_l1 shall be in the range of -215 to 215-1, inclusive.

Section 8.5.2.1 (Derivation process for motion vector components and reference indices)

Add to the end of the section: The variable mvLX shall be in the range of -215 to 215-1, inclusive.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003_d4, June 2012.

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).