| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0336r3 |
| --- | --- |

| Title: | Clarification of the semantics of no_residual_data_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhijie Yang Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Peisong Chen Broadcom Corporation 16340 West Bernardo Dr San Diego, CA 92127 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA; Tel: Email: Tel: Email:; +1-949-926-6387 wwan@broadcom.com +1-978-719-1348 timhellman@broadcom.com. | Tel: Email: Tel: Email: Tel: Email: | +1-949-926-9552 zhyang@broadcom.com +1-858-521-4835 peisongc@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution recommends clarifying the semantics of no_residual_data_flag to avoid potential ambiguities in interpretation of this element and also prohibit an error condition in the current HM’s handling of this flag.

# Problem Statement

In the current HEVC specification [1], the semantics of no_residual_data_flag is defined as follows:

no_residual_data_flag equal to 1 specifies that no residual data are present for the current coding unit. no_residual_data_flag equal to 0 specifies that residual data are present for the current coding unit.

When no_residual_data_flag is not present, its value is inferred to be equal to 0.

The meaning of “no_residual_data_flag equal to 0 specifies that residual data are present” is not well defined and could be interpreted in two different ways:

Interpretation #1: “residual data are present” could mean there exists residual syntax element(s) in the current CTB such as cbf_cb, cbf_cr, etc.

OR

Interpretation #2: “residual data are present” could mean there are one or more non-zero transform coefficients in the current CTB.

These two interpretations have conflicting meaning in some situations.

Consider the example shown in Figure 1 of a 64x64 CTB with a partition type equal to 2Nx2N and merge_flag equal to 1. The value of no_residual_data_flag is inferred to be equal to 0 and the first layer split transform flag is inferred to be equal to 1. Furthermore, all four cbf_luma flags, one cbf_cb flag and one cbf_cr flag are equal to 0:

Figure 1: A 64x64 CTB with all cbf flags equal to 0

According to the Interpretation #1, the stream is compliant because there exists residual syntax element(s) in the current CTB.

According to the Interpretation #2, the stream is NOT compliant because all the transform coefficients are zero.

Therefore, this example demonstrates the ambiguity of the current definition of no_residual_data_flag.

Furthermore, the current HM software cannot handle the example described above correctly when we use the Interpretation#1 and will instead incorrectly derive the context index of the skip flag [2].

The appropriate changes to the HEVC standard text by following the Interpretation #1 are provided in Section 5 of this contribution.

Furthermore, current HM software uses QtRootCbf instead of no_residual_data_flag for this flag. The semantics of QtRootCbf is opposite to that of no_residual_data_flag, which makes the HM and text inconsistent.

Conclusion

The semantics of no_residual_data_flag in the current specification has the potential to be ambiguous.

This contribution describes an example where this may be the case and recommends a clarification of the semantics to avoid potential ambiguities and also prohibit an error condition in the current HM’s handling of this flag.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003_d4, June 2012.

- http://hevc.kw.bbc.co.uk/trac/ticket/573

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes

no_residual_data_flag equal to 1 specifies that transform_tree() syntax is not present for the current coding unit. no_residual_data_flag equal to 0 specifies that transform_tree() syntax is present for the current coding unit.

When no_residual_data_flag is not present, its value is inferred to be equal to 0.