| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0337 |
| --- | --- |

| Title: | Fix for ALF Padding Process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peisong Chen Broadcom Corporation 16340 West Bernardo Dr San Diego, CA 92127 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-858-521-4835 peisongc@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

Virtual boundary processing has been proposed and adopted into the HEVC draft text to remove the line buffer requirement for ALF processing in LCU-based decoder implementations. The idea behind virtual boundary processing is to adjust the filter support depending on the filter location such that the last several LCU rows do not need to be stored in line buffers and wait to process these rows until the bottom neighboring LCUs become available. It is reported in this contribution that the current padding defined for chroma processing contradicts this purpose. This contribution provides a fix to this problem.

# Problem Statement

Virtual boundary processing [1] has been proposed to remove ALF line buffer requirements in LCU-based decoding implementations. Figure 1 illustrates the virtual boundary of luma ALF. Pixels of each LCU are classified into three different regions.  Since ALF uses 9x7 2D FIR filters, filtering on the pixels on one side of a region boundary may need to access pixels on the other side of the region boundary. Virtual boundary processing was designed to remove this dependency.

Figure 1: Virtual boundary for a luma block.

illustrates the virtual boundary of chroma ALF. The difference between Figure 1 and   is the height of region R3.

Figure 2: Virtual boundary for a chroma block.

The padding processing may use pixels from neighboring CTBs which would reintroduce a dependency across the virtual boundary. For example, to filter the pixel which is located on the left edge of block 1 and three pixels away from block 1’s bottom edge pixels as indicated by the “+” in Figure 3, one need to use the padded pixels generated based on the top edge pixels of block 2, which are not available yet. This means it is not possible to filter the pixel “+” until block 2 becomes available. Therefore, there is still a need to use a line buffer to store pixel “+” which cannot be processed until block 2 becomes available.

Figure 3: original left extrapolation.

# Proposed Solution

A modification of the padding process for chroma is proposed to remove the current dependency and resulting line buffer requirement. When chroma top edge pixels are extrapolated upwards, it is proposed to extrapolate by two rows, except when the top edge is also the picture boundary, where it is proposed to extrapolate by three rows.

The proposed padding process is illustrated in Figure 4.

Figure 4: modified left extrapolation.

The corresponding change to the HEVC draft [2] is highlighted in the following (assuming slice_granularity equal to 0):

2. The left margin is derived as follows:

  - If the left block is marked as “available for ALF",

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB+x, yB+y ] for x = −1..− 4 and y = 0..(gbSize−1)

  - Otherwise (the left block is marked as “not available for ALF”),

  - If the below-left block is marked as "available for ALF" and one or more of the following conditions are true,

    - the left boundary of the current block does not coincide with an CTB boundary.

    - both the left and bottom boundaries of the current block coincide with CTB boundaries.

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB, yB+y ] for x = −1..−4 and y = 0..(gbSize− 42−1)

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB+x, yB+gbSize ] for x = −1..− 4 and y= (gbSize−42)..(gbSize−1)

  - Otherwise, the following applies:

padBlock[ xD+4+x, yD+4+y ] = recPicture[ xB, yB+y ] for x = −1..− 4 and y = 0..(gbSize−1)

# Experimental results

To evaluate the performance impact, high efficiency coding configuration is used with the following changes:

--SliceMode=2 --LFCrossSliceBoundaryFlag=0.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 145% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.0% | -0.2% |
| Class D | 0.0% | -0.4% | 0.6% |
| Class E | 0.1% | 0.0% | 0.4% |
| Overall | 0.0% | -0.1% | 0.2% |
|  | 0.0% | -0.1% | 0.2% |
| Class F | 0.1% | -0.5% | -0.1% |
| Enc Time[%] | 74% |  |  |

The performance impact of the proposed change is negligible. Please be advised that the runtime may not be accurate because of heterogeneous clusters.

# Conclusion

The current ALF padding processing defined in the HEVC draft may inadvertently require pixels from neighboring CTBs that are unavailable, thus reintroducing a dependency across the virtual boundary and the need for line buffer.  A fix has been provided in this contribution to avoid this problematic corner case and allow virtual buffer processing to eliminate the need for additional line buffers. We recommend adopting this fix into the HEVC standard.

# References

- C.-Y. Chen, et. al, “CE8.c.6: Virtual boundary processing of SAO and ALF for removing line buffers,” Document of Joint Collaborative Team on Video Coding, JCTVC-G211, November 2011.

- B. Bross, W-J. Han et al, “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003_d5, Geneva, Switzerland, May 2012

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).