| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0338 |
| --- | --- |

| Title: | Specification of Parallelization Tools in HEVC Version 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA | Tel: Email: Tel: Email: | +1-949-926-6387 wwan@broadcom.com +1-978-719-1348 timhellman@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

Several parallelization techniques have been proposed and already included into the general HEVC framework to enable parallelism for multi-core architectures.  These include tiles and wavefront parallel processing with the current draft text specifying that these two tools can only be used exclusively and are not required to be used.  This contribution discusses the specification of these tools in the first version of the HEVC standard and recommends that (1) tiles and wavefronts remain exclusive tools and (2) the use of tiles and/or wavefronts continue to be optional tools and not mandatory even at high resolutions.

# Introduction

At the 9th JCT-VC meeting, it was decided to include wavefront parallel processing (WPP) into the Main Profile along with tiles which was already adopted into the Main Profile.  The current draft standard [1] specifies that these two tools can only be used exclusively as a result of the discussion with JCTVC-H0463 [2].  This decision was based on the significant amount of complexity with supporting each of these concepts by themselves and the combination of these two techniques presents a very difficult problem for implementers to guarantee support for all the different possible permutations.  It is also unclear whether significant additional functionality beyond what is supportable with each of the tools individually is provided by combining them.

This contribution would like to reiterate the concerns expressed in JCTVC-H0463 as JCTVC-I0118 [3] introduced the concept of subpictures which suggested that large pictures (such as 4Kx2K) could be divided into subpictures (which is conceptually the same as tiles) and the use of WPP or tiles inside each of these subpictures was implied to be easily supported since each subpicture (such as a 1080p subpicture that is a part of a 4Kx2K picture) would be decoded by a single core decoder.  This does not change the difficulties a single core 4Kx2K decoder would have handling various combinations of tiles and WPP.

Another proposal of JCTVC-I0118 was to mandate tiles for high levels to enable parallel decoding on multi-core platforms.  While there certainly are challenges with supporting increasing picture resolutions, the authors of this contribution disagree that the capabilities of single core decoders will be exceeded at higher levels and believe that single core decoders can be designed in a cost-effective fashion to support higher resolutions like 4K and 8K video without mandating a parallel bitstream structure.  There are also concerns on the negative impact on coding efficiency and potential visual artifacts with mandating a parallel bitstream structure.

# Recommendation

This contribution recommends that the following two aspects of tiles and wavefronts remain as currently specified in the HEVC draft standard text: (1) tiles and wavefronts remain exclusive tools that cannot be used together and (2) the use of tiles and/or wavefronts is optional (and not mandatory) even at high resolutions.

# References

[1]	B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003_d4, June 2012.

[2]	W.Wan, T. Hellman, “Comments on specifying tile support in HEVC Profiles and Levels”, JCT-VC Document JCTVC-H0463, San Jose, CA, Feb. 2012.

[3]	M. Zhou, “AHG4: Enable parallel decoding with tiles”, JCT-VC Document JCTVC-I0118, Geneva, CH, May 2012.

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).