| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0339r1 |
| --- | --- |

| Title: | AHG11: Methods to Encode Lossless Coding Indication Flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wen Gao Minqiang Jiang Haoping Yu Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95050 | Tel: Email: | +1 408 330 4813 wen.gao@huawei.com minqiang.jiang@huawei.com haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

In this contribution, two methods to encode cu_transquant_bypass_flag are proposed.

# Introduction

In [1], two different flags, i.e., a flag in PPS, transquant_bypass_enable_flag and a flag at each CU, cu_transquant_bypass_flag, in all slices that refer to the PPS are used to enable lossless coding operation through bypassing of transform, quantization, and in-loop filter and sign data hiding in residual coding. When transquant_bypass_enable_flag=1, cu_transquant_bypass_flag will exist at every coding unit (CU) and lossless operation only applies to a CU when cu_transquant_bypass_flag=1. The flag, cu_transquant_bypass_flag, is encoded using one context model that is initialized by setting most probable symbol (MPS) to 1 and the probability of MPS to 0.5. Here we propose two methods to encoder this flag cu_transquant_bypass_flag for better efficiency.

# Method 1: Prediction based Encoding of cu_transquant_bypass_flag

In [1], cu_transquant_bypass_flag is CABAC coded using one context model with initial value equal to 154, which is equivalent to setting the most probable symbol (MPS) equal to 1 and the probability of the MPS equal to 0.5.

We also observe that lossless coding is often applied to a whole frame or in multiple regions of a frame. Thus the flag, cu_transquant_bypass_flag, usually has the same value as the previous flag in the encoding order. A prediction based method can be used to convert all flags to zero, except those flags at transition between lossless and regular lossy coded regions.

Denotes the current cu_transquant_bypass_flag as x[n] and the flag encoded immediately before the current flag as x[n-1]. A bin value is computed as following:

y[n] = x[n] XOR X[n-1],

where XOR represents bit exclusive-or operation and X[-1]=1. In a decoder, the current cu_transquant_bypass_flag is obtained by the following operation:

x[n] = y[n] XOR x[n-1].

The bin value, y[n], is called pred_cu_transquant_bypass_flag, which is CABAC coded in an encoder using one context model and is initialized at the beginning of a slice as following:

MPS=0, Probability (MPS) = a value close to 1.

Thus the initial value of this context can be set to 128, which is equivalent to set MPS=0 and Probability(MPS)=0.98. The change to HEVC CD [1] is shown in Appendix A

# Method 2: Encoding of cu_transquant_bypass_flag using three context models

To improve the coding efficiency of cu_transquant_bypass_flag, another method proposed in this contribution is to change one context model to three context models and initialize their states properly at the beginning of a slice.

An encoder chooses one context model among the three context models as following:

(1) If upper neighboring CU is available

Upper_flag = cu_transquant_bypass_flag used in upper neighboring CU

else

Upper_flag = 0

(2) If left neighboring CU is available

Left_flag = cu_transquant_bypass_flag used in left neighboring CU

else

Left_flag = 0

(3) The index of the context model to be used = Upper_flag + Left_flag

Since the value of upper_flag and left_flag is either 0 or 1, the index of context model to be used takes the value of 0,1 and 2, corresponding to indexes of the three context models defined above. The spatial relationship between the current CU and its upper and left CUs is shown in Figure 1.

Current CU

Upper neighboring CU

Left neighboring CU

Figure 1 - Current CU and its upper and left neighboring CUs

Furthermore, the context models are initialized as following:

- Context model[0]: MPS=0, Probability(MPS) = a value close to 1

- Context model[1]: MPS=1, Probability(MPS) = 0.5

- Context model[2]: MPS=1, Probability(MPS) = a value close to 1

Thus, in the HM software, the three context models are initialized as following:

- Context model[0]: initial value=128, which is equivalent to MPS=0, Probability(MPS) = 0.98

- Context model[1]: initial value = 154, which is equivalent to MPS=1, Probability(MPS) = 0.5

- Context model[2]: initial value = 255, which is equivalent to MPS=1, Probability(MPS) = 0.98 when quantization parameter (QP) ≥ 12

The change to HEVC CD [1] is shown in Appendix B.

# Test Results

Based on the common test condition established in AHG11, we have run simulation for both Scenario 1 and 2, specified as follows:

Common test conditions: AI-main, RA-main, LB-main

Test Scenarios 1:  Frame level Lossless coding

All frames are lossless coded

Sequences: Class A-F

QP setting: QP = 0

Configuration setting related lossless coding:

#============ Lossless ================

LosslessCuEnabled                   : 0

TransquantBypassEnableFlag          : 1                # Value of PPS flag.

CUTransquantBypassFlagValue         : 1                # Constant lossless-value signaling per CU,

Test Scenario 2: Region based lossless coding

In each frame, three regions are lossless coded. The top-left and bottom right coordinates of the three regions are:

Region 1: (272, 32), (1119, 143)

Region 2: (208, 352), (495, 623)

Region 3: (1152, 192+16*floor(POC/100)),  (1231, 223+16*floor(POC/100))

Test Sequences: SlideEditing and SlideShow

QP setting: QP=22, 27, 32, 37

Configuration setting related lossless coding:

#============ Lossless ================

LosslessCuEnabled                   : 0

TransquantBypassEnableFlag          : 1                # Value of PPS flag.

CUTransquantBypassFlagValue         : 0                # Constant lossless-value signaling per CU,

For Test Scenario 1, HM7.0-dev_r2461 is used as the anchor. However, HM7.0-dev_r2461 can only achieve frame-level lossless coding. Hence, we have modified it to realize region based lossless coding. The modified HM software is used as anchor for Test Scenario 2.

We have implemented the above two methods to encode cu_transquant_bypass_flag using HM7.0-dev_r2461 as the basis and run simulations for Test Scenario 1 and 2. Following are the results:

Test Scenario 1:

- Prediction based encoding with one context model:

  - Compared to anchor codec, the average bit rate reduction is 0.00% for AI-main, 0.00% for RA-main, and 0.00% for LB-main.

|  | All Intra Main | Random Access Main | Low delay B Main |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class A | 0.00% | 0.00% |  |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% |  | 0.00% |
| Overall (w/o F) | 0.00% | 0.00% | 0.00% |
| Overall (w/ F) | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.01% | 0.00% |

- Encoding with three context model:

  - Compared to anchor codec, the average bit rate reduction is 0.00% for AI-main, 0,00% for RA-main, and 0.00% for LB-main.

|  | All Intra Main | Random Access Main | Low delay B Main |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class A | 0.00% | 0.00% |  |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% |  | 0.00% |
| Overall (w/o F) | 0.00% | 0.00% | 0.00% |
| Overall (w/ F) | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% |

Test Scenario 2:

- Prediction based encoding with one context model:

  - Compared to anchor codec, the average bit rate reduction is 0.1% for AI-main, 0.1% for RA-main, and -0.2% for LB-main.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class F | -0.1% | 0.0% | 0.0% |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class F | 0.2% | 0.1% | 0.2% |

- Encoding with three context model:

  - Compared to anchor codec, the average bit rate reduction is 0.2% for AI-main, 1.0% for LB-main, and 0.6% for RA-main.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class F | -0.2% | -0.2% | -0.2% |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class F | -0.6% | -0.4% | -0.5% |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class F | -1.0% | -1.0% | -0.9% |

# Conclusions

Suggest the JCT-VC committee adopt one of the proposed methods to encode cu_transquant_bypass_flag in HEVC DIS and HM software.

# References

[1]  B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification draft 7” JCT-VC Document, JCTVC-I1003, Geneva, CH,  May. 2012.

# Patent rights declaration(s)

Huawei Technologies (USA) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A

The modified text is highlighted in yellow color.

### 7.3.6 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( transquant_bypass_enable_flag ) { |  |
| pred_cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y2 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y3 , log2CbSize ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x2, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x3, y0 , log2CbSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| prediction_unit( x1, y1 , log2CbSize ) |  |
| } |  |
| if( !pcm_flag ) { |  |
| if( PredMode != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_data_flag | ae(v) |
| if( !no_residual_data_flag ) { |  |
| MaxTrafoDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

### 7.4.3   Slice header semantics

......

Set the variable prev_cu_transquant_bypass_flag to 1.

### 7.4.4   Slice data semantics

......

### 7.4.6   Coding unit semantics

pred_cu_transquant_bypass_flag is used to derive the variable cu_transquant_bypass_flag as following:

cu_transquant_bypass_flag = pred_cu_transquant_bypass_flag ^ prev_cu_transquant_bypass_flag

The variable cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in subclause Error: Reference source not found and the in-loop filter process as specified in subclause Error: Reference source not found are bypassed.

#### Initialization process for context variables

......

Table 9-1 – Values of variable initValue for pred_cu_transquant_bypass_flag ctxIdx

| Initialization variable | cu_transquant_bypass_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 128 | 128 | 128 |

# Appendix B

The modified text is highlighted in yellow color.

#### 9.3.1.1  Initialization process for context variables

......

Table 9-11 – Values of variable initValue for cu_transquant_bypass_flag ctxIdx

| Initialization variable | cu_transquant_bypass_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 128 | 154 | 255 | 128 | 154 | 255 | 128 | 154 | 255 |