| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0340r2 |
| --- | --- |

| Title: | AHG11: New signalling mechanism for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wen Gao Minqiang Jiang Haoping Yu Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95050 | Tel: Email: | +1 408 330 4813 wen.gao@huawei.com Minqiang.Jiang@huawei.com haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

In this contribution, a new signaling mechanism for lossless coding is proposed

# Introduction

In the last JCTVC meeting, two flags [1], i.e., a PPS level flag transquant_bypass_enable_flag and a flag at each CU in a slice, cu_transquant_bypass_flag, were adopted to enable lossless coding operation through bypassing of transform, quantization, an in-loop filter and sign data hiding at residual coding. When transquant_bypass_enable_flag=1, cu_transquant_bypass_flag will exist at every CU and lossless operation only applies to a CU when cu_transquant_bypass_flag=1, otherwise the normal operation with transform/quantization/in-loop-filter will apply to all CUs with cu_transquant_bypass_flag=0. We observe that this signaling mechanism will cause significant overhead if only a small area in a slice is coded in lossless mode. Hence we propose an alternative signaling mechanism that can mitigate this overhead problem.

# Concept of Transquant Bypass Group

Similar to a CU quantization group, we propose to introduce the concept of a CU transquant bypass group (TQBypassGroup in short), specified as follows:

- If a CU with split_coding_unit_flag equal to 0 and the CU size is greater than or equal to a pre-defined size, MinTQBypassGroupSize, this TQBypassGroup includes this CU only.

- Otherwise, if a CU with the split_coding_unit_flag equal to 1 and the CU size is equal to MinTQBypassGroupSize, this TQBypassGroup includes all CUs split from this CU.

Note that MinTQBypassGroupSize is greater or equal to minimum CU size, meanwhile it has to be no greater than the maximum CU size, i.e., the LCU size.  An encoder can choose the MinTQBypasGroupSize based on the granularity of area for lossless coding. Only one cu_transquant_bypass_flag is carried in each TQByassGroup.

We also propose to introduce a new syntax element “max_transquant_bypass_depth” to specify the size of the TQBypassGroup. This syntax element is also used to enable slice or frame wise lossless compression as described in the semantics of this syntax element below:

max_transquant_bypass_depth indicates the control granularity for cu_transquant_bypass_flag for coding unit within a picture. The value of max_transquant_bypass_depth shall not be greater than slice_granularity + 1.

The variable Log2MinTQByoassGroupSize specifying the minimum luma coding block size of coding units, i.e., the minimum size of TQBypassGroup, that convey cu_transquant_bypass_flag, is derived as follows.

# If max_transquant_bypass_depth=0

# All CUs within the current slice will be coded in the lossless mode without transmitting cu_transquant_bypass_flag.

# if max_transquant_bypass_depth > 0

# Log2MinTQBypassGroupSize = Log2CtbSize − (max_transquant_bypass_depth − 1)

The modified syntax tables are shown in the Appendix with the changes highlighted in yellow color.

# Test Results

Based on the common test condition established in AHG11, we have run simulations for both Scenario 1 and 2, specified as follows:

Common test conditions:  AI-main, RA-main, LB-main

Test Scenarios 1: Frame level Lossless coding

All frames are lossless coded

Sequences: Class A-F

QP setting: QP = 0

Configuration setting related lossless coding:

#============ Lossless ================

LosslessCuEnabled                   : 0

TransquantBypassEnableFlag          : 1                # Value of PPS flag.

CUTransquantBypassFlagValue         : 1                # Constant lossless-value signaling per CU,

Test Scenario 2: Region based lossless coding

In each frame, three regions are lossless coded. The top-left and bottom right coordinates of the three regions are:

Region 1: (272, 32) , (1119, 143)

Region 2: (208, 352), (495, 623)

Region 3: (1152,  192+16*floor(POC/100)),  (1231, 223+16*floor(POC/100))

Test Sequences: SlideEditing and SlideShow

QP setting: QP=22, 27, 32, 37

Configuration setting related lossless coding:

#============ Lossless ================

LosslessCuEnabled                   : 0

TransquantBypassEnableFlag          : 1                # Value of PPS flag.

CUTransquantBypassFlagValue         : 0                # Constant lossless-value signaling per CU,

For Test Scenario 1, HM7.0-dev_r2461 is used as the anchor. However, HM7.0-dev_r2461 can only achieve frame-level lossless coding. Hence, we have modified it to realize the region based lossless coding, and this modified HM software is used as anchor for Test Scenario 2.

We have implemented this concept of TQBypassGroup using HM7.0-dev_r2461 as the basis and run simulation for Test Scenario 1 and 2. Following are the results:

Test Scenario 1:

When max_transquant_bypass_depth=3, the proposed solution has almost the same performance as the anchor HM software

|  | All Intra Main | Random Access Main | Low delay B Main |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class A | 0.00% | 0.00% |  |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% |  | 0.00% |
| Overall (w/o F) | 0.00% | 0.00% | 0.00% |
| Overall (w/ F) | 0.00% | 0.00% | -0.01% |
| Class F | -0.01% | 0.00% | -0.02% |

When max_transquant_bypass_depth=0, no CU level cu_transquant_bypass_flag is ever transmitted. Thus the proposed solution slightly improves the coding efficiency, shown in the following table:

|  | All Intra Main | Random Access Main | Low delay B Main |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class A | 0.00% | 0.00% |  |
| Class B | -0.01% | 0.00% | 0.00% |
| Class C | -0.01% | -0.01% | -0.01% |
| Class D | -0.01% | 0.00% | -0.01% |
| Class E | -0.01% |  | -0.01% |
| Overall (w/o F) | -0.01% | -0.01% | -0.01% |
| Overall (w/ F) | -0.01% | -0.01% | -0.01% |
| Class F | -0.01% | -0.04% | -0.02% |

Test Scenario 2:

When max_transquant_bypass_depth=3, compared to the anchor software, the proposed solution reduces the luma BD rate by 0.1% for AI-main, 0.2% for RA-main, and 0.2% for LB-main.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class F | -0.2% | -0.2% | -0.2% |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class F | -0.2% | -0.4% | -0.2% |

# Conclusions

Suggest the JCT-VC committee adopts the proposed solutions in HEVC DIS and HM software.

# References

[1]  B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification draft 7” JCT-VC Document, JCTVC-I1003, Geneva, CH,  May. 2012.

# Patent rights declaration(s)

Huawei Technologies (USA) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix

The text is in the following pages with modified portion highlighted in yellow color.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| sign_data_hiding_flag | u(1) |
| if( sign_data_hiding _flag ) |  |
| sign_hiding_threshold | u(4) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| [Ed. (BB): not present in HM software ] |  |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| diff_cu_qp_delta_depth | ue(v) |
| cb_qp_offset | se(v) |
| cr_qp_offset | se(v) |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| output_flag_present_flag | u(1) |
| transquant_bypass_enable_flag | u(1) |
| dependent_slice_enabled_flag | u(1) |
| If (transquant_bypass_enable_flag) |  |
| max_transquant_bypass_depth | ue(v) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| cabac_independent_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| slice_header_extension_present_flag | u(1) |
| slice_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### Coding tree syntax

| coding_tree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] >= SliceCbAddrZS && log2CbSize > Log2MinCbSize && NumPCMBlock = = 0 ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| if( ( diff_cu_qp_delta_depth > 0 ) && log2CbSize >= Log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| if( (max_transquant_bypass_ depth > 0 ) && log2CbSize >= log2MinTQBypassGroupSize ) |  |
| IsCuTQBypssCoded =0 |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| if( MinCbAddrZS[ x1 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] > SliceCbAddrZS ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( MinCbAddrZS[ x0 >> Log2MinCbSize ][ y1 >> Log2MinCbSize ] > SliceCbAddrZS && moreDataFlag && x1 < pic_width_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( MinCbAddrZS[ x1 >> Log2MinCbSize ][ y1 >> Log2MinCbSize ] > SliceCbAddrZS && moreDataFlag && y1 < pic_height_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| if( moreDataFlag && x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| } else { |  |
| if( NumPCMBlock = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize ) |  |
| else |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| if( !( ( x0 + 1 << log2CbSize ) % ( 1 << SliceGranularity ) ) && !( ( y0 + 1 << log2CbSize ) % ( 1 << SliceGranularity ) ) && NumPCMBlock = = 0 ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

### Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( transquant_bypass_enable_flag && !IsCuTQBypassCoded ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| IsCuTQBypassCoded = 1 |  |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y2 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y3 , log2CbSize ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x2, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x3, y0 , log2CbSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| prediction_unit( x1, y1 , log2CbSize ) |  |
| } |  |
| if( !pcm_flag ) { |  |
| if( PredMode != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_data_flag | ae(v) |
| if( !no_residual_data_flag ) { |  |
| MaxTrafoDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### Picture parameter set RBSP semantics

#### ......

transquant_bypass_enable_flag specifies whether a cu_transquant_bypass_flag syntax element is coded or not. If transquant_bypass_enable_flag is equal to 1, cu_transquant_bypass_flag is coded, otherwise it is not coded.

max_transquant_bypass_depth indicates the control granularity for cu_transquant_bypass_flag for coding unit within a picture. The value of max_transquant_bypass_depth shall not be greater than slice_granularity + 1.

The variable Log2MinTQByoassGroupSize specifying the minimum luma coding block size of coding units, i.e., the minimum size of TQBypassGroup, that convey cu_transquant_bypass_flag, is derived as follows.

# If max_transquant_bypass_depth=0

# All CUs within the current slice will be coded in the lossless mode without transmitting cu_transquant_bypass_flag.

# if max_transquant_bypass_depth > 0

# Log2MinTQBypassGroupSize = Log2CtbSize − (max_transquant_bypass_depth − 1)

......

### Coding unit semantics

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in subclause Error: Reference source not found and the in-loop filter process as specified in subclause Error: Reference source not found are bypassed for a coding unit transquant bypass group (TQBypassGroup) where the TQBypassGroup is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CbSize is greater than or equal to Log2MinTQBypassGroupSize, the TQBypassGroup includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CbSize is equal to Log2MinTQBypassGroupSize, the TQBypassGroup includes all coding units split from this coding unit.

When cu_transquant_bypass_flag is not present, it is inferred to be equal to 0.