| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0341 |
| --- | --- |

| Title: | Cross-check of JCTVC-J0302 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Yoshitomo.Takahashi@jp.sony.com Ohji.Nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results on the solution 2 described in JCTVC-J0302 proposed by Samsung.

In the solution 2 of JCTVC-J0302, the motion vectors are not inserted into the candidate list when the POC difference between reference picture of current PU and reference picture of candidate PU are larger than pre-determined threshold. The same method was proposed in JCTVC-G551 [1]

The verification task has been done successfully and the results match those provided by proponent. It is noted that the encoding time of our cross-checking results may be not reliable since the simulation machines with various specifications were used. The decoding time is more reliable since the machines with same specification were used.

The experimental results in HM software version 7.1 are reported. The results show that BD-bitrate are no gain or loss for RA-MP, LDB-MP, RA-HE10 and LDB-HE10 compared with anchor on average from Class A to Class E. The results also shows that BD-bitrate loss are 0.0%, -0.1%, 0.0% and -0.1% for RA-MP, LDB-MP, RA-HE10 and LDB-HE10 compared with anchor on Class F. It is reported that these schemes does not affect the decoding time at least.

# Cross-checking

The software based on HM version 7.1 and the experimental results were provided by Samsung. The software was compiled fine and the cross-checking was performed on 64bit linux environment. The simulation has been conducted under common test conditions [2] for Main profile (MP) conditions and high efficiency (HE10) conditions in the case of random access (RA) and low delay B (LB) coding structures.

The results of anchor were generated using the released HM7.1. The results of proposal were generated with the macro “#define LT_MV_SCALING_RESTRICTION 1” provided by the proponent.

# Results

The experimental results are summarized in the following table. The detailed results are shown in the attached excel-file.

Table 1:BD-bitrate[%] and relative encoding/decoding time[%] of the proposed method

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class D | -0.1% | -0.2% | 0.3% | 0.0% | -0.1% | -0.3% |
| Class E | -0.1% | -0.6% | -0.3% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.1% | 0.1% | -0.1% | 0.0% | 1.0% |
| Enc Time[%] | 95% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Conclusion

In this contribution, the cross-checking results on the solution 2 of JCTVC-J0302 are reported. It is confirmed that the results match with the ones provided by Samsung.

# References

- I. -K. Kim, Y. Park, N. Shlyakhov, and J. H. Park, "Restriction on motion vector scaling for Merge and AMVP," Document of Joint Collaborative Team on Video Coding, JCTVC-G551, Nov. 2011.

- F. Bossen, “Common HM test conditions and software reference configurations”, Document no JCTVC-I1100. May 2012.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).