| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0342 |
| --- | --- |

| Title: | Chroma QP range extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Sandeep Kanumuri Ji-Zheng Xu Yongjun Wu One Microsoft Way Redmond WA 98052-6399 USA | Tel: Email: | +1-425-703-5308 garysull@microsoft.com skanumu@microsoft.com jzxu@microsoft.com yongjunw@microsoft.com. |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a way to extend the range of QP values used for chroma from [0, 39] to [0, 51]. The table used in the derivation of chroma QP values from luma QP is proposed to be modified to enable the extended range. In addition, the use of slice-level chroma QP offsets is discussed. The proposed scheme is asserted to have no significant impact on BD bit rate for common test conditions, as the modification mostly applies outside of the range of values that is tested in the common conditions. The results reportedly show a very small improvement for luma and a very small loss for chroma. Results are also reported for higher QP values than are used in the common conditions.

This contribution also proposes that offset range limits similar to those specified in AVC (ranging from −12 to +12) should be placed on the chroma QP offset values.

# Introduction

In the current HEVC draft design [1], the QP used for chroma is limited to the range [0, 39] whereas the QP used for luma has the larger range [0, 51] (for a bit-depth of 8). The range is increased appropriately for higher bit-depths (for both luma and chroma). With this design, the QP value used for chroma saturates at a much smaller value than for luma, and this restriction causes problems for rate control in low bit-rate applications and may not be well-suited for some color formats. These problems have been discussed extensively in proposals to previous meetings [2][3]. However, the solution that we propose is somewhat different.

According to the current HEVC draft design, the QPs used for chroma components (QPCb and QPCr) are derived from the QP used for luma component (QPY) as follows:

The values of QPCb and QPCr are equal to the value of QPC as specified in Table 8-1 based on the index qPI equal to qPICb and qPICr derived as:

qPICb = Clip3( −QpBdOffsetC, 51, QPY + cb_qp_offset)	(8-1)

qPICr = Clip3( −QpBdOffsetC, 51, QPY + cr_qp_offset)	(8-2)

Table 8-1 – Specification of QPC as a function of qPI

| qPI | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPC | = qPI | 29 | 30 | 31 | 32 | 32 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 37 | 38 | 38 | 38 | 39 | 39 | 39 | 39 |

It should be noted that a further adjustment to these QPs occurs based on bit-depth; this is not proposed to be changed and hence is not discussed here in order to keep the presentation simple. For the purpose of clarity, the equations representing this adjustment have been replicated below from the current HEVC draft design text [1].

QP’Y = QPY + QpBdOffsetY	(8-3)

QP’Cb = QPCb + QpBdOffsetC	(8-4)

QP’Cr = QPCr + QpBdOffsetC	(8-5)

To extend the range of chroma QP to match that of luma, we propose to modify the derivation of QPCb and QPCr from QPY – i.e. to modify Equations 8-6 and 8-7 and Table 8-2. The modifications to the current HEVC design are highlighted in green.

# Proposed modification

## Chroma QP range extension

We propose a small modification of the existing design to extend the range of values supported in the mapping of luma QP to chroma QP in a straightforward fashion. In the same basic manner as in the current specification, the variable QPC specifies the value of QPCb and QPCr given by Table 8-11 based on the index qPI equal to qPICb and qPICr derived as follows:

qPICb = Clip3( −QpBdOffsetC, 57, QPY + cb_qp_offset)	(8-255)

qPICr = Clip3( −QpBdOffsetC, 57, QPY + cr_qp_offset)	(8-256)

- Table 8-11 – Specification of QPC as a function of qPI

| qPI | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | >43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPC | = qPI | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPI – 6 |

Note that although the relationship between QPC and qPI is specified above as a table, the relationship is actually a piece-wise linear function that can alternatively be implemented using simple equations as follows:

if ( qPI < 30 )

QPC = qPI

else if ( qPI >= 30 && qPI <= 34 )

QPC = qPI − 1

else if ( qPI > 34 && qPI < 44 )

QPC = 33 + (qPI – 34) >> 1

else

QPC = qPI − 6

## Slice-level chroma QP offsets

The use of slice-level chroma QP offsets as proposed in [2] can provide the encoder with a greater ability to precisely control the chroma QP. However, this should be done in a way that avoids adding slice-level syntax overhead when the encoder does not choose to use this feature. We therefore propose that the use of slice-level chroma QP offsets be enabled/disabled using a flag signalled in the PPS syntax where the picture-level offsets, cb_qp_offset and cr_qp_offset, are signalled. When used, the slice-level offsets are proposed to be applied in the same manner as the PPS-level offsets – i.e. added into the argument of the clipping function in Equations 8-255 and 8-256 as follows:

qPICb = Clip3( −QpBdOffsetC, 57, QPY + cb_qp_offset + slice_qp_delta_cb)	(8-255)

qPICr = Clip3( −QpBdOffsetC, 57, QPY + cr_qp_offset + slice_qp_delta_cr)	(8-256)

## Advantages

The proposed scheme has the following advantages:

- Table 8-3 is extended to support the use of higher chroma QP values. The tables are extended such that the maximum possible QP for chroma is now 51 instead of 39.

- The values used in Table 8-4 establish a straightforward relationship that can be implemented with simple formulas/logic.

- The step size difference between luma and chroma is prevented from becoming too extreme (especially for the values near and beyond the range that is currently supported). When default values are used for the chroma index offsets (i.e. when the offsets are set to 0), the ratio between the luma and chroma step sizes can be as large as 4 in the current HEVC design. The proposed scheme reduces this maximum default ratio to 2, thus preventing excessive bit usage for chroma when the quantization needs to be really coarse.

- A slope of 1 is enabled at high QP operation. This is desirable so that we do not continue to alter the balance between luma and chroma when changing QP values (typically done during rate control) – i.e. the ratio between luma and chroma quantization step sizes remains the same at high QP operation.

- A fixed offset of 6 can be used to achieve equal step sizes for luma and chroma at high QP operation if desired. Some applications may desire to code all three planes using the same step size. In the current design, this means that the chroma QP offset has to be adjusted depending on the luma QP value, which is not a capability supported in the syntax below the PPS level. The proposed scheme can instead simply use a fixed offset of 6 to achieve this goal at high QP values.

- The offset needed to achieve a desired relative relationship (between the quantization step size for luma and chroma) is much smaller. For example, with the current HEVC design, if the encoder wanted to use a QP of 39 for both luma and chroma, we need an offset of 12. This becomes even larger if we were to simply extend the table at the same slope seen at the end of its current range. The proposed scheme can achieve the same effect using a much smaller offset of 6.

The following graph illustrates the effect of Table 8-5, for HM7.0 and the proposal, in terms of the QP mapping from input QP (qPI) to output QP (QPC). As we can see, the proposed scheme extends the chroma QP range to 51 with a slope of 1 at high QPs.

The following graph illustrates the effect of Table 8-6, for HM7.0 and the proposal, in terms of the ratio of quantization step-sizes represented by input QP (qPI) and output QP (QPC) as we vary the input QP (qPI). As we can see, the ratio increases exponentially for HM7.0 while the ratio saturates at 2 for the proposed scheme.

# Experimental Results

The proposed modifications do not have any significant impact on the results under common conditions because most of the changes are for QP values outside the range used in the common conditions. However, we present the results using common test conditions for the sake of completeness. As we can see in the table below, there is a small gain in luma with a small loss for chroma and they basically offset each other. Under common conditions, there is no impact on QPs used for All-Intra test configurations and hence the results are not presented (as they are identical to HM7.0).

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.02% | 0.33% | 0.34% | -0.04% | 0.25% | 0.24% |
| Class B | -0.04% | 0.33% | 0.26% | -0.04% | 0.39% | 0.31% |
| Class C | -0.07% | 0.38% | 0.41% | -0.05% | 0.39% | 0.47% |
| Class D | -0.06% | 0.37% | 0.44% | -0.07% | 0.34% | 0.39% |
| Class E |  |  |  |  |  |  |
| Overall | -0.05% | 0.35% | 0.36% | -0.05% | 0.35% | 0.35% |
|  | -0.04% | 0.27% | 0.26% | -0.05% | 0.26% | 0.25% |
| Class F | -0.05% | 0.19% | 0.23% | -0.04% | 0.24% | 0.27% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.03% | 0.23% | -0.04% | -0.03% | 0.27% | 0.16% |
| Class C | -0.04% | 0.38% | 0.42% | -0.08% | 0.26% | 0.52% |
| Class D | -0.02% | 0.34% | 0.13% | -0.05% | -0.02% | 0.21% |
| Class E | 0.05% | 0.16% | 0.16% | 0.13% | -0.24% | -0.16% |
| Overall | -0.01% | 0.28% | 0.16% | -0.02% | 0.10% | 0.20% |
|  | -0.01% | 0.19% | 0.10% | -0.01% | 0.08% | 0.11% |
| Class F | -0.06% | 0.39% | 0.27% | 0.00% | 0.23% | 1.10% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |

BD bit-rate comparison with HM7.0

We also did experiments using high QP values (40, 43, 46 and 49) for 3 ‘Main’ test configurations and 1 ‘HE10’ configuration. As we can see in the tables below, there is a gain in luma with a loss in chroma and the gain in luma is better than just offsetting the loss in chroma.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.7% | 19.1% | 18.3% | #VALUE! | #VALUE! | #VALUE! |
| Class B | -3.5% | 16.5% | 17.3% | #VALUE! | #VALUE! | #VALUE! |
| Class C | -4.8% | 15.1% | 15.1% | #VALUE! | #VALUE! | #VALUE! |
| Class D | -4.6% | 13.6% | 14.5% | #VALUE! | #VALUE! | #VALUE! |
| Class E | -2.6% | 13.4% | 15.3% | #VALUE! | #VALUE! | #VALUE! |
| Overall | -3.7% | 15.7% | 16.2% | #VALUE! | #VALUE! | #VALUE! |
|  | -3.7% | 12.2% | 12.2% | #VALUE! | #VALUE! | #VALUE! |
| Class F | -3.3% | 12.9% | 13.8% | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 101% |  |  | #NUM! |  |  |
| Dec Time[%] | 102% |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -3.2% | 27.2% | 24.5% | -2.9% | 25.5% | 23.2% |
| Class B | -4.3% | 24.4% | 25.2% | -4.2% | 22.3% | 24.8% |
| Class C | -6.3% | 23.2% | 24.2% | -6.0% | 22.6% | 24.0% |
| Class D | -5.0% | 22.2% | 22.4% | -5.0% | 21.8% | 23.4% |
| Class E |  |  |  |  |  |  |
| Overall | -4.7% | 24.3% | 24.1% | -4.5% | 23.0% | 23.9% |
|  | -4.7% | 23.3% | 23.1% | -4.5% | 22.1% | 22.7% |
| Class F | -4.5% | 17.9% | 17.8% | -3.9% | 15.6% | 15.3% |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 105% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.7% | 34.1% | 37.2% | #VALUE! | #VALUE! | #VALUE! |
| Class C | -6.3% | 29.1% | 29.7% | #VALUE! | #VALUE! | #VALUE! |
| Class D | -4.9% | 27.0% | 26.4% | #VALUE! | #VALUE! | #VALUE! |
| Class E | -2.2% | 17.9% | 22.8% | #VALUE! | #VALUE! | #VALUE! |
| Overall | -4.4% | 28.0% | 29.9% | #VALUE! | #VALUE! | #VALUE! |
|  | -4.4% | 27.8% | 30.3% | #VALUE! | #VALUE! | #VALUE! |
| Class F | -4.9% | 23.5% | 22.9% | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 100% |  |  | #NUM! |  |  |
| Dec Time[%] | 99% |  |  | #NUM! |  |  |

BD bit-rate comparison with HM7.0 at high QPs (40, 43, 46 and 49)

# Limits for chroma QP offsets

In AVC, the chroma QP offsets are limited to the range −12 to +12, inclusive. We think similar limits are useful for HEVC to simplify decoding, as it does not seem necessary to support huge quality differences between luma and chroma fidelity. In particular, we propose that the same range of −12 to +12 would be a good choice for HEVC as well (when used with the other proposed modifications). For example, when combined with the proposed scheme, at high QPs, since an offset of 6 represents the case where luma quantization step-size is equal to the chroma quantization step-size, an offset of 12 represents the counter-point to an offset of 0 (at both these offset values, the larger quantization step-size is exactly 2x the smaller quantization step-size).

Furthermore, we note that the HM-7.0 software already limits cb_qp_offset and cr_qp_offset to the range −12 to +12, inclusive.

If slice-level QP offsets are not supported, we propose that the range limit (−12 to +12) be imposed on cb_qp_offset and cr_qp_offset.

If slice-level QP offsets are supported, we propose that the range limit (−12 to +12) be imposed on all of the following:

- cb_qp_offset and cr_qp_offset

- slice_qp_delta_cb and slice_qp_delta_cr

- (cb_qp_offset + slice_qp_delta_cb) and (cr_qp_offset + slice_qp_delta_cr).

# Conclusion

We recommend extending the QP range for chroma in the HEVC draft design as proposed. We further recommend that slice-level QP offsets be supported, as proposed, since this would enable the additional flexibility of signalling offsets at the slice-level, which would give encoders greater control over the fidelity and bit rate for each colour component.

We also recommend that limits be placed on the possible values for chroma QP offsets. We further recommend that the allowed range of values be essentially same as that in AVC – i.e. to define the allowed range to be from −12 to +12.

# References

- B. Bross et. al, “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003, 9th meeting, Geneva, April 2012.

- J. Xu et. al, “Chroma QP extension and signalling enhancement”, JCTVC-I0265, 9th meeting, Geneva, April 2012.

- J. Xu et. al, “Chroma QP extension”, JCTVC-H0400, 8th meeting, San Jose, February 2012.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

The changes to syntax and semantics for enabling slice-level chroma QP offsets are highlighted in green below:

## Syntax

#### 7.3.2.3		Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| cb_qp_offset | se(v) |
| cr_qp_offset | se(v) |
| slicelevel_chroma_qp_flag | u(1) |
| … |  |
| } |  |

### 7.3.3	Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_qp_delta | se(v) |
| if (slicelevel_chroma_qp_flag) { |  |
| slice_qp_delta_cb | se(v) |
| slice_qp_delta_cr | se(v) |
| } |  |
| … |  |
| } |  |

## Semantics

### 7.4.2.3		Picture parameter set RBSP semantics

diff_cu_qp_delta_depth specifies the difference between the control granularity for QPY values for coding units within a picture and slice_granularity plus 1. The value of diff_cu_qp_delta_depth shall be in the range of 0 to log2_diff_max_min_coding_block_size − slice_granularity + 1, inclusive.

The variable Log2MinCUDQPSize specifying the minimum luma coding block size of coding units that convey cu_qp_delta, is derived as follows.

Log2MinCUDQPSize = Log2CtbSize − diff_cu_qp_delta_depth − slice_granularity + 1	(7-8)

cb_qp_offset, cr_qp_offset are specifying the offset to the luma quantization parameter QP’Y used for deriving QP’Cb and QP’Cr , respectively specify a base offset used in obtaining QPCb and QPCr respectively as specified in subclause 8.6.1.

slicelevel_chroma_qp_flag equal to 1 specifies that syntax elements slice_qp_delta_cb and slice_qp_delta_cr are present in the associated slice headers. Otherwise, these syntax elements are not present in the associated slice headers.

weighted_pred_flag equal to 0 specifies that weighted prediction shall not be applied to P slices. weighted_pred_flag equal to 1 specifies that weighted prediction shall be applied to P slices.

### 7.4.3	Slice header semantics

slice_qp_delta specifies the initial value of QPY to be used for all the coding blocks in the slice until modified by the value of cu_qp_delta in the coding unit layer. The initial QPY quantization parameter for the slice is computed as

SliceQPY = 26 + pic_init_qp_minus26 + slice_qp_delta	(7-9)

The value of slice_qp_delta shall be limited such that SliceQPY is in the range of −QpBdOffsetY to +51, inclusive.

slice_qp_delta_cb, slice_qp_delta_cr specify a delta offset used in obtaining QPCb and QPCr respectively as specified in subclause 8.6.1. When not present, the value of these syntax elements is inferred to be 0.

deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the active picture parameter set shall be used for deblocking the current slice. deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the slice header shall be used for deblocking the current slice. When not present, the value of deblocking_filter_override_flag is inferred to be equal to 1.