| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0343 |
| --- | --- |

| Title: | Use of Chroma QP offsets in Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sandeep Kanumuri Gary J. Sullivan One Microsoft Way Redmond WA 98052-6399 USA | Tel: Email: | +1-425-707-3140 skanumu@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Abstract

This contribution proposes that the effect of chroma QP offsets be taken into account when determining the parameter tC for deblocking chroma components. It is asserted that the current derivation of this parameter in the current HEVC design does not properly reflect the actual QP used for chroma when non-zero QP offsets are used.

# Introduction

In the current HEVC design [1], the filter “strength” (tC parameter) used while deblocking a block edge of a chroma component is determined using a value QPC obtained as follows:

The variable QPC is determined as specified in Error: Reference source not found based on the index qPI derived as:

qPI = ( ( QPQ + QPP + 1 ) >> 1 )	(8-1)

Here QPQ and QPP represent the luma QP values for the blocks present on either side of the edge.

The general idea is to adjust the filter strength based on the QP values used to quantize the samples around the edge. While equation 8-325 makes sense when chroma QP offsets (cb_qp_offset and cr_qp_offset) are both equal to zero, it does not make sense when they are not equal to zero. For different values of offsets, the QP used for chroma components would be different, but the filter strength remains the same.

# Proposed solution

We propose to modify equation 8-325 such that the effect of chroma QP offsets is taken into account. The modified equation is as follows:

qPI = Clip3( 0, 51, ( ( QPQ + QPP + 1 ) >> 1 + cqp_offset ) )	(8-2)

Here cqp_offset represents cb_qp_offset and cr_qp_offset for components Cb/U and Cr/V respectively.

# References

- B. Bross et. al, “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003, 9th meeting, Geneva, April 2012.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).