| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0344 |
| --- | --- |

| Title: | Refinement of random access point support |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sandeep Kanumuri Gary J. Sullivan One Microsoft Way Redmond WA 98052-6399 USA | Tel: Email: | +1-425-707-3140 skanumu@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Abstract

This contribution proposes three modifications relating to RAP pictures: 1) a constraint on IDR pictures to provide a simplified form of random access, 2) a constraint that leading pictures of RAP pictures must precede non-leading pictures in decoding order, in order to simplify the scanning of a bitstream for leading pictures, and 3) to modify the NAL unit type definitions for RAP pictures to avoid duplicate functionality and convey more RAP type information in the NAL unit type.

# Introduction

In the current HEVC design [1], a RAP (Random Access Point) picture is represented by NAL unit types 4 to 8. Depending on the characteristics of the RAP picture, it will map to one of the following SAP (Stream Access Point) types defined in the 4th edition of ISO/IEC 14496-12 [3] (a total of 6 types are defined, but only the first three types are directly relevant to RAP pictures):

- Type 1 corresponds to what is known in some coding schemes as a “Closed GoP random access point” (in which all access units, in decoding order, starting from ISAP can be correctly decoded, resulting in a continuous time sequence of correctly decoded access units with no gaps) and in addition the access unit in decoding order is also the first access unit in presentation order.

- Type 2 corresponds to what is known in some coding schemes as a “Closed GoP random access point”, for which the first access unit in decoding order in the media stream starting from ISAU is not the first access unit in presentation order.

- Type 3 corresponds to what is known in some coding schemes as an “Open GoP random access point”, in which there are some access units in decoding order following ISAU that cannot be correctly decoded and have presentation times less than TSAP.

From a systems perspective, it is desirable to make this mapping as simple as possible while allowing the use of as many types as possible. With this motivation in mind, we propose the following changes to the current HEVC design.

# Current Proposal

A RAP picture can be further classified into a CRA or BLA or IDR picture, depending on the NAL unit type. In the current HEVC design, a CRA picture is represented by NAL unit types 4 and 5, a BLA picture is represented by NAL unit types 6 and 7, and an IDR picture is represented by NAL unit type 8. NAL unit types 5 and 7 can be used for CRA and BLA pictures respectively only when the CRA or BLA picture does not have any associated TFD pictures.

## Removal of IDR pictures or Constraint on IDR pictures

At the last JCT-VC meeting held in April 2012, the concept of BLA pictures was adopted into the current HEVC design from JCTVC-I0404 [2]. JCTVC-I0404 also pointed out that a BLA picture can achieve the functionality of an IDR picture (and more) and hence recommended that the concept of IDR picture be dropped from the HEVC design. This aspect was recommended for further study as experts wanted more time to consider its implications.

In the event that it is considered desirable not to remove the specification of IDR pictures, we propose that a constraint be placed on IDR pictures such that it becomes very easy to do the mapping of IDR to a SAP type.

In the current HEVC design, an IDR picture can map to SAP types 1 or 2, but in general it is only possible to determine which one of those cases applies by scanning the bitstream to see if it contains leading pictures. If the IDR picture has leading pictures (coded pictures that follow the current picture in decoding order but precede it in output order), it will be mapped to SAP type 2. If the IDR picture does not have leading pictures, it will be mapped to SAP type 1. So when a system encounters an IDR picture, the system has to scan the bitstream to check whether there are leading pictures or not in order to determine the correct SAP type to use.

We propose that IDR pictures be constrained to not have leading pictures. With this constraint, an IDR picture always maps to SAP type 1. This does not reduce functionality, because it is possible to use a BLA picture instead of an IDR picture if there is a desire to encode a RAP with leading pictures.

## NAL unit types for CRA/BLA pictures

When there are no TFD pictures, the functionality of CRA and BLA pictures is actually the same. Hence we can avoid defining two NAL unit types for this purpose.

Moreover, a CRA picture or a BLA picture with no associated TFD pictures can map to SAP types 1 or 2 depending on whether it has leading pictures or not. We propose to use a new NAL unit type to indicate the case where a CRA picture or a BLA picture directly maps to SAP type 1 (occurs when the CRA picture or BLA picture has no leading pictures).

We propose a reallocation of NAL unit types 4 to 7 as follows:

| NAL unit type | Description | SAP types possible |
| --- | --- | --- |
| 4 | CRA picture | 1, 2, 3 |
| 5 | BLA picture | 1, 2, 3 |
| 6 | BLA picture with no associated TFD pictures | 1, 2 |
| 7 | BLA picture with no leading pictures | 1 |

## Constraint on the bitstream order of leading pictures

When the system has to map a RAP picture into one of the possible SAP types, it has to check the bitstream for the presence of leading pictures and when they are present, the system has to check whether any of them is a TFD picture. Leading pictures of the current RAP picture can be present anywhere in the bitstream after the current RAP picture and the next RAP picture.

In order to make this search simpler, we propose that a constraint be placed to ensure the occurrence of all leading pictures in the bitstream prior to any non-leading picture.

# Conclusion

We recommend that the above constraints and reallocation of NAL unit types be adopted into HEVC design.

# References

- B. Bross et. al, “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003, 9th meeting, Geneva, April 2012.

- G. J. Sullivan, “CRA pictures with broken links”, JCTVC-I0404, 9th meeting, Geneva, April 2012.

- ISO/IEC 14496-12 4th Edition, “Information technology — Coding of audio-visual objects — Part 12: ISO base media file format”, w12640, 100th MPEG meeting, Geneva, April 2012.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).