| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0346r1 |
| --- | --- |

| Title: | Unifying ALF coefficient coding with coeff_abs_level_remaining coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Yue Yu Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2251 yyu@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In the current HEVC, fixed k-parameter Exp-Golomb code is used for ALF coefficient binarization. However, k-th Exp-Golomb code is only used for ALF coefficient coding which introduces extra complexity. Notice that coeff_abs_level_remaining is binarized by a unary code and a variable length code. The length of the variable length code depends on the unary code and a parameter (ranged from 0 to 4). It is proposed to unify the ALF coefficient coding with coeff_abs_level_remaining coding.

# Introduction

In the current HEVC, 10-tap symmetric 2D FIR filter is used as the adaptive loop filter to achieve better coding efficiency. It was adopted that fixed k-parameter Exp-Golomb code is used for ALF coefficient coding [1]. Notice that k-th Exp-Golomb code is only used for ALF coefficient coding which introduces extra complexity in HEVC.

In the current HEVC coefficient level coding, syntax element coeff_abs_level_remaining is used to represent the remaining of the absolute value of a quantized transform coefficient level. It was adopted that coeff_abs_level_remaining is binarized by a unary code and a variable length code. The length of the variable length code depends on the unary code and a parameter (ranged from 0 to 4) [2].

It is proposed to unify the ALF coefficient coding with coeff_abs_level_remaining coding by using the same binarization scheme for both of them.

# Unifying ALF coefficient coding with coeff_abs_level_remaining coding

Scheme 1: It is proposed to apply the scheme of coeff_abs_level_remaining binarization, i.e. the combination of a unary code and a variable length code with different parameters (0-5), to Luma ALF coefficient coding while keeping the Chroma ALF coefficient coding unchanged, i.e., using 0th order EG code.

Scheme 2: In addition to scheme 1, The Chroma ALF coefficients are binarized with a unary code and a variable length code with parameter 0.

Scheme 3: Both the Luma and Chroma ALF coefficients are binarized with a unary code and a variable length code with parameter 3.

# Simulation results

The software used to run the simulations is HM7.0 [3]. The coding conditions used for the simulations are specified in [4]. The experimental results of the rate-distortion performance are summarized in Table 1-6. Detailed results are included in the attachments (JCTVC-J0346Scheme1QP22-37.xls, JCTVC-J0346Scheme1QP32-47.xls, JCTVC-J0346Scheme2QP22-37.xls, JCTVC-J0346Scheme2QP32-47.xls, JCTVC-J0346Scheme3QP22-37.xls and JCTVC-J0346Scheme3QP32-47.xls). Cross-check will be provided by Samsung [5].

Table 1 Rate-Distortion performance of proposed scheme 1 (QP22-37)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% |
| Class B | -0.01% | 0.00% | 0.00% |
| Class C | -0.01% | 0.00% | 0.00% |
| Class D | -0.01% | 0.00% | 0.00% |
| Class E | -0.01% | 0.00% | 0.00% |
| Overall | -0.01% | 0.00% | 0.00% |
|  | -0.01% | 0.00% | 0.00% |
| Class F | -0.01% | 0.00% | 0.00% |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.03% | -0.18% | 0.12% |
| Class B | -0.02% | 0.00% | -0.02% |
| Class C | -0.02% | -0.07% | -0.05% |
| Class D | -0.04% | 0.07% | -0.03% |
| Class E |  |  |  |
| Overall | -0.01% | -0.04% | 0.00% |
|  | -0.01% | -0.04% | 0.00% |
| Class F | 0.00% | 0.01% | 0.00% |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.04% | -0.15% | 0.14% |
| Class C | -0.02% | -0.10% | -0.04% |
| Class D | -0.05% | 0.32% | 0.46% |
| Class E | 0.04% | -0.12% | -0.18% |
| Overall | -0.02% | -0.01% | 0.11% |
|  | -0.02% | 0.04% | 0.16% |
| Class F | 0.00% | 0.00% | 0.61% |

Table 2 Rate-Distortion performance of proposed scheme 1 (QP32-47)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | -0.01% | 0.00% | 0.00% |
| Class D | -0.01% | 0.00% | 0.00% |
| Class E | -0.01% | 0.00% | 0.00% |
| Overall | -0.01% | 0.00% | 0.00% |
|  | -0.01% | 0.00% | 0.00% |
| Class F | -0.01% | 0.00% | 0.00% |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.04% | 0.21% | 0.36% |
| Class B | 0.00% | 0.02% | 0.01% |
| Class C | -0.01% | 0.00% | -0.04% |
| Class D | 0.01% | 0.11% | 0.06% |
| Class E |  |  |  |
| Overall | 0.01% | 0.08% | 0.09% |
|  | 0.01% | 0.07% | 0.11% |
| Class F | -0.02% | -0.02% | -0.01% |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.02% | 0.28% | -0.02% |
| Class C | 0.11% | 0.32% | 0.29% |
| Class D | -0.06% | 1.57% | 0.92% |
| Class E | -0.05% | -0.43% | -0.64% |
| Overall | 0.01% | 0.48% | 0.18% |
|  | 0.01% | 0.48% | 0.33% |
| Class F | -0.11% | -0.14% | 0.04% |

Table 3 Rate-Distortion performance of proposed scheme 2 (QP22-37)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | 0.05% | 0.04% |
| Class B | 0.01% | 0.01% | 0.01% |
| Class C | 0.02% | 0.02% | 0.02% |
| Class D | -0.01% | 0.49% | 0.47% |
| Class E | 0.04% | 0.04% | 0.04% |
| Overall | 0.01% | 0.12% | 0.12% |
|  | 0.01% | 0.12% | 0.12% |
| Class F | 0.01% | 0.03% | 0.07% |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.00% | 0.05% | 0.05% |
| Class B | -0.06% | 0.17% | 0.21% |
| Class C | -0.04% | 0.19% | 0.25% |
| Class D | -0.05% | 0.38% | 0.30% |
| Class E |  |  |  |
| Overall | -0.04% | 0.20% | 0.20% |
|  | -0.04% | 0.21% | 0.21% |
| Class F | -0.01% | 0.08% | 0.12% |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.06% | 0.18% | 0.56% |
| Class C | -0.09% | 0.48% | 0.31% |
| Class D | -0.08% | 0.60% | 0.97% |
| Class E | -0.01% | 0.38% | 0.57% |
| Overall | -0.06% | 0.40% | 0.60% |
|  | -0.06% | 0.45% | 0.67% |
| Class F | 0.11% | 0.33% | 1.32% |

Table 4 Rate-Distortion performance of proposed scheme 2 (QP32-47)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.01% | 0.11% | -0.11% |
| Class B | 0.01% | 0.03% | 0.21% |
| Class C | 0.01% | 0.39% | 0.44% |
| Class D | -0.12% | 1.49% | 2.19% |
| Class E | 0.12% | 0.18% | 0.21% |
| Overall | 0.00% | 0.43% | 0.59% |
|  | 0.00% | 0.45% | 0.80% |
| Class F | -0.02% | 0.31% | 0.44% |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.00% | 1.02% | 0.87% |
| Class B | -0.06% | 0.35% | 0.28% |
| Class C | -0.09% | 0.36% | 0.59% |
| Class D | -0.05% | 0.68% | 0.81% |
| Class E |  |  |  |
| Overall | -0.05% | 0.59% | 0.62% |
|  | -0.05% | 0.55% | 0.62% |
| Class F | -0.03% | 0.29% | 0.20% |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.17% | 1.10% | 1.16% |
| Class C | -0.16% | 1.07% | 0.74% |
| Class D | -0.13% | 1.73% | 1.23% |
| Class E | -0.16% | 0.28% | 1.63% |
| Overall | -0.16% | 1.10% | 1.16% |
|  | -0.16% | 1.14% | 1.30% |
| Class F | -0.11% | 0.76% | 2.23% |

Table 5 Rate-Distortion performance of proposed scheme 3 (QP22-37)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.02% | -0.01% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | -0.01% | -0.01% | -0.01% |
| Class D | -0.01% | -0.29% | -0.21% |
| Class E | 0.00% | -0.02% | -0.02% |
| Overall | 0.00% | -0.07% | -0.05% |
|  | 0.00% | -0.07% | -0.06% |
| Class F | 0.00% | -0.01% | 0.01% |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.04% | -0.21% | 0.16% |
| Class B | 0.04% | -0.10% | -0.19% |
| Class C | 0.03% | -0.16% | -0.27% |
| Class D | 0.03% | -0.26% | -0.29% |
| Class E |  |  |  |
| Overall | 0.04% | -0.18% | -0.15% |
|  | 0.04% | -0.19% | -0.17% |
| Class F | 0.02% | 0.01% | 0.08% |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.01% | -0.06% | -0.02% |
| Class C | 0.05% | -0.34% | -0.22% |
| Class D | 0.00% | -0.29% | -0.50% |
| Class E | 0.21% | -0.60% | -0.26% |
| Overall | 0.05% | -0.29% | -0.24% |
|  | 0.05% | -0.27% | -0.18% |
| Class F | 0.10% | 0.08% | 0.42% |

Table 6 Rate-Distortion performance of proposed scheme 3 (QP32-47)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.01% | -0.02% | 0.12% |
| Class B | 0.00% | -0.01% | -0.05% |
| Class C | -0.02% | -0.16% | -0.15% |
| Class D | 0.08% | -1.64% | -2.05% |
| Class E | -0.05% | -0.07% | -0.07% |
| Overall | 0.01% | -0.38% | -0.44% |
|  | 0.01% | -0.31% | -0.45% |
| Class F | 0.02% | -0.14% | -0.07% |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.12% | -0.14% | 0.18% |
| Class B | 0.07% | -0.07% | -0.26% |
| Class C | 0.05% | -0.39% | -0.33% |
| Class D | 0.12% | -0.74% | -0.42% |
| Class E |  |  |  |
| Overall | 0.09% | -0.32% | -0.21% |
|  | 0.09% | -0.32% | -0.23% |
| Class F | 0.03% | -0.20% | -0.03% |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.11% | 0.30% | -0.41% |
| Class C | 0.19% | -0.26% | -0.33% |
| Class D | -0.08% | 0.53% | -1.05% |
| Class E | 0.17% | -1.80% | -1.65% |
| Overall | 0.09% | -0.18% | -0.78% |
|  | 0.10% | -0.23% | -0.62% |
| Class F | 0.10% | -0.96% | -0.70% |

# Draft changes

Scheme 1:

–	kes(v): signed syntax element using unsigned integer k-th order Exp-Golomb codes combined tu(n) and u(n) with the left bit first for the absolute value and one bit specifying the sign at the end. The parsing process for this descriptor is specified in subclause 9.2.2 9.3.2.8.

Scheme 2:

–	kes(v): signed syntax element using unsigned integer k-th order Exp-Golomb codes combined tu(n) and u(n) with the left bit first for the absolute value and one bit specifying the sign at the end. The parsing process for this descriptor is specified in subclause 9.2.2 9.3.2.8.

| alf_picture_info( cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_idc | ue(v) |
| if( alf_no_filters_idc = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_idc > 1 ) |  |
| for( i=0; i<15; i++ ) |  |
| alf_filter_pattern_flag[ i ] | u(1) |
| for( i = 0; i < AlfNumFilters; i++ ) |  |
| for( j = 0; j < 10; j++ ) |  |
| alf_filt_coeff[ cIdx ][ i ][ j ] | kes(v) |
| } else |  |
| for( j = 0; j < 10; j++ ) |  |
| alf_filt_coeff[ cIdx ][ 0 ][ j ] | sekes(v) |
| } |  |

Scheme 3:

alf_filt_coeff[ cIdx ][ i ][ j ] specifies the j-th filter coefficient of i-th filter used in the adaptive loop filtering process for the colour component cIdx. When cIdx is equal to 0, k-th order exponential-golomb (Exp-Golomb) code with a sign bit at the end is used where the order k for the j-th filter coefficient is equal to the value of alfkMap[ j ] which is specified as follows:

alfkMap[ j ] = {1, 2, 3, 4, 3, 1, 3, 3, 5, 0 } with j = 0..9	(7-39)

### 9.2.2 Parsing process for k-th order Exp-Golomb codes

This process is invoked when the descriptor of a syntax element in the syntax tables in subclause 7.3 is equal to kes(v).

Inputs to this process are bits from the RBSP.

Outputs of this process are syntax element values.

Syntax elements coded as kes(v) are k-th order Exp-Golomb-coded followed by a sign bit. The parsing process for these syntax elements begins with reading the bits starting at the current location in the bitstream up to and including the first zero bit, and counting the number of leading bits that are equal to 1. This process is specified as follows:

leadingOneBits = –1
for( b = 1; b!=0; leadingOneBits++ )	(9-4)
	b = read_bits( 1 )

The variable codeNum is then assigned as follows:

codeNum = (2leadingOneBits – 1) * 2k + read_bits( leadingOneBits+k )	(9-5)

where the value returned from read_bits( leadingOneBits+k ) is interpreted as a binary representation of an unsigned integer with most significant bit written first. When codeNum is not equal to zero, one signed bit is parsed as follows:

if ( codeNum != 0 ) {
	sign = read_bits( 1 )	(9-5)
	codeNum = (sign = = 1) ? codeNum : (−codeNum)
}

The value of a syntax element is equal to codeNum.

| alf_picture_info( cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_idc | ue(v) |
| if( alf_no_filters_idc = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_idc > 1 ) |  |
| for( i=0; i<15; i++ ) |  |
| alf_filter_pattern_flag[ i ] | u(1) |
| for( i = 0; i < AlfNumFilters; i++ ) |  |
| for( j = 0; j < 10; j++ ) |  |
| alf_filt_coeff[ cIdx ][ i ][ j ] | kes(v) |
| alf_filt_coeff_prefix[ cIdx ][ i ][ j ] | tu(v) |
| alf_filt_coeff_suffix[ cIdx ][ i ][ j ] | u(v) |
| } else |  |
| for( j = 0; j < 10; j++ ) |  |
| alf_filt_coeff[ cIdx ][ 0 ][ j ] | se(v) |
| alf_filt_coeff_prefix[ cIdx ][ 0 ][ j ] | tu(v) |
| alf_filt_coeff_suffix[ cIdx ][ 0 ][ j ] | u(v) |
| } |  |

# References

[1] M. Budagavi, “Simplification of ALF filter coefficients coding,” JCT-VC document, JCTVC-I0346, April 2012

[2] W.-J. Chien, M. Karczewicz, J. Sole, and J. Chen, “On coefficient level remaining coding,” JCT-VC document, JCTVC-I0487, April 2012

[3] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-7.0/

[4] F. Bossen, “Common HM test conditions and software reference configurations”, JCT-VC document, JCTVC-I1100, April 2012

[5] Z. Ma and F.C. Fernandes, “Cross-check for J0346 on Unifying ALF coefficient coding with coeff_abs_level_remaining coding,” JCT-VC document, JCTVC-J0434, July 2012

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).