| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0347 |
| --- | --- |

| Title: | AHG6/AHG5: Simplified SAO coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | In Suk Chong, J. Sole, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Chih-Ming Fu, Chih-Wei Hsu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 E. Alshina, A. Alshin, J.H. Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | In Suk Chong +1-858-658-5673 {ichong, joels, martak} @qualcomm.com Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, chihming.fu, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com +82 10 3026 1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com |
| Source: | Qualcomm Inc., MediaTek Inc., Samsung |  |  |

_____________________________

# Abstract

In HM-7.0, each LCU has one set of SAO signaling for each color component. Each set of SAO signalling includes merge signals (i.e., merge_up and merge_left) and if merge signals are not used, new set of SAO syntax (i.e., sao_type_idx, sao_offset_abs, sao_offset_sign, and sao_band_position) is sent to the decoder. In this proposal, we propose to simplify above SAO signaling, especially for merge  flags, sao_type_idx. Results reportedly show 0.0%/0.3%/0.3%/0.4% luma coding gains for Main-AI/RA/LB/LP, respectively. We further propose to simplify sao_offset_abs coding. Results reportedly show 0.0%/0.3%/0.3%/0.3% luma coding gains for Main-AI/RA/LB/LP, respectively.

# Introduction

In HM-7.0, SAO uses LCU-based syntax. Each LCU has one set of SAO signaling for each color component. Each set of SAO signalling includes merge flags (i.e., merge_up and merge_left) and if merge are not used, new set of SAO syntax (i.e., sao_type_idx, sao_offset_abs, sao_offset_sign, and sao_band_position) is signaled. As shown in Table 1, these syntax elements are coded with either regular bins or bypass bins. The majority of them are coded with contexts.

In this proposal, we propose a simplfied SAO syntax coding that reduces the number of contexts, the maximum number of coded bins, and the maximum number of context-coded bins. Note that for sao_offset_abs, there are three cases depending on the bit-depth (8/9/10 or higher).

| Syntax element | Bin Type | # of contexts | Max # of bins | Max # of ctx bins |
| --- | --- | --- | --- | --- |
| merge_up | U(1) with Regular | 1 | 1 | 1 |
| merge_left | U(1) with Regular | 3 | 1 | 1 |
| sao_type_idx | U with Regular | 6 | 6 | 6 |
| if( sao_type_idx = = 5 ) |  |  |  |  |
| sao_band_position | FL(5) with Bypass | - | 5 | - |
| sao_offset_abs | TU with Regular | 6 | (7/15/31)*4 | (7/15/31)*4 |
| sao_offset_sign | U(1) with Bypass | - | 1*4 | - |
| Total |  | 16 | 45/77/141 | 36/68/132 |

Table 1. Number of contexts, worst-case total bins and worst-case context-coded bins for HM7.0 SAO syntax elements (separation x/x/x indicates bins for bit-depth 8/9/10).

# Proposed method

In the proposed method, we used the approach in [1] for merge signal coding and the approach in [2, 3] for sao_type_idx coding. Proposal [1] decreases the number of contexts from 4 to 1 without loss and proposals [2, 3] reduce the number of contexts from 6 to 2 and number of maximum context-coded bins from 6 to 1. Table 2 summarizes the context usage and worst-cases for the combination. Numbers to be compared with Table 1.

| Syntax element | Bin Type | # of contexts | Max # of bins | Max # of ctx bins |
| --- | --- | --- | --- | --- |
| sao_lcu_enable | U(1) with Regular | 2 | 1 | 1 |
| merge_up | U(1) with Regular | 1 | 1 | 1 |
| merge_left | U(1) with Regular | - | 1 | 1 |
| sao_type_idx | TU with Regular/Bypass | 0 | 4 | 0 |
| if( sao_type_idx = = 5 ) |  |  |  |  |
| sao_band_position | FL(5) with Bypass | - | 5 | - |
| sao_offset_abs | TU with Regular | 6 | (7/15/31)*4 | (7/15/31)*4 |
| sao_offset_sign | U(1) with Bypass | - | 4 | - |
| Total |  | 9 | 44/76/140 | 31/63/127 |

Table 2. SAO syntax elements for proposed method [1] + [2] + [3]

When we further combine the approach in [4] for sao_offset_abs coding, the number of maximum coded bins and maximum context-coded bins reduces significantly, as can be shown in Table 3. For sao_offset_abs coding in [4], we chose to use the solution with all bins bypass coded.

With this combination and coding sao_offset_abs in bypass, the worst-case context-coded bins is reduced form 132 to 3 for bit-depth of 10 and reduced from 36 to 3 for bit-depth equals 8.

| Syntax element | Bin Type | # of contexts | Max # of bins | Max # of ctx bins |
| --- | --- | --- | --- | --- |
| sao_lcu_enable | U(1) with Regular | 2 | 1 | 1 |
| merge_up | U(1) with Regular | 1 | 1 | 1 |
| merge_left | U(1) with Regular | - | 1 | 1 |
| sao_type_idx | TU with Regular/Bypass | 0 | 4 | 0 |
| if( sao_type_idx = = 5 ) |  |  |  |  |
| sao_band_position | FL(5) with Bypass | - | 5 | - |
| sao_offset_abs | TU with Regular | 0 | (5/7/9)*4 | (0/0/0)*4 |
| sao_offset_sign | U(1) with Bypass | - | 4 | - |
| Total |  | 3 | 36/44/52 | 3/3/3 |

Table 3. SAO syntax elements for proposed method [1] + [2] + [3] + [4]

We also tested the approach in [5] for sao_offset_abs coding on top of [1]+[2]+[3], the number of maximum coded bins and maximum context-coded bins reduces significantly, as can be shown in Table 3. For sao_offset_abs coding in [5], we chose to use the solution with all bins bypass coded.

With this combination and coding sao_offset_abs in bypass, the worst-case context-coded bins is reduced form 132 to 3 for bit-depth of 10 and reduced from 36 to 3 for bit-depth equals 8.

| Syntax element | Bin Type | # of contexts | Max # of bins | Max # of ctx bins |
| --- | --- | --- | --- | --- |
| sao_lcu_enable | U(1) with Regular | 2 | 1 | 1 |
| merge_up | U(1) with Regular | 1 | 1 | 1 |
| merge_left | U(1) with Regular | - | 1 | 1 |
| sao_type_idx | TU with Regular/Bypass | 0 | 4 | 0 |
| if( sao_type_idx = = 5 ) |  |  |  |  |
| sao_band_position | FL(5) with Bypass | - | 5 | - |
| sao_offset_abs | TU with Regular | 0 | (7/15/31)*4 | (0/0/0)*4 |
| sao_offset_sign | U(1) with Bypass | - | 4 | - |
| Total |  | 3 | 44/76/140 | 3/3/3 |

Table 3. SAO syntax elements for proposed method [1] + [2] + [3] + [5]

# Simulation results

Experiments were conducted for Main-AI/RA/LB/LP conditions [6]. For [1]+[2]+[3], Results show 0.0%/0.3%/0.3%/0.4% luma coding gains for Main-AI/RA/LB/LP, respectively. For [1]+[2]+[3]+[4], Results show 0.0%/0.3%/0.3%/0.3% luma coding gains for Main-AI/RA/LB/LP, respectively.

|  |  | All Intra Main |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V |  |
| Class A |  | 0.0% |  | 0.1% |  | 0.1% |  |
| Class B |  | 0.0% |  | 0.1% |  | 0.0% |  |
| Class C |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Class D |  | 0.0% |  | -0.2% |  | -0.3% |  |
| Class E |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Class F |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Overall |  | 0.0% |  | 0.0% |  | 0.0% |  |
|  |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Enc Time[%] |  | 101% |  |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  |
|  |  | Random Access Main |  |  |  |  |  |
|  |  | Y |  | U |  | V |  |
| Class A |  | -0.2% |  | -0.2% |  | -0.3% |  |
| Class B |  | -0.2% |  | -0.1% |  | -0.2% |  |
| Class C |  | -0.3% |  | -0.2% |  | -0.2% |  |
| Class D |  | -0.6% |  | -0.4% |  | -0.4% |  |
| Class E |  |  |  |  |  |  |  |
| Class F |  | -0.3% |  | -0.1% |  | -0.1% |  |
| Overall |  | -0.3% |  | -0.2% |  | -0.2% |  |
|  |  | -0.3% |  | -0.2% |  | -0.2% |  |
| Enc Time[%] |  | 101% |  |  |  |  |  |
| Dec Time[%] |  | 99% |  |  |  |  |  |
|  |  | Low delay B Main |  |  |  |  |  |
|  |  | Y |  | U |  | V |  |
| Class A |  |  |  |  |  |  |  |
| Class B |  | -0.1% |  | -0.2% |  | -0.3% |  |
| Class C |  | -0.1% |  | 0.1% |  | 0.1% |  |
| Class D |  | -0.4% |  | 0.2% |  | 0.9% |  |
| Class E |  | -0.5% |  | -0.7% |  | -0.9% |  |
| Class F |  | -0.6% |  | 0.0% |  | -0.4% |  |
| Overall |  | -0.3% |  | -0.1% |  | -0.1% |  |
|  |  | -0.3% |  | -0.1% |  | -0.1% |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  |
|  |  | Low delay P Main |  |  |  |  |  |
|  |  | Y |  | U |  | V |  |
| Class A |  |  |  |  |  |  |  |
| Class B |  | -0.4% |  | -0.4% |  | -0.6% |  |
| Class C |  | -0.2% |  | -0.1% |  | 0.0% |  |
| Class D |  | -0.3% |  | 0.8% |  | 1.5% |  |
| Class E |  | -0.7% |  | -1.3% |  | -0.7% |  |
| Class F |  | -0.6% |  | -0.4% |  | -0.1% |  |
| Overall |  | -0.4% |  | -0.2% |  | 0.0% |  |
|  |  | -0.4% |  | -0.2% |  | 0.0% |  |
| Enc Time[%] |  | 99% |  |  |  |  |  |
| Dec Time[%] |  | 102% |  |  |  |  |  |

Table 4. Results for [1] + [2] + [3]

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | -0.1% | 0.0% |
| Class B | -0.1% | 0.3% | 0.2% |
| Class C | -0.3% | 0.1% | 0.2% |
| Class D | -0.6% | -0.3% | -0.2% |
| Class E |  |  |  |
| Class F | -0.3% | 0.0% | 0.0% |
| Overall | -0.3% | 0.0% | 0.0% |
|  | -0.3% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | 0.6% | 0.4% |
| Class C | -0.1% | 0.5% | 0.7% |
| Class D | -0.4% | 0.6% | 1.1% |
| Class E | -0.4% | -0.3% | -0.2% |
| Class F | -0.4% | -0.2% | 0.2% |
| Overall | -0.3% | 0.3% | 0.5% |
|  | -0.3% | 0.3% | 0.5% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | 0.7% | 0.4% |
| Class C | -0.2% | 0.4% | 0.7% |
| Class D | -0.3% | 1.1% | 1.0% |
| Class E | -0.5% | 0.2% | 0.9% |
| Class F | -0.5% | -0.1% | -0.4% |
| Overall | -0.3% | 0.5% | 0.5% |
|  | -0.3% | 0.4% | 0.4% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 102% |  |  |

Table 5. Results for [1] + [2] + [3] + [4]

| All Intra Main. Y; U; V. Class A; 0.0%; 0.2%; 0.2%. Class B; 0.0%; 0.3%; 0.2%. Class C; 0.0%; 0.1%; 0.1%. Class D; 0.0%; -0.2%; -0.2%. Class E; 0.0%; 0.2%; 0.3%. Class F; 0.0%; 0.0%; 0.0%. Overall; 0.0%; 0.1%; 0.1%. 0.0%; 0.1%; 0.1%. Enc Time[%]; 101%. Dec Time[%]; 101%. Random Access Main. Y; U; V. Class A; -0.1%; -0.1%; -0.2%. Class B; -0.1%; 0.2%; 0.1%. Class C; -0.3%; 0.0%; 0.1%. Class D; -0.6%; -0.5%; -0.4%. Class E. Class F; -0.2%; 0.0%; 0.0%. Overall; -0.2%; -0.1%; -0.1%. -0.2%; -0.1%; -0.1%. Enc Time[%]; 100%. Dec Time[%]; 99%. Low delay B Main. Y; U; V. Class A. Class B; -0.1%; 0.4%; 0.2%. Class C; -0.1%; 0.4%; 0.4%. Class D; -0.4%; -0.1%; 1.2%. Class E; -0.6%; -0.3%; 0.3%. Class F; -0.3%; -0.4%; 0.3%. Overall; -0.3%; 0.0%; 0.5%. -0.3%; 0.0%; 0.5%. Enc Time[%]; 101%. Dec Time[%]; 101%. Low delay P Main. Y; U; V. Class A. Class B; -0.2%; 0.4%; 0.5%. Class C; -0.2%; 0.3%; 0.5%. Class D; -0.3%; 0.4%; 0.9%. Class E; -0.6%; -0.1%; 1.0%. Class F; -0.5%; -0.5%; -0.2%. Overall; -0.3%; 0.1%; 0.5%. -0.3%; 0.1%; 0.5%. Enc Time[%]; 100%. Dec Time[%]; 102%. |  |  |  |
| --- | --- | --- | --- |

Table 6. Results for [1] + [2] + [3] + [5]

# Conclusions

In this contribution, a simplified SAO syntax coding. For [1]+[2]+[3], results show 0.0%/0.3%/0.3%/0.4% luma coding gains for Main-AI/RA/LB/LP, respectively. For [1]+[2]+[3]+[4], results show 0.0%/0.3%/0.3%/0.3% luma coding gains for Main-AI/RA/LB/LP, respectively. For [1]+[2]+[3]+[5], results show 0.0%/0.2%/0.3%/0.3% luma coding gains for Main-AI/RA/LB/LP, respectively. We propose to adopt this proposal for HEVC.

# References

- E. Alshina, A.Alshina, J. H. Park, C.-M. Fu, Y.-W. Huang, S. Lei, “AHG5/AHG6: On reducing context models for SAO merge syntax”, Joint Collaborative Team on Video Coding, JCTVC-J0041, July, 2012.

- I. S. Chong, J. Sole, M. Karczewicz, C.-M. Fu, Y.-W. Huang, S. Lei, “AHG6/AHG5: Fix and simplification for SAO type index”, Joint Collaborative Team on Video Coding, JCTVC-J0104, July, 2012.

- I. S. Chong, J. Sole, M. Karczewicz, C.-W. Hsu, C.-M. Fu, T.-D. Chuang, Y.-W. Huang, S. Lei, “AHG6: Decoupling SAO LCU on/off from SAO type”, Joint Collaborative Team on Video Coding, JCTVC-J0103, July, 2012.

- I. S. Chong, J. Sole, M. Karczewicz, “AHG6: SAO offset coding”, Joint Collaborative Team on Video Coding, JCTVC-J0106, July, 2012.

- E. Alshina, A.Alshina, J. H. Park, “AhG5: On bypass coding for SAO syntax elements”, Joint Collaborative Team on Video Coding, JCTVC-J0043, July, 2012.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).