| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0351 |
| --- | --- |

| Title: | AHG5: Cross-verification of JCTVC-J0194 on max exponential Golomb code for reducing number of bins |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono Kenta Tokumitsu 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 44 431 7588 chono@ct.jp.nec.com k-tokumitsu@cw.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of JCTVC-J0194 on max exponential Golomb code. The cross-verification task was successfully completed and it is confirmed that their software implementation matches with the descriptions.

- Analysis

Through software inspection, it was confirmed that an Exponential Golomb coding, which uses TU representation for the prefix part configured by a parameter indicating the maximum length of the prefix, is implemented according to the JCTVC-J0194 description. The software was successfully compiled and run on Window-based computing platform.

- Simulation results

The Exponential Golomb coding is applied to ref_idx_lx, cu_qp_delta, sao_offset_abs, and abs_mvd_minus2 syntax elements, respectively. The summary of simulation results for ref_idx_lx, cu_qp_delta, and sao_offset_abs tests are shown below.

Table1: JCTVC-J0194 ref_idx_lx test results

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.01% | -0.17% | -0.15% | 0.01% | -0.13% | 0.03% |
| Class B | 0.01% | -0.02% | 0.00% | 0.00% | 0.01% | 0.02% |
| Class C | -0.01% | 0.05% | 0.06% | 0.01% | -0.03% | -0.06% |
| Class D | 0.02% | -0.04% | -0.02% | -0.02% | 0.04% | -0.01% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | -0.05% | -0.02% | 0.00% | -0.03% | 0.00% |
|  | 0.01% | -0.04% | -0.03% | 0.00% | -0.02% | -0.01% |
| Class F | 0.01% | 0.06% | 0.02% | -0.01% | 0.00% | 0.04% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 106% |  |  | 114% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.01% | 0.06% | -0.01% | 0.00% | -0.05% | 0.31% |
| Class C | -0.04% | 0.04% | 0.03% | 0.00% | -0.07% | 0.08% |
| Class D | -0.03% | 0.16% | 0.23% | 0.02% | 0.11% | -0.27% |
| Class E | -0.03% | 0.07% | -0.09% | 0.12% | -0.34% | -0.04% |
| Overall | -0.02% | 0.08% | 0.05% | 0.03% | -0.07% | 0.04% |
|  | -0.02% | 0.03% | 0.03% | 0.03% | -0.02% | 0.07% |
| Class F | -0.05% | 0.31% | 0.08% | 0.05% | 0.37% | 0.81% |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 105% |  |  | 108% |  |  |

Table2: JCTVC-J0194 cu_qp_delta test results

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.16% | 0.12% | 0.12% | 0.17% | 0.06% | 0.16% |
| Class B | 0.15% | 0.11% | 0.12% | 0.15% | 0.11% | 0.11% |
| Class C | 0.19% | 0.13% | 0.17% | 0.20% | 0.13% | 0.14% |
| Class D | 0.19% | 0.16% | 0.17% | 0.20% | 0.15% | 0.17% |
| Class E | 0.29% | 0.21% | 0.22% | 0.29% | 0.21% | 0.24% |
| Overall | 0.19% | 0.14% | 0.15% | 0.19% | 0.13% | 0.16% |
|  | 0.19% | 0.14% | 0.15% | 0.19% | 0.13% | 0.16% |
| Class F | 0.18% | 0.18% | 0.25% | 0.22% | 0.14% | 0.11% |
| Enc Time[%] | 95% |  |  | 98% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.12% | -0.09% | 0.28% | 0.14% | 0.05% | 0.48% |
| Class B | 0.17% | 0.05% | 0.18% | 0.18% | 0.16% | 0.31% |
| Class C | 0.24% | 0.20% | 0.30% | 0.22% | 0.18% | 0.17% |
| Class D | 0.22% | 0.41% | 0.37% | 0.23% | 0.43% | -0.35% |
| Class E |  |  |  |  |  |  |
| Overall | 0.18% | 0.14% | 0.28% | 0.19% | 0.20% | 0.16% |
|  | 0.19% | 0.15% | 0.26% | 0.19% | 0.19% | 0.21% |
| Class F | 0.20% | 0.11% | 0.21% | 0.26% | 0.10% | 0.06% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 93% |  |  | 92% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.18% | -0.10% | 0.06% | 0.20% | 0.28% | 0.89% |
| Class C | 0.23% | 0.24% | 0.19% | 0.20% | 0.13% | 0.10% |
| Class D | 0.20% | 0.50% | 1.42% | 0.25% | 0.22% | 0.35% |
| Class E | 0.07% | 0.10% | 0.15% | 0.04% | 0.23% | 0.45% |
| Overall | 0.18% | 0.17% | 0.45% | 0.18% | 0.22% | 0.47% |
|  | 0.18% | 0.13% | 0.40% | 0.18% | 0.30% | 0.50% |
| Class F | 0.31% | 0.08% | 0.49% | 0.28% | -0.05% | 0.96% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 94% |  |  | 92% |  |  |

Table3: JCTVC-J0194 sao_abs_offset test results (Missing data will be updated.)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.03% | 0.01% | 0.00% | 0.06% | 0.08% |
| Class B | 0.00% | 0.05% | 0.06% | 0.01% | 0.07% | 0.12% |
| Class C | 0.00% | 0.10% | 0.12% | 0.01% | 0.10% | 0.14% |
| Class D | 0.00% | 0.08% | 0.10% | 0.01% | 0.05% | 0.07% |
| Class E | -0.01% | 0.14% | 0.15% | 0.02% | 0.06% | 0.09% |
| Overall | 0.00% | 0.07% | 0.09% | 0.01% | 0.07% | 0.10% |
|  | 0.00% | 0.07% | 0.08% | 0.01% | 0.06% | 0.10% |
| Class F | 0.02% | 0.09% | 0.14% | 0.02% | 0.15% | 0.16% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.17% | 0.02% | 0.06% | 0.02% | 0.15% |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.01% | 0.11% | 0.14% |
| Class C | 0.00% | 0.08% | 0.12% | 0.00% | 0.08% | 0.03% |
| Class D | 0.00% | 0.20% | 0.13% | 0.01% | 0.21% | 0.05% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.02% | 0.11% | 0.10% |
|  | #VALUE! | #VALUE! | #VALUE! | 0.02% | 0.10% | 0.09% |
| Class F | 0.06% | 0.15% | 0.24% | 0.04% | 0.24% | 0.29% |
| Enc Time[%] | #NUM! |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.03% | 0.11% | 0.04% | 0.05% | 0.18% | 0.29% |
| Class C | 0.02% | 0.17% | 0.25% | 0.04% | 0.14% | 0.29% |
| Class D | -0.01% | 0.15% | 0.27% | 0.01% | 0.36% | 0.34% |
| Class E | -0.01% | -0.02% | -0.15% | 0.16% | -0.04% | 0.21% |
| Overall | 0.01% | 0.11% | 0.12% | 0.06% | 0.17% | 0.29% |
|  | 0.01% | 0.13% | 0.16% | 0.05% | 0.18% | 0.31% |
| Class F | 0.06% | 0.18% | 0.42% | 0.13% | 0.12% | 1.48% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

- Conclusion

It is confirmed that software implementation matches with the JCTVC-J0194 descriptions.

# Patent rights declarations

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).