| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0352 |
| --- | --- |

| Title: | BD-rate performance vs. dictionary size and hash-table memory size in Dual-coder Mixed Chroma-sampling-rate (DMC) coding for 4:4:4 screen content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Peijun Zhang Tao Lin Xianyi Chen Shuhui Wang Kailun Zhou | Tel: Email: | 86-21-5103-0961 lintao@tongji.edu.cn |
| Source: | Tongji University, Shanghai, China |  |  |

_____________________________

# Abstract

This contribution presents BD-rate performance comparison of Dual-coder Mixed Chroma-sampling-rate (DMC) coding for full-chroma (YUV444) screen content using different dictionary (as part of DPB) size from 4MB to 16KB and hash-table memory size from 16MB to 10.5KB.

# Introduction

In DMC coding [1] - [5], a hash-table based dictionary encoder is used to find best matching string. Figure 1 shows the dictionary encoder architecture.

Dictionary (= part of Constructed Pixel Buffer)

Search Engine

to Find Best Matching String

Output: unmatchable sample residual if no good match or distance/length of the best matching string

Hash Table: Linked Lists of Distances

Input Sample Buffer

Hash Function

Input Sample

Three Samples

(24-bit)

Hash Value

(8-16 bits)

Distance to Position of First Three Matching Samples

- Dictionary encoder architecture.

The dictionary reuses part of the Constructed Pixel Buffer, so no additional memory space is required. To accelerate the matching string search, a hash-table based approach is used. The hash-table requires additional memory space. More memories are used, better R-D performance can be achieved. It is very critical to completely understand the impact of hash-table memory space size on R-D performance of DMC coding.

# BD-rate comparison

A total of 11 pictures, 6 from [1] and 5 from [2], are used in the BD-rate comparison. The following seven different levels of dictionary and hash-table memory size configurations are used.

| Dictionary/hash-table level | Dictionary size (KB) share memory with hybrid coder | Hash-table size (KB) additional memory required | Hash value bits |
| --- | --- | --- | --- |
| 1 | 4 | 10.5 | 8 |
| 2 | 8 | 19 | 9 |
| 3 | 16 | 36 | 10 |
| 4 | 64 | 138 | 12 |
| 5 | 256 | 1,092 | 14 |
| 6 | 1,024 | 4,356 | 16 |
| 7 | 4,096 | 16,644 | 16 |

Using HEVC YUV 420 hybrid coding (HM4.0) as the reference, the average BD-rate summary of level 1-7 DMC coding is as follows.

| Dictionary/hash-table level | BD-rate (cubic) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 1 | -40.4% | -90.2% | -88.6% |
| 2 | -43.3% | -90.2% | -88.9% |
| 3 | -45.4% | -90.3% | -88.9% |
| 4 | -48.3% | -90.6% | -89.2% |
| 5 | -52.6% | -90.5% | -89.5% |
| 6 | -54.9% | -90.9% | -89.7% |
| 7 | -55.5% | -90.9% | -89.7% |
| Enc Time | 132% | Dec Time | 86% |

# Conclusions

BD-rate comparison between DMC coding and HEVC YUV 420 hybrid coding of 11 test screen content pictures shows an average 40% to 55% Y BD-rate and even bigger UV BD-rate reduction depending on the additional hash-table memory used. When the additional hash-table memory size is 10.5KB, the Y BD-rate reduction is 40%. When the additional hash-table memory size is 16MB, the Y BD-rate reduction is 55%.

# References

- Tao Lin, Peijun Zhang, Shuhui Wang, and Kailun Zhou, “4:4:4 Screen Content Coding using Dual-coder Mixed Chroma-sampling-rate (DMC) Techniques,” JCT-VC Doc JCTVC-I0272, May 2012.

- Peijun Zhang, Tao Lin, Shuhui Wang, and Kailun Zhou, “Subjective Quality Comparison between DMC Coding and Chroma-subsampled 420 Coding Using YUV444 Screen Content Test Sequences,” JCT-VC Doc JCTVC-H0065, May 2012.

- Tao Lin and Shuhui Wang, “Mixed Chroma Sampling-rate coding: combining the merits of 4:4:4 and 4:2:0 and increasing the value of past 4:2:0 investment,” JCT-VC Doc JCTVC-H0065, Feb. 2012.

- Shuhui Wang and Tao Lin, “4:4:4 screen content coding using Macroblock-Adaptive Mixed Chroma-Sampling-Rate,” JCT-VC Doc JCTVC-H0073, Feb. 2012.

- J. Cao, K. Zhou, S. Wang, T. Lin, “YUV4:4:4 test sequences for screen content coding,” JCT-VC Doc JCTVC-H0294, Feb. 2012.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).