| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0353 |
| --- | --- |

| Title: | R-D cost based effectiveness analysis of Dual-coder Mixed Chroma-sampling-rate (DMC) coding for 4:4:4 screen content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xianyi Chen Tao Lin Peijun Zhang Shuhui Wang Kailun Zhou | Tel: Email: | 86-21-5103-0961 lintao@tongji.edu.cn |
| Source: | Tongji University, Shanghai, China |  |  |

_____________________________

# Abstract

This contribution presents R-D cost based effectiveness analysis of Dual-coder Mixed Chroma-sampling-rate (DMC) coding for full-chroma (YUV444) screen content. The DMC coding technique codes a CU using a dictionary-entropy coder and a hybrid coder simultaneously and calculating two R-D costs Jdict and Jhybrid. The coder with smaller R-D cost is selected as the optimal coder to code the CU. For a given screen picture, to look at and understand the overall coder selection distribution across all CUs in the picture, a ratio Jhybrid/Jdict map can be plotted to visualize the coder selection distribution and to evaluate how effective the two coders are. The ratio distribution maps reveal that the two coders are indeed quite complementary and play very different roles to compress effectively different contents.

# Introduction

The DMC coding [1] - [5] architecture is shown in Error: Reference source not found

Decoder 1: discontinuous-tone content oriented luma and full-chroma dictionary-entropy decoding tool chain

Coder 1: discontinuous-tone content oriented luma and full-chroma dictionary-entropy encoding tool chain

Decoder 2: continuous-tone content oriented luma & mixed chroma-sampling rate hybrid decoding tool chain

Coder 2: continuous-tone content oriented luma & mixed chroma-sampling rate hybrid encoding tool chain

Pred

Trans/Qu/Entropy encode & Pixel reconstruction

Input CU

R-D Cost Based Selector

DMC Bitstream

Constru-cted CU

MB Coding Type Parser

DMC Bitstream

UV

Pred

Inv.Tran

Entropy Decode

DeQu

UV

(a) Encoder

(b) Decoder

Entropy encoding

String search & matching

Entropy decoding

String construction

Chroma-subsampling

Chroma-upsampling

- Dual-coder Mixed Chroma-sampling-rate coding architecture.

A dictionary-entropy coder and a hybrid coder code a CU simultaneously and calculate two R-D costs Jdict and Jhybrid. The coder with smaller R-D cost is selected as the optimal coder to code the CU.

For a given screen picture, to look at and understand the overall coder selection distribution across all CUs in the picture, a ratio Jhybrid/Jdict map can be plotted to visualize the coder selection distribution and to evaluate how effective the two coders are.

# R-D cost based effectiveness analysis of DMC

Fig. 2 shows a typical screen picture of webpage and its coder selection distribution map using the DMC technique with QP=28 for the hybrid coder. In the map, green square means that Jhybrid/Jdict = 1 for the CU (16x16 pixels); red square means that Jhybrid/Jdict > 1 for the CU; blue square means that Jhybrid/Jdict < 1 for the CU. For the red and blue squares, brighter color indicates bigger difference between the two R-D cost values, and the brighest color indicates the difference is more than four time (Jhybrid/Jdict ≥ 4 for red square or Jdict/Jhybrid ≥ 4 for blue square). In the map of Fig. 2, 47.97% CUs are coded by the dictionary-entropy coder. From the map, we can find that 1) two types of screen contents are consistently mapped to two colors; and 2) a majority (84% in the example) of the squares have the brighest color, which means that one coder performs at least 400% better than the other coder in terms of R-D cost.

This finding reveals that the two coders are indeed quite complementary and play very different roles to compress effectively different contents. When one coder cannot compress a CU well, the other coder has very high (84% in the example) probability to code the CU at least 4 times better. Therefore, unifying two coders into a single coding framework can improve SCC performance in a big and significant way.

To further confirm the above finding in a larger scale, we have calculated 375,696 pairs of (Jhybrid, Jdict) from six screen capture pictures shown in Fig. 3 and five screen capture video sequences (1st frame) shown in Fig. 4. For each CU, six quantization parameters (qp) of 20, 24, 28, 32, 36, 40 are used in the hybrid coder to calculate six pairs of (Jhybrid, Jdict). Table I shows the percentages of 375,696 ratio Jhybrid/Jdict in 9 intervals: (0, 1/4], (1/4, 1/3], (1/3, 1/2], (1/2, 1), [1, 1], (1, 2), [2, 3), [3, 4), [4, ∞). The first interval is for the case that Jdict is at least 4 times larger than Jhybrid. The second interval is for the case that Jdict is between 3 and 4 times of Jhybrid. The fifth interval is for the case that Jdict is equal to Jhybrid. The eighth interval is for the case that Jhybrid is between 3 and 4 times of Jdict. The last (ninth) interval is for the case that Jhybrid is at least 4 times larger than Jdict.

- Percentages of 375,696 ratio Jhybrid/Jdict in 9 intervals

|  | Jhybrid/Jdict < 1: hybrid coder is selected |  |  |  | Jhybrid/Jdict ≥ 1: dictionary-entropy coder is selected |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Jhybrid/Jdict | (0, 1/4] | (1/4, 1/3] | (1/3, 1/2] | (1/2, 1) | [1, 1] | (1, 2) | [2, 3) | [3, 4) | [4,∞) |
| Percentage | 47.66% | 7.24% | 4.30% | 2.74% | 0.48% | 6.53% | 4.65% | 3.05% | 23.35% |
|  | 61.94% |  |  |  | 38.06% |  |  |  |  |

Table I does confirm that overall more than 38% CUs are coded by the dictionary-entropy coder and when one coder cannot compress a CU well, the other coder has very high (70% overall) probability to code the CU at least 4 times better.

- A typical screen picture of webpage and its coder selection distribution map.

(b) IC design debug waveform plus nature picture.

(a) Integrated-circuit design debug waveform.

(c) Web page.

(d) PowerPoint, Word, and Excel file windows.

(e) Document editing.

(f) Various windows mix.

- Six screen capture pictures, (a), (b), (c): 704x448, (d): 1920x1200, (e): 1280x800, (f): 1280x1024.

(a) CAD_waveform

(b) CAD_waveform_Street_Campus

(c) PCB_layout

(e) ppt_doc_xls

(d) PCB_schematic

- First frame of five screen capture video sequences, 1920 × 1080p.

# Conclusions

DMC coding is a very effective technique to compress screen pictures that contain both discontinuous-tone content and continuous-tone content. The two coders are really quite complementary and play very different roles to compress effectively different contents.

# References

- Tao Lin, Peijun Zhang, Shuhui Wang, and Kailun Zhou, “4:4:4 Screen Content Coding using Dual-coder Mixed Chroma-sampling-rate (DMC) Techniques,” JCT-VC Doc JCTVC-I0272, May 2012.

- Peijun Zhang, Tao Lin, Shuhui Wang, and Kailun Zhou, “Subjective Quality Comparison between DMC Coding and Chroma-subsampled 420 Coding Using YUV444 Screen Content Test Sequences,” JCT-VC Doc JCTVC-H0065, May 2012.

- Tao Lin and Shuhui Wang, “Mixed Chroma Sampling-rate coding: combining the merits of 4:4:4 and 4:2:0 and increasing the value of past 4:2:0 investment,” JCT-VC Doc JCTVC-H0065, Feb. 2012.

- Shuhui Wang and Tao Lin, “4:4:4 screen content coding using Macroblock-Adaptive Mixed Chroma-Sampling-Rate,” JCT-VC Doc JCTVC-H0073, Feb. 2012.

- J. Cao, K. Zhou, S. Wang, T. Lin, “YUV4:4:4 test sequences for screen content coding,” JCT-VC Doc JCTVC-H0294, Feb. 2012.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).