| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0354 |
| --- | --- |

| Title: | Modification of context assignment for significance flag coding for large TUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan L Joshi Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com +1-858-845-2429 joels@qualcomm.com +1-858-658-5673 martak@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A modification of context assignment for significance flag coding for large TUs is proposed. It is claimed that the proposed modification simplifies context derivation logic. For common test conditions and AI-main, RA-main, LB-main, AI-HE10, RA-HE10, and LB-HE10 configurations the average BD-rate difference is −0.10%, −0.06%, −0.01%, −0.09%, −0.05%, and 0.04%, respectively (average: −0.045%). Two additional contexts for luma and one for chroma are reportedly used.

# Introduction

In the 9th JVC meeting, position-based context derivation (JCTVC-I0296) was adopted for significance flag coding for 16×16 and 32×32 TUs. It had a BD-rate difference of 0.1% while reducing the number of contexts used for significance flag coding by 3. In this proposal we explore bridging the performance gap while using 3 extra contexts (2 luma, 1 chroma).

In HEVC WD7, for 16×16 and 32×32 blocks, the contexts for coding significance flags are assigned based on the position of the coefficient within the 4×4 subblock, whether the 4×4 subblock is a DC subblock and the coefficient group flags for the right (CGFR) and bottom (CGFB) 4×4 subblocks. Figure 1 shows the context assignment table:

| CGFB=0, CGFR=0. 1; 1; 1; 0. 1; 1; 0; 0. 1; 0; 0; 0. 0; 0; 0; 0. | CGFB=0, CGFR=1. 1; 1; 1; 1. 1; 1; 1; 1. 0; 0; 0; 0. 0; 0; 0; 0. |
| --- | --- |
| CGFB=1, CGFR=0. 1; 1; 0; 0. 1; 1; 0; 0. 1; 1; 0; 0. 1; 1; 0; 0. | CGFB=1, CGFR=1. 2; 2; 2; 2. 2; 2; 2; 2. 2; 2; 2; 1. 2; 2; 1; 1. |

Figure 1: Context assignment in HEVC WD7

# Proposal

We propose a modification of the context assignment table as shown in Fig. 2.

| CGFB=0, CGFR=0. 1; 1; 1; 2. 1; 1; 2; 3. 1; 2; 3; 3. 2; 3; 3; 3. | CGFB=0, CGFR=1. 0; 0; 0; 0. 1; 1; 1; 1. 2; 2; 2; 2. 3; 3; 3; 3. |
| --- | --- |
| CGFB=1, CGFR=0. 0; 1; 2; 3. 0; 1; 2; 3. 0; 1; 2; 3. 0; 1; 2; 3. | CGFB=1, CGFR=1. 0; 0; 0; 0. 0; 0; 0; 0. 0; 0; 0; 0. 0; 0; 0; 0. |

Figure 2: Proposed context assignment

When CGFR or CGFB is non-zero for a subblock no additional operations (such as comparisons) are needed for context assignment. When both CGFR and CGFB are 0, an additional comparison is needed in some cases. This represents a slight simplification of the context derivation process. The proposed method uses 8 contexts for luma and 4 contexts for chroma for significance flag coding of 16×16 and 32×32 blocks. Thus 3 additional contexts are used compared to HEVC WD7. New initialization values were used for 12 contexts (8 luma and 4 chroma).

# Results

The proposed method is implemented in HM7.0 and simulated under common test conditions. Table 1 shows a summary of the BD-rate results for AI-main, RA-main, LB-main, AI-HE10, RA-HE10, and LB-HE10 configurations. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding as well as decoding were done on a LINUX cluster (64 bit) by specifying only one type of CPU.

Table 1: BD-rate results for the proposed modification of context assignment for coding significance maps for 16×16 and 32×32 TUs.

# Conclusions

A modification of context assignment for significance flag coding for large TUs is proposed. This results in slight simplification of context derivation and also provides small improvement in BD-rate. For common test conditions and AI-main, RA-main, LB-main, AI-HE10, RA-HE10, and LB-HE10 configurations the average BD-rate difference is −0.10%, −0.06%, −0.01%, −0.09%, −0.05%, and 0.04%, respectively (average: −0.045%). Two additional contexts for luma and one for chroma are used.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).