| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0355-r2 |
| --- | --- |

| Title: | Coding of SAO merge left and merge up flags |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo and David Baylon 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 kminoo@motorola.com david.baylon@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

Based on WD7 of HEVC, the coding of the SAO merge left and merge up is performed for each color component.  This contribution proposes to use a single merge left flag to signal whether all color components are merge left, and to use a single merge up flag to signal whether all color components are merge up.  Otherwise, SAO offset parameters are signaled for each color component. Compared to WD7, this method reduces the number of contexts for coding of the merge-left flag from 3 contexts to 1 context. It also reduces the number of bins for signaling the merge-left/up from 3 bin to 1 bin. Experimental results show  reduction of context-entropy-coded bins for all SAO syntaxes by a factor of 48% and 61% for main and he10 coding conditions, respectively, while the coding efficiency has been improved by 0.2%.

- Introduction

Based on the current working draft of HEVC [1], the SAO parameters for an LCU can be inferred from a left neighbor (“merge left”), an above neighbor (“merge up”), or explicitly signaled (no merge).  In WD7, the merge decision is first signaled on a color component basis.  For a given color component it is first signaled whether the SAO parameters for the current LCU will be obtained by merge left (from the left neighbor, assuming the left neighbor exists).  If merge left is not chosen, merge up is checked (assuming the above neighbor exists).  If merge up is not chosen, then the parameters are explicitly signaled. These parameters include the SAO type, which can be one of three states of Skip, Edge or Band. If the SAO type is Edge or Band, then a corresponding number of offset values would be transmitted.

- Coding of merge left and merge up across color components

In [2], it was proposed that instead of signaling merge left for each component separately and independently, it is desirable that the merge left flag is first signaled (if the left LCU exists and it is allowed to be used) to indicate whether all color components are inferred from SAO parameter from left LCU neighbor (merge left).  If so, all color components will use merge left mode.  If not, then merge up mode is checked by a single flag for all color components (as explained above for merge left).  If merge up mode is not chosen, or if it does not exist then explicit SAO parameters are signaled for each color component.

While the reduction of coded bins provides a coding efficiency of about 0.2% for average Luma BDR (refer to Table 2), the proposed scheme also offers a reduction in the complexity of the CABAC engine by reducing the number context-coded bins. Note that for each merge/left-up syntax, 3 bins are context-entropy-coded in WD7 (for each color), whereas in this proposal, only 1 bin is coded (to indicate the same flag for all three colors).

Table 1, which contains AHG5 metrics for this proposal, shows that a 66% reduction in the number of bins for the SAO merge flags translates to about a 48% and 61% reduction in the number context-coded bins for all SAO syntax elements (these numbers are for he10 and main coding conditions, respectively).

Table 1 Summary of simulation results (Entropy Coded bins), HM 70 anchor

| (Informative) Bins relative to total bins |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SAO |  |  |  |  |
|  | Bins relative to total bins |  |  |  | Delta rates % |
|  | Reference |  | Tested |  |  |
|  | ctx | bp | ctx | bp |  |
| AI Main | 1.06% | 0.14% | 0.76% | 0.14% | -5.55% |
| RA Main | 3.57% | 0.18% | 1.21% | 0.17% | -20.25% |
| LB Main | 3.76% | 0.36% | 1.28% | 0.33% | -18.78% |
| LP Main | 4.10% | 0.23% | 1.53% | 0.22% | -19.98% |
| Average | 3.12% | 0.23% | 1.19% | 0.21% | -16.14% |
|  | SAO |  |  |  |  |
|  | Bins relative to total bins |  |  |  | Delta rates % |
|  | Reference |  | Tested |  |  |
|  | ctx | bp | ctx | bp |  |
| AI HE10 | 0.80% | 0.05% | 0.43% | 0.05% | -12.43% |
| RA HE10 | 2.50% | 0.06% | 0.66% | 0.05% | -28.56% |
| LB HE10 | 2.96% | 0.08% | 0.68% | 0.07% | -31.51% |
| LP HE10 | 3.03% | 0.06% | 0.76% | 0.06% | -30.28% |
| Average | 2.32% | 0.06% | 0.63% | 0.06% | -25.69% |
| (Informative) Bins relative to reference |  |  |  |  |  |
|  | Total |  | SAO |  |  |
|  | ctx | bp | ctx | bp |  |
| AI Main | 99.70% | 100.00% | 78.63% | 102.33% |  |
| RA Main | 97.60% | 99.98% | 38.75% | 95.30% |  |
| LB Main | 97.49% | 99.97% | 43.35% | 93.04% |  |
| LP Main | 97.34% | 99.86% | 47.91% | 95.66% |  |
| Average | 98.03% | 99.95% | 52.16% | 96.58% |  |
|  | Total |  | SAO |  |  |
|  | ctx | bp | ctx | bp |  |
| AI HE10 | 99.64% | 100.00% | 61.73% | 90.93% |  |
| RA HE10 | 98.15% | 99.99% | 29.15% | 88.91% |  |
| LB HE10 | 97.68% | 100.02% | 31.42% | 88.28% |  |
| LP HE10 | 97.72% | 99.99% | 34.28% | #DIV/0! |  |
| Average | 98.30% | 100.00% | 39.14% | #DIV/0! |  |

# Proposed text changes

### 7.3.4  Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ][0] ) |  |
| sao_param( xCtb >> Log2CtbSize, yCtb >> Log2CtbSize, cIdx ) |  |
| } |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| if( slice_alf_flag[ cIdx ] ) |  |
| ctb_alf_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| CtbAddrRS = CtbAddrTStoRS[ CtbAddrTS ] |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) |  |
| rbsp_alignment( ) |  |
| } while( moreDataFlag ) |  |
| } |  |

#### 7.3.4.1  Sample adaptive offset parameter syntax

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Table 9-1 – Values of variable initValue for sao_merge_left_flag ctxIdx

| Initialization variable | sao_merge_left_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 118 | 118 | 118 | 102 | 102 | 102 | 102 | 102 | 102 |

| Table 9-2 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| sao_merge_left_flag | 0 | FL, cMax = 1 | 0 | Table 9-1 | 0 |
|  | 1 |  | 0 | Table 9-1 | 13 |
|  | 2 |  | 0 | Table 9-1 | 26 |

| Table 9-3 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| sao_merge_left_flag | Table 9-1 | 0 | cIdx | na | na | na | na |
|  |  | 31 | cIdx | na | na | na | na |
|  |  | 62 | cIdx | na | na | na | na |

# Simulation results

Table 1 shows the results of the proposed method compared to HM7.0 [3], for an LCU size of 64, based on the test condition in [3].

Table   Summary of simulation results (BD-rate), HM 70 anchor (d

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.3% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.3% | 0.3% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.3% | 0.2% | 0.0% | 0.4% | 0.4% |
| Overall | 0.0% | 0.2% | 0.1% | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 112% |  |  | 136% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.1% | -0.1% | 0.3% | 0.3% |
| Class B | -0.1% | -0.1% | -0.2% | -0.1% | 0.1% | 0.1% |
| Class C | -0.2% | -0.1% | -0.2% | -0.2% | 0.0% | 0.1% |
| Class D | -0.4% | -0.2% | -0.2% | -0.3% | -0.1% | -0.2% |
| Class E |  |  |  |  |  |  |
| Class F | -0.2% | 0.1% | 0.0% | -0.2% | 0.1% | 0.2% |
| Overall | -0.2% | -0.1% | -0.1% | -0.2% | 0.1% | 0.1% |
|  | -0.2% | -0.1% | -0.1% | -0.2% | 0.1% | 0.1% |
| Enc Time[%] | 83% |  |  | 84% |  |  |
| Dec Time[%] | 157% |  |  | 128% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.2% | -0.5% | -0.1% | 0.1% | 0.2% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.3% |
| Class D | -0.3% | -0.3% | 0.8% | -0.3% | 0.5% | 0.2% |
| Class E | -0.3% | -1.3% | -1.7% | -0.3% | -1.2% | -0.6% |
| Class F | -0.4% | -0.7% | -0.3% | -0.1% | 0.5% | 0.5% |
| Overall | -0.2% | -0.5% | -0.3% | -0.2% | 0.0% | 0.2% |
|  | -0.2% | -0.5% | -0.3% | -0.2% | 0.1% | 0.3% |
| Enc Time[%] | 81% |  |  | 81% |  |  |
| Dec Time[%] | 139% |  |  | 135% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.4% | -0.6% | -0.2% | 0.0% | -0.2% |
| Class C | -0.2% | -0.3% | -0.3% | -0.1% | 0.0% | 0.3% |
| Class D | -0.3% | -0.1% | 0.4% | -0.3% | 0.0% | -0.5% |
| Class E | -0.5% | -2.1% | -2.0% | -0.4% | -0.4% | -0.1% |
| Class F | -0.5% | -0.8% | -0.4% | -0.2% | 0.2% | 0.1% |
| Overall | -0.4% | -0.6% | -0.5% | -0.2% | 0.0% | -0.1% |
|  | -0.4% | -0.6% | -0.5% | -0.2% | 0.1% | 0.0% |
| Enc Time[%] | 88% |  |  | 90% |  |  |
| Dec Time[%] | 98% |  |  | 103% |  |  |

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm. T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7”, JCTVC-I1003, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 9th meeting, Geneva, April 2012.

- K. Minoo, D. Baylon, I. S. Chong, M. Karczewicz, “Non-Ce1: Coding of SAO merge left and merge up” JCT-VC document JCTVC-I0507, Geneva, April 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-I1100, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 9th meeting, Geneva, April 2012.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).