| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0357 |
| --- | --- |

| Title: | AHG12: 4:2:2/4:4:4 chroma format extension for HEVC Version 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories) |  |  |

_____________________________

# Abstract

This contribution proposes an extension policy for supporting 4:2:2/4:4:4 chroma formats that is part of HEVC Version 2. The support is achieved by the minimum change from the current specification. The modified HM7.0 software, in which all video coding tools support extended chroma formats, is provided.

# Introduction

In the joint meeting of MPEG and VCEG at the previous meeting, it was decided that JCT-VC would start to standardize the new profile that supports the higher bit depth and color sampling schemes beyond 4:2:0 [1]. Supporting the 4:2:2 chroma format is a critical issue of video material transmission fields. In this contribution, a policy to handle the 4:2:2/4:4:4 chroma formats for HEVC is proposed. Based on the proposed policy, such extension can be realized by the minimum modification of coding tools under the TU layer. It is noted that the policy in this contribution is based on JCTVC-I0521 [2].

# 4:2:2 chroma format support in HEVC

The modification policy for the current working draft 7 is listed.

When chroma_format_idc is equal to 2, which indicates the 4:2:2 chroma format, the following modifications are applied.

- The height of a TU is double the width of the TU. The rectangle unit is not padded with square DCTs like the AVC, but is applied through horizontal and vertical DCT with sample length similar to the NSQT. Consequently, normalization factors are the multiplication of 11585/8192 at a quantization and of 11585/16384 at a de-quantization.

- No syntax is changed except the number of coefficients is doubled in TU. For example, most of the loop is initialized by n = 31 and some statements become firstNQPosInCG = 32 in the residual coding syntax.

- The number of applied pixels is simply increased in the process of deblocking filter, SAO filter, and ALF.

- Intra prediction is simply extended in order to support Nx2N size.

- Coefficient scanning is identical to HM7.0. It is noted that the diagonal scan is not changed.

- Vertical MC interpolation of chroma utilizes an 8-tap filter that is identical to that of luma.

- Context index maps are vertically extended in the coefficient coding.

# 4:4:4 chroma format support in HEVC

The modification policy for the current working draft 7 is listed.

When chroma_format_idc is equal to 3, which indicates the 4:4:4 chroma format, the following modifications are applied.

- The minimum TU size of chroma is identical to that of luma.

- No syntax is changed except that the TU size of chroma is identical to that of luma. For example, ( log2TrafoSize > 2) condition is eliminated from the transform tree syntax and the transform unit syntax.

- Adapting the rate-distortion optimization process to RGB materials, the intra RDO process optimizes RGB components simultaneously, which is similar to the inter process.

# Experimental Results

## 4:2:2 chroma format

Comparison results of the BD-rate are shown in the following tables. The anchor method is AVC High Profile (JM18.3).

As for the test materials, four sequences at YCbCr 4:2:2, 10-bit were selected from ITU-R Recommendation BT-1210-4 [3]. The video format is 1080p at the frame-rate of 60fps. The coding condition followed the main profile and the high efficiency with 10-bit conditions, and the coding structure was selected from all intra, random access and low delay (B) [4].

We consider the comparison between the results of the 4:2:2 chroma format and that of the 4:2:0 chroma format, which were previously reported [5]. The BD-rate is mostly similar between two chroma formats. In the HE10 condition, chroma coding efficiency is improved by using the LM mode. Enc/Dec time is increased due to the impact of the number of processing pixels.

Table 1 Results of the all intra / main profile and the HE10 conditions.

|  | All Intra Main |  |  |  | All Intra HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| s209 | -20.2% | -6.3% | -28.0% | -21.2% | -21.1% | -6.9% | -30.1% | -22.3% |
| s210 | -20.9% | -20.8% | -23.2% | -21.6% | -25.1% | -27.8% | -28.8% | -26.7% |
| s214 | -27.7% | -28.5% | -26.1% | -27.7% | -31.4% | -36.9% | -31.4% | -32.7% |
| s215 | -19.3% | -15.4% | -18.2% | -18.6% | -22.5% | -18.6% | -21.4% | -21.8% |
| Overall | -22.0% | -17.8% | -23.9% | -22.3% | -25.0% | -22.6% | -27.9% | -25.9% |
| Enc Time[%] | 81.4% |  |  |  | 117.7% |  |  |  |
| Dec Time[%] | 230.5% |  |  |  | 234.5% |  |  |  |

Table 2 Results of the random access / main profile and HE10 conditions.

|  | Random Access Main |  |  |  | Random Access HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| s209 | -28.8% | -36.0% | -35.5% | -30.2% | -30.8% | -6.7% | -33.2% | -30.6% |
| s210 | -35.8% | -35.5% | -40.1% | -37.0% | -38.1% | -37.4% | -42.8% | -39.3% |
| s214 | -39.8% | -38.8% | -37.3% | -39.2% | -39.4% | -37.9% | -35.6% | -38.6% |
| s215 | -41.5% | -45.5% | -44.0% | -43.1% | -41.5% | -47.2% | -45.8% | -43.9% |
| Overall | -36.5% | -38.9% | -39.2% | -37.4% | -37.5% | -32.3% | -39.3% | -38.1% |
|  | -36.4% | -39.1% | -39.4% | -37.4% | -37.3% | -32.2% | -39.2% | -38.0% |
| Enc Time[%] | 121.0% |  |  |  | 126.1% |  |  |  |
| Dec Time[%] | 206.7% |  |  |  | 251.3% |  |  |  |

Table 3 Results of the low delay B / main profile and HE10 conditions.

|  | Low Delay B Main |  |  |  | Low Delay B HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| s209 | -24.0% | -30.9% | -32.9% | -26.2% | -24.2% | -23.4% | -33.2% | -26.0% |
| s210 | -38.0% | -39.1% | -40.0% | -38.8% | -39.0% | -40.2% | -40.9% | -39.8% |
| s214 | -47.0% | -51.9% | -50.3% | -48.7% | -46.9% | -50.9% | -50.2% | -48.4% |
| s215 | -49.9% | -47.4% | -48.6% | -49.4% | -49.1% | -45.2% | -47.7% | -48.3% |
| Overall | -39.7% | -42.3% | -42.9% | -40.8% | -39.8% | -39.9% | -43.0% | -40.7% |
|  | -39.9% | -42.1% | -43.3% | -40.9% | -39.9% | -39.8% | -43.4% | -40.8% |
| Enc Time[%] | 177.6% |  |  |  | 184.2% |  |  |  |
| Dec Time[%] | 207.4% |  |  |  | 244.6% |  |  |  |

## 4:4:4 chroma format

Comparison results of the BD-rate are shown in the followings. The anchor method is AVC High Profile (JM18.3).

As for the test materials, three sequences at RGB 4:4:4, 8-bit were selected from the VQEG ftp site [6]. The video format is 1080p. "Traffic" is the frame-rate of 30fps, and "CrowdRun" and "ParkJoy" are the frame-rate of 50fps. Materials of the YCbCr 4:4:4 chroma format are converted from that of the RGB 4:4:4 chroma format. The coding condition followed the main profile and the high efficiency with 10-bit conditions, and the coding structure was selected from all intra, random access, and low delay (B) [4].

At the AI condition, the coding efficiency in HE10 is significantly higher than that of Main condition. From the result of low-delay conditions, the difference of an inter-coding efficiency in the two coding conditions is small. It is difficult to investigate the comparison between the color spaces of YUV and RGB due to the small number of sequences. Since Dec time for JM18.3 of the 4:4:4 chroma format is longer than that of the 4:2:2 chroma format, Dec time for the modified HM7.0 of the 4:4:4 chroma format is relatively faster than that of the 4:2:2 chroma format.

Table 4 Results of the all intra / main profile and HE10 conditions.

|  | All Intra Main |  |  |  | All Intra HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Traffic | -10.6% | -12.8% | -14.4% | -12.8% | -34.2% | -30.9% | -34.2% | -33.3% |
| CrowdRun | -5.0% | -7.7% | -7.6% | -7.0% | -20.9% | -20.3% | -20.3% | -20.7% |
| ParkJoy | -5.1% | -7.4% | -7.4% | -6.8% | -18.9% | -18.7% | -19.2% | -19.1% |
| Traffic | -14.1% | -19.3% | -21.7% | -17.8% | -19.1% | -26.4% | -28.6% | -23.9% |
| CrowdRun | -8.0% | -13.4% | -15.8% | -11.9% | -12.7% | -18.1% | -20.4% | -16.6% |
| ParkJoy | -6.8% | -12.0% | -15.7% | -11.1% | -14.6% | -18.4% | -23.3% | -18.4% |
| GBR | -6.9% | -9.3% | -9.8% | -8.9% | -24.6% | -23.3% | -24.6% | -24.3% |
| YUV444 | -9.7% | -14.9% | -17.7% | -13.6% | -15.4% | -20.9% | -24.1% | -19.7% |
| Overall | -8.3% | -12.1% | -13.8% | -11.3% | -20.0% | -22.1% | -24.4% | -22.0% |
| Enc Time[%] | 87.6% |  |  |  | 182.5% |  |  |  |
| Dec Time[%] | 207.5% |  |  |  | 238.3% |  |  |  |

Table 5 Results of the random access / main profile and HE10 conditions.

|  | Random Access Main |  |  |  | Random Access HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Traffic | -13.0% | -22.5% | -20.7% | -18.6% | -26.8% | -32.2% | -33.7% | -30.9% |
| CrowdRun | -1.3% | -14.4% | -13.9% | -10.5% | -12.2% | -21.0% | -22.1% | -18.9% |
| ParkJoy | -2.0% | -14.2% | -13.1% | -9.9% | -10.8% | -20.3% | -20.7% | -17.4% |
| Traffic | -24.6% | -32.3% | -33.7% | -28.9% | -27.8% | -41.5% | -41.4% | -34.7% |
| CrowdRun | -13.2% | -28.4% | -30.6% | -21.3% | -16.8% | -33.8% | -35.5% | -25.7% |
| ParkJoy | -8.9% | -22.0% | -32.5% | -18.3% | -14.2% | -28.5% | -38.5% | -24.2% |
| GBR | -5.4% | -17.0% | -15.9% | -13.0% | -16.6% | -24.5% | -25.5% | -22.4% |
| YUV444 | -15.6% | -27.6% | -32.2% | -22.8% | -19.6% | -34.6% | -38.5% | -28.2% |
| Overall | -10.5% | -22.3% | -24.1% | -17.9% | -18.1% | -29.5% | -32.0% | -25.3% |
| Enc Time[%] | 134.4% |  |  |  | 153.1% |  |  |  |
| Dec Time[%] | 171.7% |  |  |  | 203.9% |  |  |  |

Table 6 Results of the low delay B / main profile and HE10 condition.

|  | Low Delay B Main |  |  |  | Low Delay B HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Traffic | -36.9% | -39.9% | -44.8% | -40.8% | -42.1% | -42.6% | -49.4% | -44.9% |
| CrowdRun | -12.5% | -15.4% | -16.3% | -14.7% | -16.9% | -18.8% | -20.2% | -18.7% |
| ParkJoy | -11.8% | -14.9% | -15.3% | -13.8% | -15.2% | -17.5% | -18.9% | -17.1% |
| Traffic | -39.9% | -53.8% | -61.9% | -50.6% | -42.3% | -58.9% | -66.2% | -54.5% |
| CrowdRun | -15.2% | -28.9% | -35.3% | -25.1% | -18.3% | -31.6% | -38.2% | -28.1% |
| ParkJoy | -14.4% | -21.7% | -33.7% | -22.4% | -17.1% | -24.7% | -35.7% | -25.2% |
| GBR | -20.4% | -23.4% | -25.4% | -23.1% | -24.7% | -26.3% | -29.5% | -26.9% |
| YUV444 | -23.2% | -34.8% | -43.6% | -32.7% | -25.9% | -38.4% | -46.7% | -35.9% |
| Overall | -21.8% | -29.1% | -34.5% | -27.9% | -25.3% | -32.3% | -38.1% | -31.4% |
| Enc Time[%] | 160.2% |  |  |  | 181.7% |  |  |  |
| Dec Time[%] | 152.4% |  |  |  | 184.0% |  |  |  |

# Conclusion

In this contribution, we proposed the modification policy of handling the 4:2:2/4:4:4 chroma formats for the working draft 7. We recommend establishing a core experiment to investigate the support of 4:2:2/4:4:4 chroma formats, starting from the attached software.

# References

- ISO/IEC JTC1/SC29/WG11 N12742, "Responses to NBs on HEVC issues," May 2012.

- JCTVC-I0521, "AHG14: Color format extension for HEVC," May 2012.

- ITU-R Recommendation BT.1210-4, "Test materials to be used in assessment of picture quality," 01/2012. http://www.itu.int/rec/R-REC-BT.1210

- JCTVC-I1100, "Common test conditions and software reference configurations," May 2012.

- JCTVC-I0407, "Compression Performance of HEVC WD6 based on ITU-R Rec. BT.1210", May 2012.

- "The SVT High Definition Multi Format Test Set", ftp://vqeg.its.bldrdoc.gov/HDTV/
SVT_MultiFormat/1080p50_CgrLevels_SINC_FILTER_SVTdec05_/

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).