| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, Sweden, July 11-20, 2012 | Document: JCTVC-J0359 |
| --- | --- |

| Title: | Cross-check for JCTVC-J0133 TU Depth Clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xingyu Zhang Oscar C. Au | Tel: Email: | +852 2358 8529 eexyzhang@ust.hk |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This contribution presents results of cross-check for JCTVC-J0133, “TU Depth Clean-up”. The contribution studied two aspects of TU depth related issues. The first aspect is about limiting the value of maximum TU depths in SPS, while the second aspect is about deriving the context index for split_transform_flag. For each aspect, J0133 proposed a solution. This cross-check result confirms the result the second solution reported by the proponent.

# Introduction

JCTVC-0133 proposes a modified scheme to derive the context index of split_transform_flag. The proposed method is based on the transform size, instead of using the depth value as in HM7.0. Since the split_transform_flag is signaled only when the transform size is larger than 4x4, only 3 contexts are needed. In addition, the context number for split_transform_flag is reduced with the proposed solution.

# Test Settings and Conditions

The source code provided by the proponents is inspected with respect to the description in JCTVC-J0133. The source code was found to follow the description. The simulations of this document use HM7.0[1] software. The test conditions are All Intra Main, Random Access Main, and Low Delay B Main. All of these conditions follow the common text conditions in [2] expect the LCU size is set to 16x16. Both the anchor and proposed solution are running under this condition. The results match the ones reported in JCTVC-J0133. Please be advised the run time reported in the following tables might not be accurate.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class A | 0.0% | 1.7% | -0.3% |
| Class B | 0.0% | 0.1% | 0.3% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.2% |
| Class E |  |  |  |
| Overall | 0.0% | 0.4% | 0.1% |
|  | 0.0% | 0.4% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | -0.6% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.2% | 0.3% |
| Class E | 0.0% | 0.2% | -0.6% |
| Overall | 0.0% | 0.1% | -0.2% |
|  | 0.0% | 0.0% | -0.2% |
| Class F | 0.0% | -0.6% | 1.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# References

- HM7.0 Software available under the website: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-7.0

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-I1100, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012