| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0361 |
| --- | --- |

| Title: | Cross-check report of JCTVC-J0187 on temporal_mvp_enable_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Hui Yong Kim Jinho Lee 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-5772 {sclim, hykim5, jinosoul} @etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This document presents a cross-check report for "on temporal_mvp_enable_flag" (JCTVC-J0187) proposed by Sony. It is confirmed that all the R-D performance results are exactly matched with the results provided by the proponent.

# Cross-check results

Sony provided the source code and simulation result of the experiments 1 and 2 in JCTVC-J0187 [1]. Since the experiment 1 is HM7.0 with TMVP off, there is no need to check the source code. So, the source code check of experiment 2 was carried out by setting the macro definitions in the source code as in Table I.

Table I. Macro definition setting in the source code

//TMVP List 0/1 #define TMVP_LIST 1

The HM7.0 and draft text 7 have a small mismatch. That is, in the text, pic_temporal_mvp_enable_flag is signaled in slice header. But, in the HM7.0, pic_temporal_mvp_enable_flag is signaled in PPS. This mismatch was fixed in HM7.1. But, by the late release of HM7.1, the proponent implemented JCTVC-J0187 on HM7.0. Thus, the mismatch is existed in JCTVC-J0187 as well. That is, in the text of JCTVC-J0187, pic_temporal_mvp_enable_flag_L0 and pic_temporal_mvp_enable_flag_L1 are signaled in slice header. But, in the S/W of JCTVC-J0187, pic_temporal_mvp_enable_flag_L0 and pic_temporal_mvp_enable_flag_L1 are signaled in PPS. However, this mismatch occurs 1 bit difference in high level syntax and was caused by the late release of HM7.1, not by the proponent. It can be noted that the source code for experiment 2 is matched with the description in the JCTVC-J0187 document.

Using the anchor as HM7.0, simulations were conducted with running the encoder with option --TMVP=0 for experiment 1, --TMVP_L0=1 and TMVP_L1=0 for experiment 2 under four test conditions (MAIN-RA, HE10-RA, MAIN-LB, and HE10-LB) as specified in common test condition [2]. Cross-check results compared to the HM7.0 anchor for experiment 1 and 2 are summarized in Table II and Table III, respectively. All the R-D results are exactly matched with the results provided by the proponent.

Table II. Cross-check results of experiment 1 in JCTVC-J0187

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 2.6% | 2.1% | 2.4% | 2.7% | 2.1% | 2.4% |
| Class B | 2.4% | 1.6% | 1.7% | 2.3% | 1.5% | 1.5% |
| Class C | 2.4% | 2.2% | 2.4% | 2.4% | 2.2% | 2.2% |
| Class D | 2.8% | 2.2% | 2.4% | 2.8% | 2.2% | 2.1% |
| Class E |  |  |  |  |  |  |
| Overall | 2.5% | 2.0% | 2.2% | 2.5% | 2.0% | 2.0% |
|  | 2.5% | 2.0% | 2.2% | 2.5% | 2.0% | 2.0% |
| Class F | 1.0% | 1.2% | 1.1% | 1.1% | 1.2% | 1.2% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.7% | 1.7% | 1.6% | 2.5% | 1.6% | 1.7% |
| Class C | 2.8% | 2.6% | 2.9% | 2.8% | 2.8% | 2.9% |
| Class D | 2.5% | 1.8% | 2.5% | 2.5% | 2.2% | 2.3% |
| Class E | 2.2% | 2.1% | 1.8% | 2.4% | 2.2% | 2.4% |
| Overall | 2.6% | 2.0% | 2.2% | 2.6% | 2.2% | 2.3% |
|  | 2.6% | 2.0% | 2.2% | 2.5% | 2.2% | 2.3% |
| Class F | 2.3% | 2.4% | 2.6% | 1.9% | 2.0% | 3.2% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |

Table III. Cross-check results of experiment 2 in JCTVC-J0187

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.6% | 1.2% | 1.5% | 1.6% | 1.3% | 1.4% |
| Class B | 1.8% | 1.2% | 1.2% | 1.7% | 1.2% | 1.1% |
| Class C | 1.7% | 1.6% | 1.7% | 1.7% | 1.6% | 1.6% |
| Class D | 2.0% | 1.6% | 1.6% | 2.1% | 1.6% | 1.6% |
| Class E |  |  |  |  |  |  |
| Overall | 1.8% | 1.4% | 1.5% | 1.8% | 1.4% | 1.4% |
|  | 1.8% | 1.4% | 1.5% | 1.8% | 1.4% | 1.4% |
| Class F | 0.7% | 0.8% | 0.8% | 0.6% | 0.7% | 0.8% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4% | 0.1% | 0.3% | 0.3% | 0.2% | 0.3% |
| Class C | 0.3% | 0.4% | 0.3% | 0.3% | 0.3% | 0.3% |
| Class D | 0.2% | -0.1% | 0.7% | 0.2% | 0.6% | -0.1% |
| Class E | 0.2% | 0.1% | -0.3% | 0.4% | 0.1% | 0.1% |
| Overall | 0.3% | 0.1% | 0.3% | 0.3% | 0.3% | 0.2% |
|  | 0.3% | 0.1% | 0.3% | 0.3% | 0.3% | 0.2% |
| Class F | 0.0% | 0.0% | 0.1% | 0.2% | 0.3% | 1.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

# Conclusion

ETRI confirms that cross-check results are exactly matched with the results in JCTVC-J0187.

# Reference

- K. Sato, "On temporal_mvp_enable_flag," JCTVC-J0187, Jul. 2012.

- F. Bossen, "Common HM test conditions and software reference configurations," JCTVC-I1100, Apr. 2012.