| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0364 |
| --- | --- |

| Title: | Implicit transform block split process for asymmetric partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng (HiSilicon) | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution provides an implicit TU split solution for asymmetric partitions when “QuadtreeTUMaxDepthInter” is set to 1. Experimental results show that the proposed solutions can contribute average coding gain from 0.3% to 1.3% at Main profile configurations. Both encoder and decoder complexity are same as HM7.0.

# Introduction

AMP was adopted to Main profile at 9th JCT-VC meeting. It allows to partitions a coding unit into two prediction unit with asymmetric sizes. When "QuadtreeTUMaxDepthInter" is set to 1, the transform block is implicitly split into the blocks with half horizontal and vertical size at Main profile with the purpose of aligning transform blocks' boundaries with the prediction unit's boundaries. Since asymmetric partitions exist in AMP modes, asymmetric partitions might still cross transform blocks' boundary at the case of "QuadtreeTUMaxDepthInter = 1". Fig. 1 illustrates this implicit split process.

Fig. 1. Implicit transform block split process for asymmetric partitions (PU boundary is mark by red bold line, transform block’s boundary is marked by dash line)

The cross-boundary problem will cause impose negative impact on both coding performance and subjective quality. At the next sections, two implicit TU split solutions for asymmetric partitions is proposed, and its performance will be discussed.

# Implicit TU split for asymmetric partitions

## Solution 1: implicit non-square transform split for asymmetric partitions

Similar to NSQT concept, the 2Nx0.5N and 0.5Nx2N non-square transforms can be used in implicit transform split process for asymmetric partitions. Since these two non-square transforms contain "0.5N" boundary that is similar to asymmetric partitions, the transform blocks' boundary is aligned with asymmetric partitions' boundary. The split process is show at Fig. 2. To make the design consistent, non-square transforms are also applied to 2NxN and Nx2N partitions in the proposed solution.

Fig. 2. Implicit non-square transform split process for asymmetric partitions (PU boundary is mark by red bold line, transform block’s boundary is marked by dash line)

## Solution 2: implicit further transform split for asymmetric partitions

To avoid the cross-boundary problem for asymmetric partitions, further split process is proposed which means the transform unit is split into the transform units with quarter horizontal size and vertical size as the coding unit. Define a CU size as 2Nx2N. If "QuadtreeTUMaxDepthInter" is set to 1, NxN transform is applied to 2NxN and Nx2N partitions, and 0.5Nx0.5N transform is applied to AMP partitions. The split process is illustrated in Fig. 3. As show in Fig. 3 the further split process can also align transform blocks' boundary with prediction unit's boundary.

Fig. 3. Further implicit transform block split process for asymmetric PU (PU boundary is mark by red bold line, transform block’s boundary is marked by dash line)

# Experimentatl results

The proposed method is implemented on HM7.0. The coding efficiency is evaluated in RA-Main, and LDB-Main configurations. Considering that the proposed solution concentrates on implicit transform block split process, the parameter "QuadtreeTUMaxDepthInter" is set to 1 HM7.0 during the tests. Results of solution1 and solution2 are shown at Table 1 and Table 2 respectively. Solution1 can achieve average coding gain of 0.6% for RA-Main and 1.3% for LDB-Main. Solution2 can achieve average coding gain of 0.3% for RA-Main and 0.4% for LDB-Main. Both encoder and decoder time of the two solution are almost same as HM7.0.

Table 1: Solution1 (non-square transforms for asymmetric partitions)

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.4% | -0.7% | -0.6% |
| Class B | -0.6% | -1.7% | -1.5% |
| Class C | -0.7% | -0.9% | -0.6% |
| Class D | -0.6% | -0.5% | -0.6% |
| Class E |  |  |  |
| Overall | -0.6% | -1.0% | -0.9% |
|  | -0.5% | -1.0% | -0.9% |
| Class F | -0.4% | -0.5% | -0.5% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -1.2% | -4.2% | -3.9% |
| Class C | -1.2% | -2.3% | -2.2% |
| Class D | -1.1% | -2.2% | -1.0% |
| Class E | -1.7% | -3.7% | -2.5% |
| Overall | -1.3% | -3.1% | -2.5% |
|  | -1.3% | -3.0% | -2.5% |
| Class F | -1.2% | -1.3% | -1.4% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2: Solution2 (further transform block split for asymmetric partitions)

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% |
| Class B | -0.1% | -0.2% | -0.1% |
| Class C | -0.1% | -0.1% | 0.0% |
| Class D | -0.2% | -0.3% | -0.5% |
| Class E |  |  |  |
| Overall | -0.1% | -0.2% | -0.2% |
|  | -0.1% | -0.2% | -0.2% |
| Class F | -0.2% | -0.3% | -0.3% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | -0.6% | -0.1% |
| Class C | -0.2% | -0.1% | -0.1% |
| Class D | -0.3% | -1.2% | -0.7% |
| Class E | 0.0% | -0.1% | 0.7% |
| Overall | -0.2% | -0.5% | -0.1% |
|  | -0.2% | -0.5% | -0.1% |
| Class F | -0.4% | -0.2% | -0.3% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |

# Proposed draft text

The solution1 doesn't have the impact on the syntax of implicit transform block split process. The definition of non-square transform and non-square transform split direction is same as NSQT.

The change of the text for the solution2 is show as follow (yellow highlight part):

7.4.8 Transform tree semantics

The variable interSplitFlag is derived as follows.

- If max_transform_hierarchy_depth_inter is equal to 0 and PredMode is equal t oMODE_INTER and PartMode is not equal to PART_2Nx2N and trafoDepth is equal to 0, interSplitFlag is set to 1.

- Otherwise, if max_transform_hierarchy_depth_inter is equal to 0 and trafoDepth is equal to 1 and ( log2CbSize - Log2MinTrafoSize ) is greater than 1 and nsrqt_enabled_flag is equal to 0 and PartMode is equal to PART_2NxnU or PART_2NxnD or PART_nLx2N or PART_nRx2N, interSplitFlag is set to 1.

- Otherwise, interSplitFlag is set to 0.

# Patent rights declaration(s)

HiSilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).