| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0367 |
| --- | --- |

| Title: | On level considerations for interlaced content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jérôme Viéron, Pierre Larbier, Jean-Marc Thiesse ATEME 26 Burospace, Route de Gisy, 91570 Bièvres, FRANCE | Tel: Email: | +33 16 935 8998 j.vieron@ateme.com, p.larbier@ateme.com, jm.thiesse@ateme.com |
| Source: | ATEME |  |  |

_____________________________

# Abstract

Support for interlaced formats, in the current state of HEVC development, is mainly achieved by considering field coding in one hand, and signaling of interlaced content in other hand.

When dealing with field coding only, the current constant DPB size constraint has to be reconsidered. Moreover, in order to allow efficient hierarchical coding structures as commonly used in H.264/AVC, it may be highly desirable to increase the current fixed limit.

The current contribution proposes to extend the MaxDpbSize for levels dealing with interlaced content coding for broadcast distribution, while taking into account the respective memory footprints.

# Introduction

In the current state of HEVC development, support for interlaced formats is only achieved through the signaling of interlaced content using the field indication SEI message. Moreover, field coding is considered leading to half-height picture processing (i.e. half the height of a conventional non deinterleaved picture) and double picture rate.

In the current draft text [1], the decoded picture buffers MaxDpbSize is fixed to 6 for all levels. When dealing with field coding only, such constant DPB size constraint has to be reconsidered. Furthermore, in order to allow “conventional” and efficient hierarchical coding structures (as in H.264/AVC), it may be highly desirable to increase the current fixed limit.

In the current contribution, we propose to extend the MaxDpbSize to a more convenient value for levels classically dealing with interlaced content coding (i.e. SDi and 1080i) for broadcast. Memory footprint constraint is conjointly considered in the current proposal.

# Proposal

## Description

The first natural trend would be to double the current MaxDpbSize to 12 for levels used when encoding interlaced content for broadcast distribution. Thus, Level 3 is used for conventional SDi content (720x576) at 25/30 fps and Level 4 is used for 1080i (1920x1080) at 25/30fps.

When doubling the MaxDpbSize value hierarchical coding structures commonly used with H.264/AVC become possible. However, the memory footprint could be considered as too important for decoder provider. Finally, one must remark that conventional memory capacity is a power of 2 Mo.

Consequently, we propose to set the MaxDpbSize to 10 for both Level 3 and Level 4.  Thus, considering HD (1080i30) content, memory needs remain lower than 32Mo. This constraint would not “technically speaking” increase the memory needs since a MaxDpbSize equal to 6 involves a memory footprint of 17Mo which is already more than 16Mo.

Moreover a DPB size of 10 is sufficient in order to build a hierarchical structure with 7 B-frames considering H.264/AVC-like discardability features and inter-field references.

## Draft text modifications

It is proposed to modify the draft text as described in Table 1.

Table 1.  Proposed DPB size modifications.

| Level | Max luma sample rate MaxLumaPR (samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MaxDpbSize (picture storage buffers) | Max CPB size (1000 bits) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 10 | 5,000 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 10 | 15,000 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 |

## On interim levels

In order to reduce the memory footprint, one could propose to add interim levels. For instance, considering SDi content a Level 2.1 and for 1080i content a Level 3.2 could be defined as follows.

- Level 2.1: SD (576i)

MaxLumaPR = 11,059,200, but MaxLumaFS= 458,752/2 and MaxDpbSize = 6 * 2  = 12

- Level 3.2: HD (1080i)

MaxLumaPR = 62,668,800, but MaxLumaFS= 2,088,960/2 and MaxDpbSize = 6 * 2 = 12

With such an approach the memory footprint remains unchanged, i.e. 3.9 Mo for SD and 17,9Mo for HD.

# Conclusion

In this contribution, we propose to modify levels definition in order to take into account constraints induced by efficient interlaced content coding. Hence, MaxDpbSize is proposed to be increased to 10 for both level 3 and 4.

We suggest the JCTVC group to consider the current proposal for adoption in the profile/levels definition.

# Reference(s)

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand, “High Efficiency Video Coding

(HEVC) text specification draft 7,” document JCTVC-I1003_d2, JCT-VC, Geneva, Switzerland, April 2012.

# Patent rights declaration(s)

ATEME does not have any current or pending patent rights relating to the technology described in this contribution.