| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0368 |
| --- | --- |

| Title: | Cross-check of additional horizontal and vertical scans (JCTVC-J0281) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the results of the cross-check performed by Qualcomm of the two methods proposed by Sony in JCTVC-J0281. Method A extends the mode dependant scanning (horizontal/vertical scans) to 8x8 TU chroma. Method B extends the scans to large transforms (16x16/32x32). The cross-check performance results for common conditions settings in HM7.0 match those provided by the proponent. The implemented algorithm agrees with what is described in Sony’s proposal.

# Description

Software implementing JCTVC-J0281 on top HM7.0 basis was provided by Sony.

HM7.0 applies horizontal/vertical scans for some intra modes for TU 4x4 and 8x8 luma and for 4x4 chroma. 8x8 chroma always uses diagonal scan.

Method A in  JCTVC-J0281 adds the horizontal/vertical scans to 8x8 chroma. Code modifications for this addition are very small and found in file TComDataCU.cpp. The change allows the selection of horizontal and vertical scans depending on the intra mode for the 8x8 TU in the same way as done for the 4x4 TU.

Method B builds on the top of method A. It extends the mode dependant coefficient scans for luma and chroma to the large TU sizes used in intra (16x16/32x32). Code changes for method B are the following:

- TComDataCU.cpp: allow MDCS for all TU sizes, luma and chroma. Mode thresholds for scan index assigment are different for 16x16 and 32x32 (2 instead of 5). Thresholds are the same for luma and chroma.

- TComRom.cpp: The two additional coefficient group scans are defined here.

- TComTrQuant.cpp: Includes additional logic to compute the neighboring coefficient group significance for the additional scans. Includes additional logic for significance map context derivation (patterns) for the coefficient group shapes created by the additional scans.

- TDecSbac.cpp / TEncSbac.cpp: Includes logic to assign the additional coefficient group scans when the horizontal/vertical scans are applied to large TU. Includes the logic for the coefficient group flag context derivation for the coefficient group shapes created by the additional scans.

# Results

Coding results using common conditions with HM7.0 anchor have been obtained and match those of the proponent. Results for both methods are provided in the accompanying excel file.