| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0371 |
| --- | --- |

| Title: | Cross-check of JCTVC-J0179 on sao_merge_left_flag for effective Mx1 CTB coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross verification for JCTVC-J0179 on sao_merge_left_flag for effective Mx1 CTB coding. Simulation results report that BD-rate numbers match with results from proponents of JCTVC-J0179. Encoding and decoding time is not accurate in the cross-check report.

# Algorithm and Software

JCTVC-J0179 proposed to combine the signalling of SAO information of several CTB into one by enforcing sao_merge_left_flag to be 1 when the CTB size is as small as 32x32 or 16x16.

Software implementation includes three macros. This cross verification only studies the settings as follows:

#define SAO_Nx1LCU_UNIT                  1 // enabled SAO unit of Nx1 LCUs

#define SAO_Nx1LCU_ENCONLY               0

#define SAO_Nx1LCU_SLICE                 0

The setting is claimed to be the one proposed in JCTVC-J0179. Study of the software implementation shows that it matches with the text in JCTVC-J0179.

In the encoder side, SAO RDO is changed to estimate SAO information based on Mx1 CTB.

# Simulations

The software is compiled using macros setting as follows:

#define SAO_Nx1LCU_UNIT                  1 // enabled SAO unit of Nx1 LCUs

#define SAO_Nx1LCU_ENCONLY               0

#define SAO_Nx1LCU_SLICE                 0

Configurations are changed based on common test conditions for the two simulations:

CTB size is 64, *--SAOUnitWidth=2*.

CTB size is 32, *--SAOUnitWidth=8, --MaxCUWidth=32, --MaxCUHeight=32,

--MaxPartitionDepth=3*

Simulations are conducted in a heterogeneous cluster. Thus encoding and decoding time is not accurate. BD-rate numbers from cross-checking results match with results from JCTVC-J0179.