| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0375 WG11 Number: m25709 |
| --- | --- |

| Title: | Cross-verification of Modification of context assignment for significance flag coding for large TUs (JCTVC-J0354) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of JCTVC-J0354, “Modification of context assignment for significance flag coding for large TUs” [1]. The simulation results performed for this cross-verification match those provided by the proponents. The software changes are consistent with what is described in the proposal.

# Introduction and examination of software

The proposal JCTVC-J0354 [1] modifies the adoption from JCTVC-I0296 [2] by adding 3 contexts to the significance flag in a way that is consistent with how the existing significance flag contexts are derived. The software modifications provided by the proponent generate the context assignment table in a way that is consistent with Figure 1 of JCTVC-J0354.

# Simulation results

The proposal was compared to a reference of HM 7.0, using common test conditions [3].

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.14% | -0.06% | -0.08% | -0.12% | -0.08% | -0.04% |
| Class B | -0.15% | -0.14% | -0.11% | -0.14% | -0.08% | -0.10% |
| Class C | -0.05% | -0.07% | -0.04% | -0.04% | -0.05% | -0.03% |
| Class D | -0.06% | 0.01% | -0.02% | -0.04% | 0.00% | 0.00% |
| Class E | -0.08% | -0.03% | -0.09% | -0.05% | -0.04% | -0.05% |
| Overall | -0.10% | -0.06% | -0.07% | -0.09% | -0.05% | -0.05% |
|  | -0.10% | -0.07% | -0.07% | -0.09% | -0.05% | -0.04% |
| Class F | -0.06% | 0.02% | 0.06% | -0.07% | -0.14% | 0.00% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.67% | -0.47% | -0.09% | -0.11% | -0.19% |
| Class B | -0.11% | 0.06% | -0.12% | -0.08% | 0.01% | -0.03% |
| Class C | -0.06% | 0.07% | -0.04% | 0.00% | 0.04% | -0.03% |
| Class D | 0.01% | 0.37% | -0.04% | -0.01% | 0.07% | -0.11% |
| Class E |  |  |  |  |  |  |
| Overall | -0.06% | -0.04% | -0.17% | -0.05% | 0.00% | -0.09% |
|  | -0.14% | -0.06% | -0.19% | -0.05% | -0.02% | -0.10% |
| Class F | -0.07% | 0.18% | 0.10% | -0.01% | 0.08% | -0.08% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.05% | 0.26% | 0.06% | 0.01% | 0.28% | 0.29% |
| Class C | 0.02% | 0.02% | 0.11% | 0.04% | 0.24% | 0.26% |
| Class D | 0.05% | 0.32% | 0.09% | 0.06% | 0.17% | -0.05% |
| Class E | -0.06% | 0.49% | 0.17% | 0.06% | -0.55% | -0.15% |
| Overall | -0.01% | 0.26% | 0.10% | 0.04% | 0.09% | 0.11% |
|  | -0.01% | 0.17% | 0.08% | 0.04% | 0.12% | 0.16% |
| Class F | 0.01% | -0.38% | 1.33% | 0.13% | 0.31% | 0.40% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |

# Conclusions

The simulation results for the cross-check match those of the proponent. Encode and decode times are consistent as well. The modifications to the software are consistent with the additional contexts described in the proposal.

# References

- R. Joshi, J. Sole, and M. Karczewicz, “Modification of context assignment for significance flag coding for large TUs,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0354, Stockholm, SE, July 2012

- T. Kumakura and S. Fukushima, “Non-CE3: Simplified context derivation for significance map,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I0296, Geneva, CH, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1100, Geneva, CH, May 2012.