| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0376 WG11 Number: m25710 |
| --- | --- |

| Title: | Cross-check report for Simplification on context derivation on cbf_luma syntax element (JCTVC-J0303) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hisao Sasai 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-6-6900-9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result for simplification on context derivation on cbf_luma syntax element (JCTVC-J0303). The experimental results perfectly match with the one provided by proponent in the rate-distortion performance.

# Source code check

In the contribution JCTVC-J0303[1], the context derivation process on cbf_luma was modified as follows.

draft text : 		ctxIdxInc = ( trafoDepth = = 0 ) | | ( log2TrafoSize = =  Log2MaxTrafoSize ) ? 1 : 0 
JCTVC-J0303:	ctxIdxInc = ( trafoDepth = = 0 )  ? 1 : 0

The following MACRO "LUMA_CBF_CTX_2CTX" is used for the proposal. The changes match the proposed text.

UInt TComDataCU::getCtxQtCbf( UInt uiAbsPartIdx, TextType eType, UInt uiTrDepth ) { if( eType ) { return uiTrDepth; } else { #if LUMA_CBF_CTX_2CTX const UInt uiCtx = ( uiTrDepth == 0 ? 1 : 0 ); #else const UInt uiDepth = getDepth( uiAbsPartIdx ); const UInt uiLog2TrafoSize = g_aucConvertToBit[ getSlice()->getSPS()->getMaxCUWidth() ] + 2 - uiDepth - uiTrDepth; const UInt uiCtx = uiTrDepth == 0 ¦¦ uiLog2TrafoSize == getSlice()->getSPS()->getQuadtreeTULog2MaxSize() ? 1 : 0; #endif return uiCtx; } }

# Experimental results

The software to be tested was provided by the proponent and its performance relative to HM7.0 software was checked under the common test condition described in JCTVC-I1100 [2]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. Table 1 shows RD performance and execution time of the proposed method relative to HM7.0.

Table 1: experimental result of the proposed method Conclusion

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.07% | 0.04% | 0.01% | 0.05% | 0.04% |
| Class B | 0.01% | 0.03% | 0.03% | 0.01% | 0.04% | 0.02% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | 0.00% |
| Class D | 0.00% | 0.01% | 0.01% | 0.00% | 0.01% | 0.00% |
| Class E | 0.00% | 0.05% | 0.00% | 0.00% | 0.02% | 0.03% |
| Overall | 0.00% | 0.03% | 0.02% | 0.01% | 0.02% | 0.02% |
|  | 0.00% | 0.03% | 0.02% | 0.01% | 0.02% | 0.02% |
| Class F | 0.00% | 0.00% | 0.08% | 0.02% | -0.14% | 0.03% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.19% | -0.28% | -0.01% | 0.27% | 0.25% |
| Class B | 0.00% | 0.10% | 0.03% | 0.01% | 0.03% | 0.06% |
| Class C | 0.00% | 0.06% | 0.04% | 0.02% | -0.02% | -0.03% |
| Class D | 0.00% | 0.03% | -0.06% | 0.02% | 0.06% | -0.02% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.06% | 0.01% | 0.08% | 0.06% |
|  | 0.01% | -0.01% | -0.08% | 0.01% | 0.08% | 0.06% |
| Class F | 0.03% | 0.15% | 0.11% | 0.07% | 0.01% | -0.06% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.01% | -0.05% | 0.18% | -0.03% | 0.19% | 0.09% |
| Class C | 0.00% | 0.05% | 0.17% | 0.01% | -0.04% | -0.16% |
| Class D | 0.01% | 0.23% | 0.20% | 0.01% | 0.05% | 0.22% |
| Class E | -0.04% | 0.09% | 0.25% | 0.10% | -0.71% | 0.01% |
| Overall | 0.00% | 0.07% | 0.20% | 0.01% | -0.07% | 0.04% |
|  | 0.00% | 0.10% | 0.14% | 0.01% | -0.06% | 0.04% |
| Class F | -0.02% | 0.09% | 0.19% | 0.16% | -0.05% | 0.49% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

# Conclusion

The results provided by the proponent for "simplification on context derivation on cbf_luma syntax element " were confirmed.

# References

- Il-Koo Kim et al, “Simplification on context derivation of cbf_luma syntax element”, JCTVC-J0303, Stockholm, July 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, Geneva, April 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).