| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0377 |
| --- | --- |

| Title: | Cross-check of JCTVC-J0099 on sign data hiding simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross-verification of the Method 1 of Qualcomm’s contribution JCTVC-J0099 which removes the first non-zero position dependency from the sign hiding criterion.  The software implementation matches the description in the contribution document. The simulation results matched the results provided by proponents in BD-rate numbers. Encoding and decoding time information was not accurate and therefore not reported.

# Introduction

The contribution JCTVC-J0099 [3] proposed two methods to simplify the condition to turn on sign hiding. This document reports cross-verification of Method 1 of JCTC-J0099. In Method 1, the condition for turning on sign data hiding in HM7 [2]

lastNZPosInCG – firstNZPosInCG >= 4

is simplified to

lastNZPosInCG >= 4

The software implementation matches the description in the contribution document.

# Results

The BDR results matched the results in JCTC-J0099 [3]. Since the simulation times were not accurate and therefore they are not reported here.

Table 1 is the BDR results with Common Test Conditions [1]. In particular, the Method 1 has BD-Rate reduction of -0.1% for the All Intra Main test cases for both luminance and chrominance. For Random Access Main and Low Delay B Main, there appears to have a shift of coding gain from chrominance to luminance. For Random Access HE10 and Low Delay B HE10, there is a coding loss of BDR from 0.0% to 0.1% for luminance and 0.0% to 0.4% for chrominance.

Table 2 is the BDR results with RDOQ off. In general, there are coding gain of -0.1 to -0.2% BDR on luminance and coding loss of 0.0% to 0.7% BDR on chrominance.

Table 1: Method 1 results with Common Test Conditions.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.05% | -0.15% | -0.09% | 0.00% | 0.03% | 0.00% |
| Class B | -0.07% | -0.13% | -0.10% | 0.03% | 0.01% | -0.01% |
| Class C | -0.11% | -0.16% | -0.18% | 0.01% | 0.01% | -0.01% |
| Class D | -0.10% | -0.15% | -0.22% | 0.01% | 0.02% | 0.02% |
| Class E | -0.12% | -0.01% | 0.02% | 0.01% | -0.05% | 0.03% |
| Overall | -0.09% | -0.13% | -0.12% | 0.01% | 0.01% | 0.00% |
|  | -0.09% | -0.13% | -0.12% | 0.01% | 0.00% | 0.00% |
| Class F | -0.16% | -0.17% | -0.25% | 0.18% | -0.10% | 0.24% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.20% | 0.13% | 0.00% | 0.51% | 0.72% |
| Class B | -0.05% | 0.29% | 0.10% | 0.03% | 0.35% | 0.16% |
| Class C | -0.02% | 0.09% | -0.09% | 0.06% | 0.37% | 0.02% |
| Class D | -0.05% | 0.32% | 0.16% | 0.03% | 0.17% | -0.10% |
| Class E |  |  |  |  |  |  |
| Overall | -0.05% | 0.14% | 0.08% | 0.03% | 0.35% | 0.20% |
|  | -0.02% | 0.13% | 0.06% | 0.04% | 0.34% | 0.18% |
| Class F | -0.08% | 0.01% | -0.11% | 0.23% | 0.65% | 0.29% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.08% | 0.61% | 0.50% | 0.07% | 0.80% | 0.49% |
| Class C | -0.08% | 0.43% | 0.17% | 0.07% | 0.29% | 0.31% |
| Class D | -0.09% | 0.65% | 0.72% | 0.03% | 0.58% | -0.13% |
| Class E | -0.14% | 0.59% | -0.78% | 0.20% | -0.42% | 0.17% |
| Overall | -0.09% | 0.57% | 0.23% | 0.09% | 0.39% | 0.23% |
|  | -0.09% | 0.51% | 0.22% | 0.09% | 0.47% | 0.30% |
| Class F | -0.21% | 0.11% | 0.36% | 0.30% | 1.28% | 0.78% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 2: Method 1 results with RDOQ off.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.08% | 0.03% | -0.02% | -0.14% | 0.14% | 0.04% |
| Class B | -0.13% | 0.10% | 0.07% | -0.17% | 0.15% | 0.12% |
| Class C | -0.11% | -0.01% | 0.00% | -0.11% | 0.05% | 0.02% |
| Class D | -0.11% | 0.09% | 0.06% | -0.10% | 0.11% | 0.02% |
| Class E | -0.10% | 0.07% | 0.01% | -0.11% | 0.05% | 0.01% |
| Overall | -0.11% | 0.06% | 0.03% | -0.13% | 0.11% | 0.05% |
|  | -0.11% | 0.05% | 0.03% | -0.13% | 0.11% | 0.05% |
| Class F | -0.08% | 0.04% | 0.06% | 0.10% | 0.08% | 0.18% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.14% | 0.04% | 0.25% | -0.14% | 0.72% | 0.20% |
| Class B | -0.07% | 0.25% | 0.23% | -0.08% | 0.35% | 0.43% |
| Class C | -0.18% | -0.05% | 0.13% | -0.05% | 0.10% | 0.19% |
| Class D | -0.25% | 0.10% | -0.19% | -0.15% | -0.01% | 0.39% |
| Class E |  |  |  |  |  |  |
| Overall | -0.15% | 0.10% | 0.11% | -0.10% | 0.29% | 0.31% |
|  | -0.15% | 0.10% | 0.14% | -0.10% | 0.23% | 0.29% |
| Class F | -0.08% | -0.17% | -0.08% | 0.15% | 0.18% | 0.41% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.12% | 0.48% | 0.48% | -0.08% | 0.55% | 0.32% |
| Class C | -0.21% | 0.23% | 0.20% | -0.18% | 0.27% | 0.09% |
| Class D | -0.21% | 0.16% | 0.32% | -0.09% | 1.04% | 0.15% |
| Class E | -0.10% | -0.46% | 0.43% | 0.08% | 1.18% | -0.24% |
| Overall | -0.16% | 0.16% | 0.36% | -0.08% | 0.72% | 0.11% |
|  | -0.16% | 0.13% | 0.34% | -0.07% | 0.72% | 0.15% |
| Class F | -0.22% | 0.56% | -0.40% | 0.11% | -0.14% | 1.25% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, 9th Meeting: Geneva, CH, 27 Apr – 7 May, 2012.

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003_d4, 9th Meeting: Geneva, CH, 27 Apr – 7 May, 2012.

- Joel Sole and Marta Karczewicz, “Sign data hiding simplification”, JCTVC-J0099, 10th Meeting: Stockholm, SE, 11 – 20 July 2012.