| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0380 |
| --- | --- |

| Title: | Cross-check of JCTVC-J0171 on transform skip mode for chroma TUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross-verification of the Proposal 2 of Canon’s proposal JCTVC-J0171 which propose to disable the Transform Skip mode of 4x4 chroma TU based on conditions on the corresponding 8x8 luma TU.  The software implementation matches the description in the contribution document. Since transform skip is not activated in main test configurations , the cross-check results were limited to HE10 test configurations, and they matched the results provided by proponents in BD-rate numbers. Encoding and decoding time information was not accurate and therefore not reported.

# Introduction

The Proposal 2 of the contribution JCTVC-J0171 [3] proposed to disable Transform Skip mode of 4x4 chroma TU when one of the following conditions on the corresponding 8x8 luma TU is true:

- The 8x8 luma TU is not split into 4x4  TUs.

- The 8x8 luma TU is split into 4x4 TUs and none of the 4x4 TUs uses transform skip mode.

The software implementation matches the description in the contribution document. In particular, the changes in HM7 were localized in the following three functions:

- TDecSbac::parseTransformSkipFlags

- TEncSbac::codeTransformSkipFlags

- TEncSearch::xRecurIntraChromaCodingQT

# Results

Since transform skip is only activated in HE10 test configurations (and not in main test configurations), the cross-checking were limited to HE10 test configurations. The BDR results matched the results in JCTC-J0171 [3] for AI HE10, RA HE10, and LB HE10. Since the simulation times were not accurate and therefore they were not reported here.

Table 1: HE10 BDR results.

| All Intra HE10 |  |  |
| --- | --- | --- |
| Y | U | V |
| -0.02% | 0.01% | 0.05% |
| -0.03% | 0.08% | 0.09% |
| -0.05% | 0.14% | 0.09% |
| -0.05% | 0.12% | 0.13% |
| -0.04% | -0.01% | 0.26% |
| -0.04% | 0.07% | 0.12% |
| -0.04% | 0.08% | 0.13% |
| -0.05% | 0.10% | 0.25% |
| #NUM! |  |  |
| #NUM! |  |  |
| Random Access HE10 |  |  |
| Y | U | V |
| 0.02% | 0.36% | 0.31% |
| -0.03% | 0.14% | 0.14% |
| -0.03% | 0.26% | 0.28% |
| -0.05% | 0.20% | 0.05% |
| -0.02% | 0.23% | 0.19% |
| -0.02% | 0.21% | 0.21% |
| -0.06% | 0.44% | 0.37% |
| #NUM! |  |  |
| #NUM! |  |  |
| Low delay B HE10 |  |  |
| Y | U | V |
| -0.01% | 0.09% | -0.03% |
| -0.06% | -0.01% | 0.11% |
| -0.10% | 0.25% | -0.02% |
| 0.18% | -0.19% | 0.07% |
| -0.01% | 0.05% | 0.03% |
| -0.01% | 0.15% | 0.08% |
| 0.23% | 0.69% | 0.87% |
| #NUM! |  |  |
| #NUM! |  |  |

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, 9th Meeting: Geneva, CH, 27 Apr – 7 May, 2012.

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003_d4, 9th Meeting: Geneva, CH, 27 Apr – 7 May, 2012.

- Edouard Francois, Patrice Onno, Christophe Gisquet , and Guillaume Laroche, “On transform skip mode for chroma TUs”, JCTVC-J0171, 10th Meeting: Stockholm, SE, 11 – 20 July 2012.