| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0386 |
| --- | --- |

| Title: | Cross check of coefficient scan for transform skip mode (JCTVC-J0202) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check result of JCTVC-J0202, which introduces coefficient scan modifications for blocks coded with transform skip mode. The algorithms and source codes are studied and verified. In addition, it is confirmed that the experimental resutls match with those provided by the proponents.

# Cross check

JCTVC-J0202 proposes two solutions for coefficient scan in blocks where transform skip mode is applied. In the cross check, those two solutions are tested.

- Solution 1: Always use diagonal scan

- Solution 2: Vertical scan for near vertical modes and horizontal scan for near horizontal modes

In both solutions, changes are limited to scan index derivation. Except for scan, coefficient coding process is unchanged including contexts for CABAC.

Using the software provided by the proponents, simulations were carried out based on the Common test condition. All intra with Main and HE10 are tested. Note that, in Main configuration, transform skip is enabled for both proposed method and anchor. Linux cluster were used for running the simulation, thus encoding and decoding time may not be accurate.

Table 1 and 2 show the experimental results for Solution 1 and 2 respectively. It is confirmed that BD-rate values match with those provided by the proponents.

Table 1: Results for Solution 1 (Always use diagonal scan)

Table 2: Results for Solution 2 (Vertical scan for near vertical modes and horizontal scan for near horizontal modes)

# Related contributions

J0212 and J0313 (Method 1) propose exactly the same method as Solution 2 in J0202. J0053 and J0093 propose the different coefficient scan method for transform skip mode. J0069 proposes to use constant context for coefficient of transform skip mode. J0027 and J0237 proposes application of transform skip mode in inter blocks.

# Conclusion

The algorithms and experimental results of coefficient scan for transform skip mode (JCTVC-J0202) are checked and confirmed. Since several contributions propose modifications of coefficient coding of transform skip mode, it is suggested to study and discuss relation with other proposals during the meeting.