| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11 – 20 July, 2012 | Document: JCTVC-J0387 |
| --- | --- |

| Title: | AHG6/AHG5: Cross-check of JCTVC-J0106 (SAO offset coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Takayuki Itoh | Email: | tomoo.yamakage@toshiba.co.jp takayuki2.itoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Cross-check results of Qualcomm’s proposal on JCTVC-J0106 (SAO offset coding) are reported. Coding efficiency results are exactly matched. Relative encoding and decoding time are different in some cases　(3%). However, it would be due to the test platform issue.

# Introduction

This contribution is a report of cross-check results of Qualcomm’s proposal on JCTVC-J0106 (SAO offset coding). The purposes of this proposal are (1) to reduce the worst number of bins to 5/7/9 for bit-depth 8/9/10 or higher, respectively, and (2) coding some of the bins in bypass and without coding performance loss on average.

Coding efficiency results are exactly matched with the proponent’s results in all test cases. There is no mismatch between encoder and decoders. Relative encoding and decoding time are different in some cases. However, it would be due to the test platform issue.

# Experimental Results

Detailed results in Excel sheet are attached.  The following table is the summary of experimental results.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.1% | 0.1% | 0.2% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.3% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.3% |
| Class D | 0.0% | 0.4% | 0.8% | 0.0% | 0.5% | -0.2% |
| Class E | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | 0.1% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class F | 0.1% | 0.4% | 0.5% | 0.1% | 0.4% | 0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.1% | 0.2% | -0.1% |
| Class C | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.2% |
| Class D | 0.0% | 0.2% | 0.4% | 0.0% | 0.0% | -0.3% |
| Class E | 0.1% | -0.7% | 0.0% | 0.0% | 0.4% | 0.2% |
| Class F | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.3% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 1. Results for proposed binarization with prefix bins context-coded

|  | All Intra Main |  |  |  |  |  | All Intra HE10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |  |
| Class A | 0.0% |  | 0.2% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.1% |  |  |
| Class B | 0.0% |  | 0.3% |  | 0.3% |  | 0.0% |  | 0.1% |  | 0.1% |  |  |
| Class C | 0.0% |  | 0.2% |  | 0.2% |  | 0.0% |  | 0.1% |  | 0.1% |  |  |
| Class D | 0.0% |  | 0.1% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Class E | 0.0% |  | 0.4% |  | 0.4% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Overall | 0.0% |  | 0.2% |  | 0.3% |  | 0.0% |  | 0.0% |  | 0.1% |  |  |
|  | 0.0% |  | 0.2% |  | 0.3% |  | 0.0% |  | 0.0% |  | 0.1% |  |  |
| Class F | 0.0% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.1% |  |  |
| Enc Time[%] | 99% |  |  |  |  |  | 100% |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  | 100% |  |  |  |  |  |  |
|  | Random Access Main |  |  |  |  |  | Random Access HE10 |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |  |
| Class A | 0.0% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.1% |  |  |
| Class B | 0.0% |  | 0.5% |  | 0.4% |  | 0.0% |  | 0.1% |  | 0.2% |  |  |
| Class C | 0.0% |  | 0.2% |  | 0.3% |  | 0.0% |  | 0.2% |  | 0.0% |  |  |
| Class D | 0.0% |  | 0.2% |  | 0.2% |  | 0.0% |  | 0.2% |  | 0.0% |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% |  | 0.3% |  | 0.2% |  | 0.0% |  | 0.2% |  | 0.1% |  |  |
|  | 0.0% |  | 0.3% |  | 0.2% |  | 0.0% |  | 0.1% |  | 0.1% |  |  |
| Class F | 0.1% |  | 0.2% |  | 0.2% |  | 0.0% |  | 0.2% |  | 0.3% |  |  |
| Enc Time[%] | 99% |  |  |  |  |  | 100% |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  | 100% |  |  |  |  |  |  |
|  | Low delay B Main |  |  |  |  |  | Low delay B HE10 |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | Y |  | U |  | V |  |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% |  | 0.6% |  | 0.8% |  | 0.0% |  | 0.3% |  | 0.3% |  |  |
| Class C | 0.0% |  | 0.3% |  | 0.4% |  | 0.0% |  | 0.2% |  | 0.3% |  |  |
| Class D | 0.0% |  | 0.4% |  | 0.7% |  | 0.0% |  | 0.0% |  | -0.2% |  |  |
| Class E | 0.0% |  | 0.7% |  | 1.0% |  | 0.1% |  | -0.4% |  | 0.4% |  |  |
| Overall | 0.0% |  | 0.5% |  | 0.7% |  | 0.0% |  | 0.1% |  | 0.2% |  |  |
|  | 0.0% |  | 0.5% |  | 0.7% |  | 0.0% |  | 0.2% |  | 0.2% |  |  |
| Class F | 0.0% |  | 0.4% |  | 0.5% |  | 0.1% |  | 0.3% |  | 1.0% |  |  |
| Enc Time[%] | 99% |  |  |  |  |  | 100% |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  | 100% |  |  |  |  |  |  |
|  |  | Low delay P Main |  |  |  |  |  | Low delay P HE10 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.2% |  | 0.9% |  | 1.1% |  | 0.1% |  | 0.1% |  | 0.1% |  |
| Class C |  | 0.1% |  | 0.2% |  | 0.4% |  | 0.1% |  | 0.3% |  | 0.2% |  |
| Class D |  | 0.1% |  | 0.4% |  | 0.8% |  | 0.0% |  | 0.3% |  | -0.3% |  |
| Class E |  | 0.3% |  | 0.6% |  | 1.5% |  | 0.1% |  | 0.6% |  | 0.0% |  |
| Class F |  | 0.0% |  | 0.1% |  | 0.1% |  | -0.1% |  | -0.1% |  | -0.4% |  |
| Overall |  | 0.1% |  | 0.5% |  | 0.8% |  | 0.0% |  | 0.2% |  | 0.0% |  |
|  |  | 0.1% |  | 0.5% |  | 0.7% |  | 0.0% |  | 0.2% |  | 0.1% |  |
| Enc Time[%] |  | 99% |  |  |  |  |  | 100% |  |  |  |  |  |
| Dec Time[%] |  | 100% |  |  |  |  |  | 100% |  |  |  |  |  |

Table 2. Results for proposed binarization with 1bin context-coded

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.2% | 0.3% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.4% | 0.4% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class B | 0.0% | 0.4% | 0.3% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.2% | 0.3% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | -0.1% |
| Class F | 0.1% | 0.2% | 0.2% | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.7% | 0.6% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.4% | 0.4% | 0.0% | 0.1% | 0.4% |
| Class D | 0.0% | 0.4% | 0.4% | 0.0% | 0.4% | -0.1% |
| Class E | 0.0% | 0.7% | 0.7% | 0.1% | -0.1% | 0.4% |
| Overall | 0.0% | 0.5% | 0.5% | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.5% | 0.5% | 0.0% | 0.2% | 0.3% |
| Class F | 0.1% | 0.8% | 0.3% | 0.2% | 0.1% | 0.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 1.0% | 0.8% | 0.1% | 0.2% | 0.1% |
| Class C | 0.1% | 0.3% | 0.5% | 0.0% | 0.1% | 0.3% |
| Class D | 0.1% | 0.5% | 0.3% | 0.1% | 0.4% | -0.2% |
| Class E | 0.3% | 0.8% | 1.7% | 0.0% | 0.5% | 0.5% |
| Class F | 0.0% | 0.4% | 0.1% | 0.1% | 0.3% | -0.3% |
| Overall | 0.1% | 0.6% | 0.6% | 0.1% | 0.3% | 0.0% |
|  | 0.1% | 0.6% | 0.6% | 0.1% | 0.3% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 3. Results for proposed binarization with all bins bypass-coded

# Conclusion

This contribution shows cross-check results of JCTVC-J0106. Coding efficiency results are confirmed. No mismatch between encoder and decoder is observed.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).