| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0389 |
| --- | --- |

| Title: | Simplification for intra transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Jicheng An North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Debin Zhao Department of Computer Science and Technology, Harbin Institute of Technology, Harbin, China 150001 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {liang.zhao, jicheng.an, yuwen.huang, shawmin.lei} @mediatek.com Debin Zhao dbzhao@jdl.ac.cn |
| Source: | MediaTek Inc, Harbin Institute of Technology |  |  |

_____________________________

# Abstract

In this contribution, a limitation is proposed to speed up the intra transform skip mode decision on the encoder side as follows. For a 4x4 intra TU, an intra transform skip flag will be transmitted to the decoder only when the size of CU is 8x8 and the partition mode is Intra_NxN. This limitation can be implemented both normatively and non-normatively. However, the non-normative method will transmit unnecessary transform skip flags to the decoder. It is reported that the proposed methods can achieve 9% encoder run time reduction without noticeable BD-rate changes when compared with the HM7.0 anchor in HE10-AI condition.

# Introduction

In the current HEVC (text specification draft 7) [1], an intra transform skip mode proposed in JCTVC-I0408 [2] has been adopted to improve the coding efficiency, especially for the class F sequences. For a 4x4 intra transform unit (TU), the transform skip mode can be enabled. For the transform skipping TUs, a simple scaling process is used instead of the transform.

For a 4x4 intra TU, no matter whether the size of the CU and the PU is 4x4 or not, a flag will be transmitted to the decoder to indicate whether the transform skip mode is used or not. In the common test condition, QuadtreeTUMaxDepthIntra is set to 3. Therefore, there are three cases where the transform skip flag will be transmitted: 1) the size of CU is 16x16 and the size of TU is 4x4; 2) the size of CU is 8x8, the partition mode is Intra_2Nx2N and the size of TU is 4x4; 3) the size of CU is 8x8, the partition mode is Intra_NxN and the size of TU is 4x4.

# Proposed method

However, from our observation, most gain of the transform skip mode is obtained from the third case. Inspired from that, it is proposed to signal the transform skip mode only when the condition of the third case is satisfied.

Clearly, this limitation can also be implemented as an encoder-only method. In this way, we set the transform skip flag to 0 when the first or second case is satisfied. Different from the normative modification, we should transmit this flag to the decoder although it is always 0. Both methods can speed up the encoder because of the first case and the second case are not searched.

# Simulation results

The proposed methods have been implemented on top of HM-7.0. The simulation has been done according to the HE10 common test conditions [3], since the intra transform skip mode is only turned on in HE10 conditions. Results are shown in Table 1 and 2. It can be seen that the encoder complexity reduction of the normative method and non-normative method are both about 9%. However, without transmitting the unnecessary transform skip flag in the first case and the second case, the coding efficiency of the normative method is a little better than the non-normative method.

Table 1.  Results of the normative method

Table 2.  Results of the non-normative method

# Conclusion

In this contribution, a limitation is proposed to speed up the intra transform skip mode. This limitation is that, for a 4x4 intra TU, an intra transform skip flag will be transmitted to the decoder only when the size of CU is 8x8 and the partition mode is Intra_NxN. It is reported that the proposed method can achieve 9% encoder run time reduction with unchanged BD-rate by both the normative and non-normative methods when compared to HM7.0 anchor in HE10-AI. However, without transmitting the unnecessary transform skip flag in the first case and the second case, the CABAC parsing throughput can be somewhat increased. Therefore, we recommend the normative method.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Harbin Institute of Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, May, 2012.

- C. Lan, J. Xu, G. J. Sullivan, F. Wu, “Intra transform skipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0408, April, 2012.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.