| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0389 |
| --- | --- |

| Title: | Simplification for intra transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Jicheng An North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Debin Zhao Department of Computer Science and Technology, Harbin Institute of Technology, Harbin, China 150001 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {liang.zhao, jicheng.an, yuwen.huang, shawmin.lei} @mediatek.com Debin Zhao dbzhao@jdl.ac.cn |
| Source: | MediaTek Inc, Harbin Institute of Technology |  |  |

_____________________________

# Abstract

In this contribution, a limitation is proposed to speed up the intra transform skip mode decision on the encoder side as follows: For a 4x4 intra luma or chroma TU, an intra transform skip flag will be transmitted to the decoder only when the size of CU is 8x8 in luma and the partition mode is Intra_NxN. This limitation can be implemented normatively or non-normatively. However, the non-normative method will transmit unnecessary transform skip flags to the decoder. It is reported that the proposed methods can achieve 9% encoder run time reduction without noticeable BD-rate changes for both non-normative and normative methods, when compared with the HM7.0 anchor in HE10-AI condition.

# Introduction

In the current HEVC (text specification draft 7) [1], an intra transform skip mode proposed in JCTVC-I0408 [2] has been adopted to improve the coding efficiency, especially for the class F sequences. For a 4x4 intra luma or chroma transform unit (TU), the transform skip mode can be selected. For the transform skipping TUs, a simple scaling process is used instead of the transform.

For a 4x4 intra luma or chroma TU, regardless of the size of CU and the partition mode, a flag will be transmitted to the decoder to indicate whether the transform skip mode is used or not. In the common test conditions, QuadtreeTUMaxDepthIntra is set to 3. Therefore, for luma TU, there are three cases where the transform skip flag will be transmitted: 1) the size of CU is 16x16 in luma, and the size of TU is 4x4; 2) the size of CU is 8x8 in luma, the partition mode is Intra_2Nx2N, and the size of TU is 4x4; 3) the size of CU is 8x8 in luma, the partition mode is Intra_NxN, and the size of TU is 4x4. For chroma TU, besides the above three cases, there is an additional case as follows: 4) the size of CU is 32x32 in luma and the size of TU is 4x4.

# Proposed method

From our observation, most gain of the transform skip mode is obtained from the third case. Based on the observation, it is proposed to signal the transform skip flag of a luma or chroma TU only when the third case is satisfied, which requires normative changes of the current specification.

Clearly, this limitation can also be implemented as an encoder-only method. In this way, we set the transform skip flag of the 4x4 intra luma or chroma TU to 0 when the first, second, or fourth case is satisfied. Different from the normative modification, we have to transmit this flag to the decoder although it is always 0. Both methods can speed up the encoder because of the first, second, and fourth cases are not searched.

# Simulation results

The proposed methods have been implemented on top of HM-7.0. The simulation has been done according to the HE10 common test conditions [3], since the intra transform skip mode is only turned on in HE10 conditions. Results are shown in Table 1 and 2. It can be seen that the encoder complexity reduction of the normative and non-normative methods are both about 9%. However, without transmitting the unnecessary transform skip flag for the first, second, and fourth cases, the coding efficiency of the normative method is a little better than the non-normative method.

Table 1.  Results of the normative method

Table 2.  Results of the non-normative method

# Conclusion

In this contribution, a limitation is proposed to speed up the intra transform skip mode. This limitation is that, for a 4x4 intra luma or chroma TU, the intra transform skip mode can be enabled only when the size of CU is 8x8 in luma and the partition mode is Intra_NxN. It is reported that the proposed method can achieve 9% encoder run time reduction with unchanged BD-rate in a normative or non-normative way when compared to HM7.0 anchor in HE10-AI. However, without transmitting the unnecessary transform skip flags, the design is cleaner and makes more sense. Therefore, we recommend the normative method.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Harbin Institute of Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, May, 2012.

- C. Lan, J. Xu, G. J. Sullivan, F. Wu, “Intra transform skipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0408, April, 2012.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.