| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I1002 |
| --- | --- |

| Title: | HM7: High Efficiency Video Coding (HEVC) Test Model 7 Encoder Description |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ken McCann (Samsung/ZetaCast) Benjamin Bross (HHI) Il-Koo Kim (Samsung) Kazuo Sugimoto (Mitsubishi) Woo-Jin Han (Gachon University) | Email: | ken@zetacast.com benjamin.bross@hhi.fraunhofer.de ilkoo.kim@samsung.com Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp hurumi@gmail.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

The JCT-VC established a 7th HEVC test model (HM7) at its 9th meeting in Geneva from 27 April to 7 May 2012. This document serves as a source of general tutorial information on HEVC and also provides an encoder-side description of HM7.

CONTENTS

Page

Abstract	1

1	Introduction	5

2	Scope	6

3	Description of HEVC Test Model	6

3.1	General coding structure	6

3.2	Picture partitioning	7

3.2.1	CTU partitioning	7

3.2.2	Slice structure	8

3.2.3	Tile structure	8

3.2.4	Coding unit (CU) structure	8

3.2.5	Prediction unit (PU) structure	8

3.2.6	Transform unit (TU) structure	9

3.3	Intra Prediction	9

3.4	Inter Prediction	11

3.4.1	Prediction Modes	11

3.4.1.1	Derivation of motion merge candidates	12

3.4.1.2	Spatial merge candidates	12

3.4.1.3	Temporal merge candidates	13

3.4.1.4	Generated merge candidates	14

3.4.2	Motion vector prediction	14

3.4.2.1	Derivation of motion vector prediction candidates	14

3.4.2.2	Spatial motion vector candidates	15

3.4.2.3	Temporal motion vector candidates	15

3.4.3	Interpolation filter	15

3.4.4	Weighted Prediction	16

3.5	Transform and Quantization	16

3.5.1	Transform matrices	16

3.5.2	Scaling and quantization	17

3.6	Entropy Coding	18

3.7	Loop Filtering	18

3.7.1	Deblocking filter	18

3.7.1.1	Boundary decision	18

3.7.1.2	Boundary strength calculation	18

3.7.1.3	β and tC decision	19

3.7.1.4	Filter on/off decision for 4 lines	20

3.7.1.5	Strong/weak filter selection for 4 lines	20

3.7.1.6	Strong filtering	21

3.7.1.7	Weak filtering	21

3.7.1.8	Chroma filtering	21

3.7.2	Sample Adaptive Offset (SAO)	21

3.7.3	Adaptive loop filter (ALF)	22

3.7.3.1	Overview	22

3.7.3.2	Filter shapes	23

3.7.3.3	Luma pixel classification	23

3.7.3.4	Filtering control map	24

3.8	Wavefront parallel processing	24

4	Description of encoding methods	24

4.1	Cost Functions	24

4.1.1	Sum of Square Error (SSE)	24

4.1.2	Sum of Absolute Difference (SAD)	25

4.1.3	Hadamard transformed SAD (SATD)	25

4.1.4	RD cost functions	25

4.1.4.1	Lagrangian constant values	25

4.1.4.2	Weighting factor for chroma component	25

4.1.4.3	SAD based cost function for prediction parameter decision	26

4.1.4.4	SATD based cost function for prediction parameter decision	26

4.1.4.5	Cost function for mode decision	26

4.2	Encoder configurations	26

4.2.1	Overview of Encoder Configurations	26

4.2.2	High Efficiency (HE) coding	26

4.2.3	Low Complexity (LC) coding	26

4.3	Temporal Prediction Structure	26

4.3.1	Intra-only configuration	26

4.3.2	Low-delay configurations	27

4.3.3	Random-access configuration	27

4.4	Input bit depth modification	28

4.5	Slice partitioning operation	28

4.6	Tile partitioning operation	28

4.7	Derivation process for slice-level coding parameters	28

4.7.1	Sample Adaptive Offset (SAO) parameters	28

4.7.2	Adaptive Loop Filter (ALF) parameters	29

4.7.2.1	Derivation of luma pixel classifications	30

4.7.2.2	Derivation of filter coefficients and applying the filter	31

4.7.2.3	Encoding of filter coefficients	31

4.7.2.4	Derivation of ALF control map	31

4.7.2.5	Decision of luma pixel classification and filter coefficients by estimating filtering RD cost in 1-pass encoding	32

4.7.2.6	Decision of chroma component(s) to be filtered	32

4.7.3	Rounding control	32

4.8	Derivation process for CU-level and PU-level coding parameters	32

4.8.1	Intra prediction mode and parameters	32

4.8.2	Inter prediction mode and parameters	33

4.8.2.1	Derivation of motion parameters	33

4.8.2.2	Motion estimation	34

4.8.2.3	Decision process on AMP mode evaluation procedure	36

4.8.3	Intra/Inter/PCM mode decision	36

4.9	Derivation process for TU-level coding parameters	38

4.9.1	Residual Quad-tree partitioning	38

4.9.2	Rate-Distortion Optimized Quantization	38

5	References	42

- List of figures

Figure 3-1 – Example of a picture divided into LCUs	7

Figure 3-2 – Example of Coding Unit structure	8

Figure 3-3 – Four types of Prediction Unit structure [needs to add AMP figures]	9

Figure 3-4 – Example of Transform Unit structure	9

Figure 3-5 – The 33 intra prediction directions	10

Figure 3-6 – Mapping between intra prediction direction and intra prediction mode	11

Figure 3-7 – Derivation process for motion merge candidate	12

Figure 3-8 – Positions of spatial merge candidate	13

Figure 3-9 – Positions for the second PU of Nx2N and 2NxN partitions	13

Figure 3-10 – Illustration of motion vector scaling for temporal merge candidate	13

Figure 3-11 – Candidate positions for temporal merge candidate, C3 and H	14

Figure 3-12 – Overall processing flow of deblocking filter process	14

Figure 3-13 – Illustration of motion vector scaling for spatial motion vector candidate	15

Figure 3-14 – Overall processing flow of deblocking filter process	18

Figure 3-15 – Flow diagram for Bs calculation	19

Figure 3-16 – Referred information for Bs calculation at LCU boundary.	19

Figure 3-17 – Red boxes represent pixels involving in filter on/off decision and strong/weak filter selection.	20

Figure 3-18 – Four 1-D 3-pixel patterns for the pixel classification in EO; from left to right: 0-degree, 90-degree, 135-degree, 45-degree	22

Figure 3-19 – Flow-graph of encoder decision for ALF	23

Figure 3-20 – Filter shapes for both luma and chroma samples	23

Figure 3-21 – Illustration of filtering control map derivation	24

Figure 4-1 Graphical presentation of Intra-only configuration	27

Figure 4-2 Graphical presentation of Low-delay configuration	27

Figure 4-3 Graphical presentation of Random-access configuration	28

Figure 4-4 Three step motion search strategy for integer-pel accuracy	34

Figure 4-6 The schematic of Intra/Inter/PCM mode decision	38

Figure 4-7 – Graph structure for RDO quantization based on CABAC in H.264/AVC	39

Figure 4-8 – Possible quantized values in RDO-Q	40

- List of tables

Table 1-1 – Structure of Tools in HM6 Configurations	5

Table 5-1 – Maximum quadtree depth according to test scenario and prediction modes	9

Table 5-2 – Number of supported intra modes according to PU size	10

Table 5-3 – 8-tap DCT-IF coefficients for 1/4th luma interpolation	15

Table 5-4 – 4-tap DCT-IF coefficients for 1/8th chroma interpolation	16

Table 5-5 – Derivation of threshold variables β and tC from input Q (same table with Table 8-15 in WD)	20

Table 5-6 – Specification of SAO type	22

Table 5-7 – Pixel classification rule for EO	22

Table 6-1 – Derivation of Wk	25

# Introduction

The 7th HEVC test model (HM7) was defined by decisions taken at the 9th meeting of JCT-VC in Geneva from 27 April to 7 May 2012.  Two configurations have been defined: Main and High efficiency 10 (HE10). A summary list of the tools that are included in Main and HE10 is provided in Table 1-1 below.

Table 1-1 – Structure of Tools in HM7 Configurations

| Main |  |  | High efficiency 10 (HE10) |
| --- | --- | --- | --- |
| High-level Structure: |  |  |  |
| High-level support for frame rate temporal nesting and random access |  |  |  |
| Clean random access (CRA) support |  |  |  |
| Rectangular tile-structured scanning |  |  |  |
| Wavefront-structured processing dependencies for parallelism |  |  |  |
| Slices with spatial granularity equal to coding tree unit | Slices with spatial granularity possibly smaller than coding tree unit: 16x16 or larger |  |  |
| - | Entropy slices to support parallel parsing |  |  |
| Coding units, Prediction units, and Transform units: |  |  |  |
| Coding unit quadtree structure (square coding unit block sizes 2Nx2N, for N=4, 8, 16, 32; i.e., up to 64x64 luma samples in size) |  |  |  |
| Prediction units (for coding unit size 2Nx2N: for Inter, 2Nx2N, 2NxN, Nx2N, and, for N>4, also 2Nx(N/2+3N/2) & (N/2+3N/2)x2N; for Intra, only 2Nx2N and, for N=4, also NxN) |  |  |  |
| Transform unit tree structure within coding unit (maximum of 3 levels) |  |  |  |
| Transform block size of 4x4 to 32x32 samples (always square) |  | Transform block size of 4x4 to 32x32 samples (always square for Intra; also non-square 4x16, 16x4, 8x32, 32x8 for Inter) |  |
| Spatial Signal Transformation and PCM Representation: |  |  |  |
| DCT-like integer block transform; for Intra also a DST-based integer block transform (selected based on the intra prediction mode) |  |  |  |
| Transforms can cross prediction unit boundaries for Inter; not for Intra |  |  |  |
| PCM coding with worst-case bit usage limit |  |  |  |
| Intra-picture Prediction: |  |  |  |
| Angular intra prediction (35 directions ) |  |  |  |
| Planar intra prediction |  |  |  |
| - | Chroma intra prediction separate from or using luma samples |  |  |
| Inter-picture Prediction: |  |  |  |
| Luma motion compensation interpolation: 1/4 sample precision, 8x8 separable with 6 bit tap values |  |  |  |
| Chroma motion compensation interpolation: 1/8 sample precision, 4x4 separable with 6 bit tap values |  |  |  |
| Advanced motion vector prediction with motion vector “competition” and “merging” |  |  |  |
| Entropy Coding: |  |  |  |
| Context adaptive binary arithmetic entropy coding |  |  |  |
| RDOQ on |  |  |  |
| Picture Storage and Output Precision: |  |  |  |
| 8 bit-per-sample storage and output | 10 bit-per-sample storage and output |  |  |
| In-Loop Filtering: |  |  |  |
| Deblocking filter |  |  |  |
| Sample-adaptive offset filter |  |  |  |
| - |  |  | Adaptive loop filter |

At its 1st meeting, in April 2010, the JCT-VC defined a "Test Model under Consideration" (TMuC), which was documented in JCTVC-B204 [1]. At its 3rd meeting, in October 2010, the JCT-VC defined a 1st HEVC Test Model (HM1) [2]. The majority of the tools within HM1 were in the TMuC, but HM1 had substantially fewer coding tools and hence there was a substantial decrease in the computational resources necessary for encoding and decoding. Further optimizations of the HEVC Test Model in HM2 [3], HM3 [4], HM4 [5], HM5 [6], HM6 [7] and HM7 were specified at subsequent meetings, with each successive model achieving better performance than the previous in terms of the trade-off between coding efficiency and complexity.

# Scope

This document provides an encoder-side description of HEVC Test Model (HM), which serves as tutorial information of the encoding model implemented into the HM software. The purpose of this text is to share a common understanding on reference encoding methods supported in the HM software, in order to facilitate the assessment of the technical impact of proposed new technologies during the HEVC standardization process. Although brief descriptions of HEVC design are provided to help understanding of HM, the corresponding sections of the HEVC working draft [8] should be referred to for any descriptions regarding normative processes.  A further document [9] defines the common test conditions and software reference configurations that should be used for experimental works.

# Description of HEVC Test Model

## General coding structure

The HEVC standard adopts well-known block-based hybrid coding scheme that relies on advanced intra and motion-compensated prediction and transform coding with high-efficiency entropy coding. In contrast to previous video coding standards, it employs flexible quad-tree coding block partitioning structure that also enables efficient use of large and multiple sizes of coding, prediction, and transform blocks. It also employs improved intra prediction and coding, adaptive motion parameter prediction and coding, and adaptive in-loop filtering tools, and enhanced version of CABAC. A general block-diagram of HM encoder is depicted in Figure 3-1.

As depicted in Figure 3-1, input video is divided into multiples of coding tree unit (CTU). A CTU can be part of a slice or tile. All processing is done by CTU-basis and the best coding parameters including coding, prediction, and transform block partitioning structure and quantized transform coefficients are obtained and encoded. In intra prediction process, the best intra mode among total 35 modes (Planar, DC, and 33 angular directions) is selected and coded. In the process, mode dependent context sample smoothing is applied to increase prediction efficiency and three most probable modes (MPM) are utilized to increase symbol coding efficiency. Additionally, derived mode (DM) and linear prediction from luma mode (LM) are casted to improve efficiency of chroma coding and prediction. In inter prediction process, the best motion parameters are selected and coded by merge mode and AMVP mode, in which motion predictors are selected and coded among several candidates explicitly. To increase the efficiency of motion-compensated prediction, non-cascaded interpolation structure with 1D FIR filters are used. 8-tap and 7-tap filter are directly applied to generate the samples of half-pel and quarter-pel luma samples, respectively. 4-tap filter is utilized for chroma interpolation. Residuals generated by subtracting prediction from input are spatially transformed and quantized. In the transform process, matrices which are approximated by DCT are used. Smaller size matrices such as 16x16, 8x8, and 4x4 can be derived since those are embedded in the 32x32 matrix. Non-square transform can be used for non-square input residual blocks. Additionally, 4x4 DST can be applied depending on the intra mode for intra predicted residuals only. 52-level quantization steps are used in the quantization process. By inverse quantization and inverse transform, reconstructed samples are made and stored. After reconstruction, three in-loop filtering process (deblocking filtering, sample adaptive offset, adaptive loop filtering) are applied to achieve better coding efficiency and visual quality. Reconstructed CTU are assembled to construct a picture and stored into decode picture buffer to be used to encode the next picture of input video. Generated symbols and quantized transform coefficients in the encoding process are encoded by CABAC process. In Figure 3-1, other symbols except residual, prediction, and quantized transform coefficients are omitted for simplicity. Those symbols are assumed to be fed into entropy coding process properly. [Ed. Note: Needs improvement]

Figure 3-1 Block diagram of HM encoder

## Picture partitioning

### CTU partitioning

[Ed. Note: Revise text to ensure consistent use of terminology

- block = a rectangular 2D array (one component)

- unit = collective term for both luma and chroma. Only use ‘unit’ with prefix, e.g. coding unit, prediction unit or transform unit.]

Pictures are divided into a sequence of coding tree units (CTUs).  A CTU consists of an NxN block of luma samples together with two corresponding blocks of chroma samples for a picture that has three sample arrays, or an NxN block of samples of a monochrome plane in a picture that is coded using three separate colour planes. The CTU concept is broadly analogous to that of the macroblock in previous standards such as AVC [10]. The maximum allowed size of the luma block in a CTU is 64x64.

Figure 3-2 – Example of a picture divided into CTUs

[Ed. Note: Needs expansion]

### Slice structure

Slice is specified as a unit of packetization of coded video data for transmission purpose. Slices are designed to be independently decodable, so no prediction is performed across slice borders, and entropy coding is restarted between slices. A slice consists of a slice header followed by a series of successive coding units in coding order.  Slice boundaries can be located inside a CTU to ease encoder-side control for finding slice boundary that can maximize packetization efficiency. The level at which slice boundaries may be present is determined by the slice granularity parameter in the sequence parameter set. A slice granularity of 0 means that boundaries may only occur between CTUs, a granularity of 1 means that boundaries may be present after each quarter of a CTU, and so on.

Entropy slices are similar to slices, except in that prediction may be performed between entropy slices. If both regular slices and entropy slices are used, an entropy slice is always contained in a single slice, while a slice may contain several entropy slices.

### Tile structure

[Ed. Note: Needs description]

### Coding unit (CU) structure

The Coding unit (CU) is the basic unit of region splitting used for inter/intra coding. It is always square and it may take a size from 8x8 luma samples up to the size of the LCU.

The CU concept allows recursive splitting into four equally sized blocks, starting from the CTU.  This process gives a content-adaptive coding tree structure comprised of CU blocks, each of which may be as large as the CTU or as small as 8x8.

Figure 3-3 – Example of Coding Unit structure

Both skipped CU and non-skipped CU types are allowed. The skipped CU is considered to be an inter prediction mode without coding of motion vector differences and residual information. The non-skipped CU is assigned to one of two prediction modes, intra prediction and inter prediction.

[Ed. Note: Needs expansion]

### Prediction unit (PU) structure

The Prediction unit (PU) is the basic unit used for carrying the information related to the prediction processes. In general, it is not restricted to being square in shape, in order to facilitate partitioning which matches the boundaries of real objects in the picture. Each CU may contain one or more PUs, each of which may be as large as the CU or as small as 8x4 or 4x8 in luma block size. It should be noted that PU type NxN is allowed only when the corresponding CU size is greater than the smallest allowed CU size, which is denoted as SCUSize.

NOTE: The usage of 4x4 block as luma component of a PU can be enabled via a specific flag inserted into sequence parameter set, however, the current HM4 encoder sets its value to be “disable”.

Figure 3-4 – Four types of Prediction Unit structure [needs to add AMP figures]

[Ed. Note: Needs expansion] need to add AMP

### Transform unit (TU) structure

The Transform unit (TU) is the basic unit used for the transform and quantization processes.  TU shape depends on PU partitioning mode. When PU shape is square, TU shape is also square and it may take a size from 4x4 up to 32x32 luma samples. When PU shape is non-square, TU shape shall also be non-square and it may take a size 32x8,8x32,16x4,4x16 luma samples. The use of non-square TU is applied not only to luma component but also to chroma component, while the use in chroma is limited only when the luma TU size is 32x8 or 8x32. Each CU may contain one or more TUs, where multiple TUs may be arranged in a quadtree structure, as illustrated in Figure 3-5 below.

Figure 3-5 – Example of Transform Unit structure

[Ed. Note: Needs expansion]  RQT encoding process should be described in section 6.10. What needs to be put here?

The maximum quadtree depth is adjustable and is specified in the slice header syntax. The values are set according to the test scenario as shown in Table 3-2 below.

Table 3-2 – Maximum quadtree depth according to test scenario and prediction modes

| Test scenario | Maximum quadtree depth (for inter block) | Maximum quadtree depth (for intra block) |
| --- | --- | --- |
| Intra Only High-Efficiency | - | 3 |
| Random Access High-Efficiency | 3 | 3 |
| Low Delay High-Efficiency | 3 | 3 |
| Intra Only Low Complexity | - | 3 |
| Random Access Low Complexity | 3 | 3 |
| Low Delay Low Complexity | 3 | 3 |

## Intra Prediction

The unified intra prediction coding tool provides up to 35 directional prediction modes including DC and Planar modes for luma component of each PU. The total number of available prediction modes is dependent on the size of the corresponding PU, as shown in Table 3-3 below.

Table 3-3 – Number of supported intra modes according to PU size

| PU size | Number of intra modes |
| --- | --- |
| 4 | 35 |
| 8 | 35 |
| 16 | 35 |
| 32 | 35 |
| 64 | 35 |

The 33 possible intra prediction directions are illustrated in Figure 3-6 below.

Figure 3-6 – The 33 intra prediction directions

For PU sizes where less than the full set 35 total intra prediction modes are allowed, the first N directions according to the mapping between the intra prediction direction and the intra prediction mode number specified in Figure 3-7 below are used.

Figure 3-7 – Mapping between intra prediction direction and intra prediction mode

For chroma component of intra PU, (need description on chroma intra prediction mode)

[Ed. Note: Needs expansion]

## Inter Prediction

### Prediction Modes

Each inter coded PU shall have a set of motion parameters consisting of motion vector, reference picture index, reference picture list usage flag to be used for inter prediction sample generation, in an explicit or implicit way of signaling. When a CU is coded with skip mode (i.e., PredMode == MODE_SKIP), the CU shall be represented as one PU that has no significant transform coefficients and motion vectors, reference picture index and reference picture list usage flag obtained by motion merge. The motion merge is to find neighbouring inter coded PU such that its motion parameters (motion vector, reference picture index, and reference picture list usage flag) can be inferred as the ones for the current PU. Encoder can select the best inferred motion parameters from multiple candidates formed by spatial neighbouring PUs and temporally neighbouring PUs, and transmits corresponding index indicating chosen candidate. Not only for skip mode, the Motion Merge can be applied to any inter coded PU (i.e., PredMode == MODE_INTER). In any inter coded PUs, encoder can have freedom to use motion merge or explicit transmission of motion parameters, where motion vector, corresponding reference picture index for each reference picture list and reference picture list usage flag are signalled explicitly per each PU. For inter coded PU, significant transform coefficients are sent to decoder. The details are presented in following sections.

#### Derivation of motion merge candidates

Figure 3-8 – Derivation process for motion merge candidate

Figure 3-8 summarizes derivation process for motion merge candidates. 2 types of merge candidates are considered in motion merge: spatial merge candidate and temporal merge candidate. For spatial merge candidate derivation, 4 merge candidates are selected among candidates that are located in 5 different positions. In the process of candidate selection, redundant partition shape is avoided in order not to emulate virtual 2Nx2N partition by merging two 2NxN or two Nx2N partitions. For temporal merge candidate derivation, 1 merge candidate is selected among 2 candidates. After a list of spatio-temporal candidates is made, duplicated candidates which have same motion parameters in the list are removed in order to have distinctive candidates only. Since constant number of candidates for each PU is assumed at decoder, additional candidates are generated when the number of candidates does not reach to maximum number of merge candidate (MaxNumMergeCand) which is signalled in slice header. For B-Slices, combined bi-predictive and non-scaled bi-predictive candidates are generated utilizing the candidates from list of spatio-temporal candidates. For both P- and B-slices, zero merge candidates are added at the end of the list. Between each generation step, derivation process is stopped if the number of candidates reaches to MaxNumMergeCand. In current common test condition, MaxNumMergeCand is set equal to 5. Since the number of candidates is constant, index of best motion merge candidate is encoded using truncated unary binarization (TU).

#### Spatial merge candidates

In the derivation of spatial merge candidates, maximum 4 merge candidates are selected among candidates that are located in positions as depicted in Figure 3-9. The order of derivation is A1 à B1 à B0 à A0 à (B2). Position B2 is considered only when any PU of position A1 , B1 , B0 , A0 are unavailable or intra coded.

Figure 3-9 – Positions of spatial merge candidate

For the second PU of Nx2N, nLx2N and nRx2N partitions, position A1 is not considered as a candidate to prevent from 2Nx2N partition emulation. In these cases, the order of derivation is B1 à B0 à A0 à B2. Similarly, for the second PU of 2NxN, 2NxnU and 2NxnD partitions, position B1 is not used: A1 à B0 à A0 à B2. Figure 3-10 depicts example of candidate positions for the second PU of Nx2N and 2NxN, respectively.

Figure 3-10 – Positions for the second PU of Nx2N and 2NxN partitions

#### Temporal merge candidates

In the derivation of temporal merge candidate, scaled motion vector is derived based on co-located PU belongs to the picture which has the smallest POC difference with current picture within given reference picture list. The reference picture list to be used for derivation of co-located PU is signalled in slice header explicitly. Scaled motion vector for temporal merge candidate is obtained like dotted line in Figure 3-11, which is scaled from motion vector of co-located PU using the POC distance, tb and td. tb is defined as POC difference between reference picture of current picture and current picture. td is defined as POC difference between reference picture of co-located picture and co-located picture. Reference picture index of temporal merge candidate is set as reference picture index of PU at position A1. If PU of position A1 is not available or intra coded, the reference picture index is set equal to 0. Practical realization of scaling process is described in WD [8]. For B-slice, two motion vectors, one is for reference picture list 0 and the other is for reference picture list 1, are obtained and combined to make bi-predictive motion merge candidate.

Figure 3-11 – Illustration of motion vector scaling for temporal merge candidate

Position of co-located PU is selected between 2 candidate positions, C3 and H, as depicted in Figure 3-12. If PU at position H is not available or intra coded, or outside of current LCU, position C3 is used. Otherwise, position H is used for the derivation of temporal merge candidate.

Figure 3-12 – Candidate positions for temporal merge candidate, C3 and H

#### Generated merge candidates

Besides spatio-temporal merge candidates, there are additional three types of merge candidates which are generated utilizing spatio-temporal merge candidates: combined bi-predictive merge candidate, non-scaled bi-predictive merge candidate and zero motion merge candidates. Combined bi-predictive merge candidate and non-scaled bi-predictive merge candidate are used for B-Slice only. The maximum number of each candidate is limited to 5, 1 and same with number of reference index, respectively. [Ed: Needs expansion]

### Motion vector prediction

Motion vector prediction exploits spatio-temporal correlation of motion vector with neighbouring PUs, which is used for explicit transmission of motion parameters. It constructs motion vector candidate list by firstly checking availability of left, above temporally neighbouring PU positions, removing redundant candidates and adding zero vector to make the candidate list to be constant length as a normative process. Then, encoder can select the best predictor from the candidate list and transmits corresponding index indicating chosen candidate. Similarly with merge index signalling, index of the best motion vector candidate is encoded using truncated unary as maximum number is equal to 2. In the following sections, details about derivation process of motion vector prediction candidate are provided.

#### Derivation of motion vector prediction candidates

Figure 3-13 – Overall processing flow of deblocking filter process

Figure 3-13 summarizes derivation process for motion vector prediction candidate. In motion vector prediction, 2 types of motion vector candidates are considered: spatial motion vector candidate and temporal motion vector candidate. For spatial motion vector candidate derivation, 2 motion vector candidates are derived based on motion vectors of each PU located in 5 different positions. In the process of derivation, 1 motion vector candidate is selected utilizing PUs in the left side of current PU and 1 motion vector candidate is derived utilizing Pus in the above side of current PU. For temporal motion vector candidate derivation, 1 motion vector candidate is selected between 2 candidates, which are derived based on 2 different co-located positions. After the first list of spatio-temporal candidates is made, duplicated motion vector candidates in the list are removed. If the number of candidates is larger than 2, motion vector candidates whose index is larger than 1 are removed from the list. If the number of spatio-temporal motion vector candidates is smaller than 2, additional zero motion vector candidates is added to the list.

#### Spatial motion vector candidates

In the derivation of spatial motion vector candidates, maximum 2 candidates are considered among 5 candidates, which are derived from PUs located in positions as depicted in Figure 3-9. The candidate positions of motion vector prediction are same with those of motion merge. The order of derivation for left side of current PU is set as A0 à A1 à scaled A0 à scaled A1. The order of derivation for above side of current PU is set as B0 à B1 à B2 à scaled B0 à scaled B1 à scaled B2. For each side, there are 4 cases which can be used for motion vector candidate. Despite 2 cases are not required to do spatial scaling, the other 2 cases are required to do spatial scaling. 4 different cases are summarized as follows.

- No spatial scaling

  - (1) Same reference picture list, and same reference picture index (same POC)

  - (2) Different reference picture list, but same reference picture (same POC)

- Spatial scaling

  - (3) Same reference picture list, but different reference picture (different POC)

  - (4) Different reference picture list, and different reference picture  (different POC)

No spatial scaling cases are checked first and spatial scaling cases are checked sequentially. Spatial scaling is considered when POC is different between reference picture of neighbouring PU and that of current PU regardless of reference picture list. If all PUs of left candidates is not available or intra coded, scaling for above motion vector is allowed to help parallel derivation of left and above MV candidates. Otherwise, spatial scaling is not allowed for above motion vector.

Figure 3-14 – Illustration of motion vector scaling for spatial motion vector candidate

In a spatial scaling process, motion vector of neighbouring PU is scaled as same manner of temporal scaling as depicted as Error: Reference source not found. Main difference is that the reference picture list and index of current PU is given as input. Actual scaling process is same with that of temporal scaling.

#### Temporal motion vector candidates

Except reference picture index derivation, all process is same with the derivation of temporal merge candidate. The reference picture index is signalled to decoder.

### Interpolation filter

For the luma interpolation filter, an 8-tap separable DCT-based interpolation filter is used, as shown in Table 3-4.

Table 3-4 – 8-tap DCT-IF coefficients for 1/4th luma interpolation

| Position | Filter coefficients |
| --- | --- |
| 1/4 | { -1, 4, -10, 57, 19, -7, 3, -1 } |
| 2/4 | { -1, 4, -11, 40, 40, -11, 4, -1 } |
| 3/4 | { -1, 3, -7, 19, 57, -10, 4, -1 } |

Similarly, a 4-tap separable DCT-based interpolation filter is used for the chroma interpolation filter, as shown in Table 3-5.

Table 3-5 – 4-tap DCT-IF coefficients for 1/8th chroma interpolation

| Position | Filter coefficients |
| --- | --- |
| 1/8 | { -3, 60, 8, -1 } |
| 2/8 | { -4, 54, 16, -2 } |
| 3/8 | { -5, 46, 27, -4 } |
| 4/8 | { -4, 36, 36, -4 } |
| 5/8 | { -4, 27, 46, -5 } |
| 6/8 | { -2, 16, 54, -4 } |
| 7/8 | { -1, 8, 60, -3 } |

For the bi-directional prediction, the bit-depth of the output of the interpolation filter is maintained to 14-bit accuracy, regardless of the source bit-depth, before the averaging of the two prediction signals. The actual averaging process is done implicitly with the bit-depth reduction process as:

predSamples[ x, y ] = ( predSamplesL0[ x, y ] + predSamplesL1[ x, y ] + offset ) >> shift

where

shift = ( 15 – BitDepth ) and offset = 1 << ( shift – 1 )

[Ed. Note: Need to describe]

### Weighted Prediction

[Ed. Note: Needs description. (However, note that the WP has not been enabled in the common test conditions) ]

## Transform and Quantization

Transforms of sizes 4x4 to 32x32 are supported. The transform coefficients dij (i, j=0..nS-1) are derived from the transform matrix cij (i, j=0..nS-1) of subclause 3.5.1and the residual samples rij (i, j=0..nS-1) as specified in the following ordered steps.

- The intermediate sample values eij (i, j=0..nS-1) are derived as

eij 	=	ci0*rj0 + ci1*rj1 + … + cinS-1*rjnS-1 with i,j = 0,..,nS-1

- The intermediate sample values fij (i,j=0,..nS-1) are derived as

fij = ( eij + 2nS-10+BitDepth ) >> ( nS + BitDepth – 9), with i,j = 0,...,nS-1

- The unscaled transform coefficients gij (i, j=0..nS-1) are derived as

gij 	=	ci0*fj0 + ci1*fj1 + … + cinS-1*fjnS-1 with i,j = 0,..,nS-1

- The transform coefficients dij (i, j=0..nS-1) are derived as

dij = ( gij + 2nS+5  ) >> ( nS + 6 ), with i,j = 0,...,nS-1

### Transform matrices

This subclause specifies transform matrices cij (i, j=0..nS-1) for nS = 4, 8, 16, and 32.

nS = 4

{64, 64, 64, 64}

{83, 36,-36,-83}

{64,-64,-64, 64}

{36,-83, 83,-36}

nS = 8

{64, 64, 64, 64, 64, 64, 64, 64}

{89, 75, 50, 18,-18,-50,-75,-89}

{83, 36,-36,-83,-83,-36, 36, 83}

{75,-18,-89,-50, 50, 89, 18,-75}

{64,-64,-64, 64, 64,-64,-64, 64}

{50,-89, 18, 75,-75,-18, 89,-50}

{36,-83, 83,-36,-36, 83,-83, 36}

{18,-50, 75,-89, 89,-75, 50,-18}

nS = 16

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9}

nS = 32

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 90 88 85 82 78 73 67 61 54 46 38 31 22 13  4 -4-13-22-31-38-46-54-61-67-73-78-82-85-88-90-90}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90-90-87-80-70-57-43-25 -9  9 25 43 57 70 80 87 90}

{90 82 67 46 22 -4-31-54-73-85-90-88-78-61-38-13 13 38 61 78 88 90 85 73 54 31  4-22-46-67-82-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89 89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{88 67 31-13-54-82-90-78-46 -4 38 73 90 85 61 22-22-61-85-90-73-38  4 46 78 90 82 54 13-31-67-88}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87-87-57 -9 43 80 90 70 25-25-70-90-80-43  9 57 87}

{85 46-13-67-90-73-22 38 82 88 54 -4-61-90-78-31 31 78 90 61  4-54-88-82-38 22 73 90 67 13-46-85}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{82 22-54-90-61 13 78 85 31-46-90-67  4 73 88 38-38-88-73 -4 67 90 46-31-85-78-13 61 90 54-22-82}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80-80 -9 70 87 25-57-90-43 43 90 57-25-87-70  9 80}

{78 -4-82-73 13 85 67-22-88-61 31 90 54-38-90-46 46 90 38-54-90-31 61 88 22-67-85-13 73 82  4-78}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75 75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{73-31-90-22 78 67-38-90-13 82 61-46-88 -4 85 54-54-85  4 88 46-61-82 13 90 38-67-78 22 90 31-73}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70-70 43 87 -9-90-25 80 57-57-80 25 90  9-87-43 70}

{67-54-78 38 85-22-90  4 90 13-88-31 82 46-73-61 61 73-46-82 31 88-13-90 -4 90 22-85-38 78 54-67}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{61-73-46 82 31-88-13 90 -4-90 22 85-38-78 54 67-67-54 78 38-85-22 90  4-90 13 88-31-82 46 73-61}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57-57 80 25-90  9 87-43-70 70 43-87 -9 90-25-80 57}

{54-85 -4 88-46-61 82 13-90 38 67-78-22 90-31-73 73 31-90 22 78-67-38 90-13-82 61 46-88  4 85-54}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50 50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{46-90 38 54-90 31 61-88 22 67-85 13 73-82  4 78-78 -4 82-73-13 85-67-22 88-61-31 90-54-38 90-46}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43-43 90-57-25 87-70 -9 80-80  9 70-87 25 57-90 43}

{38-88 73 -4-67 90-46-31 85-78 13 61-90 54 22-82 82-22-54 90-61-13 78-85 31 46-90 67  4-73 88-38}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{31-78 90-61  4 54-88 82-38-22 73-90 67-13-46 85-85 46 13-67 90-73 22 38-82 88-54 -4 61-90 78-31}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25-25 70-90 80-43 -9 57-87 87-57  9 43-80 90-70 25}

{22-61 85-90 73-38 -4 46-78 90-82 54-13-31 67-88 88-67 31 13-54 82-90 78-46  4 38-73 90-85 61-22}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18 18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{13-38 61-78 88-90 85-73 54-31  4 22-46 67-82 90-90 82-67 46-22 -4 31-54 73-85 90-88 78-61 38-13}

{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9 -9 25-43 57-70 80-87 90-90 87-80 70-57 43-25  9}
{ 4-13 22-31 38-46 54-61 67-73 78-82 85-88 90-90 90-90 88-85 82-78 73-67 61-54 46-38 31-22 13 -4}

### Scaling and quantization

[Ed. note: The current description is non-RDOQ version. The text of section 6.7.2 should have alignment with this description.]

The quantized transform coefficients qij (i, j=0..nS-1) are derived from the transform coeficients dij (i, j=0..nS-1) as

qij = ( dij * f[QP%6] + offset  ) >> ( 29 + QP/6 – nS – BitDepth), with i,j = 0,...,nS-1

where

f[x] = {26214,23302,20560,18396,16384,14564}, x=0,…,5

228+QP/6–nS-BitDepth  < offset < 229+QP/6–nS-BitDepth

## Entropy Coding

Two entropy coding schemes are supported in the HM5: Context Adaptive Binary Arithmetic Coding (CABAC) and Variable Length Coding (VLC). Especially, CABAC is utilized for transform coefficient coding.

[Ed. Note: Needs expansion]

## Loop Filtering

### Deblocking filter

Deblocking filter process is performed for each CU with the same order as decoding process. Vertical edge is filtered (horizontal filtering) at first, and horizontal edge is filtered (vertical filtering) next. All filtering is applied to 8x8 block boundaries which is determined to be filtered for both luma and chroma components. 4x4 block boundaries are not processed in order to reduce the complexity, which is different from H.264/AVC.

Figure 3-15 – Overall processing flow of deblocking filter process

Figure 3-15 illustrates the overall processing flow of deblocking filter process. A boundary can have three filtering status: no filtering, weak filtering and strong filtering. Each filtering decision is based on boundary strength, Bs, and threshold values, β and tC.

#### Boundary decision

There could be three kinds of boundaries involving in the filtering: CU boundary, TU boundary and PU boundary. Union of theses boundaries are involved in the deblocking filter process. For example, CU boundaries, which are outer edges of CU, are always involved in the filtering since CU boundaries are always TU boundary or PU boundary. When PU shape is 2NxN (N > 4) and RQT depth is equal to 1, TU boundary at 8x8 block grid and PU boundary between each PU inside CU are involved in the filtering. (This doesn’t mean that all involved boundaries are filtered. Actual filtering decision is done in another process.) [Ed. Note: need to check following sentence] One except is that when PU boundary is inside TU, the boundary shall not be filtered.

#### Boundary strength calculation

Generally speaking, boundary strength (Bs) reflects how strong filtering is needed for the boundary. If Bs is large, strong filtering should be considered.

Figure 3-16 – Flow diagram for Bs calculation

Let’s define P and Q as blocks which are involved in the filtering. P represents the block located in left (vertical edge case) or above (horizontal edge case) side of the boundary. In a similar fashion, Q represents the block located in right (vertical edge case) or above (horizontal edge case) side of the boundary. Figure 3-16 illustrates how Bs value is calculated based on the intra coding mode, existence of non-zero transform coefficients and motion information, e.g., reference picture, number of motion vectors and motion vector difference. At first, Bs is calculated in 4x4 block basis, but it is re-mapped to 8x8 grid. Specifically, among two Bs which are belong to 8 pixels consisting a line in 4x4 grid, maximum Bs is selected as Bs for boundaries in 8x8 grid.

Figure 3-17 – Referred information for Bs calculation at LCU boundary.

In order to reduce line buffer memory requirement, only for LCU boundary, information in every second block (4x4 grid) in left or above side is re-used as depicted in Figure 3-17.

#### β and tC decision

Threshold values β and tC which involving in filter on/off decision, strong and weak filter selection and weak filtering process are derived based on luma quantization parameter of P and Q blocks, QPP and QPQ, respectively. Q used to derive β and tC is calculated as follows.

Q = ( ( QPP + QPQ + 1 ) >> 1 ).

A variables β is specified as Table 3-6 with Q as input. If Bs is greater than 1, the variable  tC is specified as Table 3-6 with Clip3(0, 55, Q + 2 ) as input. Otherwise (BS is equal or less than 1), the variable tC is specified as Table 3-6 with Q as input.

Table 3-6 – Derivation of threshold variables β and tC from input Q (same table with Table 8-15 in WD)

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |  |
| β | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | 64 | 64 | 64 | 64 |  |
| tC | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 |  |

#### Filter on/off decision for 4 lines

Filter on/off decision is done for four lines as a unit. Figure 3-18 illustrates the pixels involving in filter on/off decision. 6 pixels in the two red boxes for the first four lines are used to determine filter on/off for 4 lines.  6 pixels in two red boxes for the second 4 lines are used to determine filter on/off for the second four  lines.

Figure 3-18 – Red boxes represent pixels involving in filter on/off decision and strong/weak filter selection.

If dp0+dq0+dp3+dq3 < β, filtering for the first four lines is turned on and strong/weak filter selection process is applied.  Each variable is derived as follows.

dp0 = | p2,0 – 2*p1,0 + p0,0 |, dp3 = | p2,3 – 2*p1,3 + p0,3 |, dp4 = | p2,4 – 2*p1,4 + p0,4 |, dp7 = | p2,7 – 2*p1,7 + p0,7 |

dq0 = | q2,0 – 2*q1,0 + q0,0 |, dq3 = | q2,3 – 2*q1,3 + q0,3 |, dq4 = | q2,4 – 2*q1,4 + q0,4 |, dq7 = | q2,7 – 2*q1,7 + q0,7 |

If the condition is not met, no filtering is done for the first 4 lines. Additionally, if the condition is met, dE, dEp1 and dEp2 are derived for weak filtering process. The variable dE is set equal to 1.  If dp0 + dp3 < (β + ( β >> 1 )) >> 3, the variable dEp1 is set equal to 1. If dq0 + dq3 < (β + ( β >> 1 )) >> 3, the variable dEq1 is set equal to 1.

For the second four lines, decision is made in a same fashion with above.

#### Strong/weak filter selection for 4 lines

After the first four lines are determined to filtering on in filter on/off decision, if following two conditions are met, strong filter is used for filtering of the first four lines. Otherwise, weak filter is used for filtering. Involving pixels are same with those used for filter on/off decision as depicted in Figure 3-18.

1) 2*(dp0+dq0) < ( β >> 2 ), | p30 – p00 | + | q00 – q30 | < ( β >> 3 ) and | p00 – q00 | < ( 5* tC  + 1 ) >> 1

2) 2*(dp3+dq3) < ( β >> 2 ), | p33 – p03 | + | q03 – q33 | < ( β >> 3 ) and | p03 – q03 | < ( 5* tC  + 1 ) >> 1

As a same fashion, if following two conditions are met, strong filter is used for filtering of the second 4 lines. Otherwise, weak filter is used for filtering.

1) 2*(dp4+dq4) < ( β >> 2 ), | p34 – p04 | + | q04 – q34 | < ( β >> 3 ) and | p04 – q04 | < ( 5* tC  + 1 ) >> 1

2) 2*(dp7+dq7) < ( β >> 2 ), | p37 – p07 | + | q07 – q37 | < ( β >> 3 ) and | p07 – q07 | < ( 5* tC  + 1 ) >> 1

#### Strong filtering

For strong filtering, filtered pixel values are obtained by following equations. It is worth to note that three pixels are modified using four pixels as an input for each P and Q block, respectively.

p0’ = ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3

q0’ = ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3

p1’ = ( p2 + p1 + p0 + q0 + 2 ) >> 2

q1’ = ( p0 + q0 + q1 + q2 + 2 ) >> 2

p2’ = ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3

q2’ = ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3

#### Weak filtering

Let’s define  D as follows.

D = ( 9 * ( q0 –  p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4

When abs(D) is less than tC *10,

D = Clip3( - tC , tC , D )

p0’ = Clip1Y( p0 + D )

q0’ = Clip1Y( q0 - D )

If dEp1 is equal to 1,

Dp = Clip3( -( tC  >> 1), tC  >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 + D ) >>1 )

p1’ = Clip1Y( p1 + Dp )

If dEq1 is equal to 1,

Dq = Clip3( -( tC  >> 1), tC  >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 – D ) >>1 )

q1’ = Clip1Y( q1 + Dq )

It is worth to note that maximum two pixels are modified using three pixels as an input for each P and Q block, respectively.

#### Chroma filtering

Bs of chroma filtering is inherited from luma. If Bs > 1 or if coded chroma coefficient existing case, chroma filtering is performed. No other filtering decision is there. And only one filter is applied for chroma. No filter selection process for chroma is used. The filtered sample values p0’ and q0’ are derived as follows.

D = Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )

p0’ = Clip1C( p0 + D )

q0’ = Clip1C( q0 - D )

### Sample Adaptive Offset (SAO)

Sample adaptive offset is applied to the reconstruction signal after the deblocking filter by using the offset values given in the slice header. For luma samples, the encoder makes a decision on whether or not the SAO is applied for current slice. If SAO is enabled, the current picture allows recursive splitting into four sub-regions and each region can select one of six SAO types as shown in Table 5-5. SAO classifies reconstructed pixels into categories and reduces the distortion by adding an offset to pixels of each category in current region. Edge properties are used for pixel classification in SAO types 1-4, and pixel intensity is used for pixel classification in SAO types 5-6.

Table 3-7 – Specification of SAO type

| - SAO type | - sample adaptive offset type to be used | - Number of categories |
| --- | --- | --- |
| - 0 | - None | - 0 |
| - 1 | - 1-D 0-degree pattern edge offset | - 4 |
| - 2 | - 1-D 90-degree pattern edge offset | - 4 |
| - 3 | - 1-D 135-degree pattern edge offset | - 4 |
| - 4 | - 1-D 45-degree pattern edge offset | - 4 |
| - 5 | - central bands band offset | - 16 |
| - 6 | - side bands band offset | - 16 |

Band offset (BO) classifies all pixels of a region into multiple bands where each band contains pixels in the same intensity interval. The intensity range is equally divided into 32 intervals from zero to the maximum intensity value (e.g. 255 for 8-bit pixels), and each interval has an offset. Next, the 32 bands are divided into two groups. One group consists of the central 16 bands, while the other group consists of the rest 16 bands. Only offsets in one group are transmitted. Regarding the pixel classification operation in BO, the five most significant bits of each pixel can be directly used as the band index.

Edge offset (EO) uses four 1-D 3-pixel patterns for pixel classification with consideration of edge directional information, as shown in Figure 3-19. Each region of a picture can select one pattern to classify pixels into multiple categories by comparing each pixel with its two neighboring pixels. The selection will be sent in bit-stream as side information.  Table 5-6 shows the pixel classification rule for EO.

|  | C |  |  |  | C |  |  |  | C |  |  |  | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 3-19 – Four 1-D 3-pixel patterns for the pixel classification in EO; from left to right: 0-degree, 90-degree, 135-degree, 45-degree

Table 3-8 – Pixel classification rule for EO

| - Category | - Condition |
| --- | --- |
| - 1 | - c < 2 neighbors |
| - 2 | - c < 1 neighbor && c == 1 neighbor |
| - 3 | - c > 1 neighbor && c == 1 neighbor |
| - 4 | - c > 2 neighbors |
| - 0 | - None of the above |

It is desirable that the SAO on the decoder side can be operated LCU-independently so that the line buffers can be saved. In order to achieve this, pixels of the top and bottom rows in each LCU are not SAO processed when the 90-degree, 135-degree, and 45-degree classification patterns are chosen; pixels of the leftmost and rightmost columns in each LCU are not SAO processed when the 0-degree, 135-degree, and 45-degree patterns are chosen.

[Ed. Note: needs expansion]

### Adaptive loop filter (ALF)

#### Overview

An adaptive loop filter is applied to the reconstruction signal after the SAO process or deblocking filter process by using the filter coefficients given in adaptive parameter set (APS). For luma and chroma samples, the encoder makes a decision on whether ALF is applied in picture level or not. If picture-level ALF is enabled, the encoder additionally makes a decision whether ALF is applied in each LCU or not and the appropriate signalling flags (LCU-level on/off flags) are interleaved in the LCU data. The filter shapes for luma and chroma are the same and as shown in Figure 3-19. For luma component, filter adaptation is supported by using region-based adaptatioe (RA). By RA, Aa pcture is divided up i sixteen regions, where they can be merged during ALF coefficient design. In each merged region or ly one filter is used and no pixel classification scheme is applied. Overall

-

#### Filter shapes

The filtering process uses one 2D shape filters (9x7 cross + 3x3 rectangular shapes) for both luma and chroma samples as depicted in Figure 3-21Figure 3-21. The filtet for each pixes is selected from multiple filters bts’he pixel position (re) of e to symmetric property of the filters, only 10 coefficients are needed to represent a filter.

Figure 3-21 – Filter shapes for both luma and chroma samples

[Ed. Note: C2 and C4 are swapped in HM7 software, but WD7 follows Figure 3-21. (Cf. Ticket #574)]

#### Luma pixel classification

There is a pixel classification method in ALF, region-based (RA) classification. This classification is to divide one picture into 16 regions (4 horizontal divisions and 4 vertical divisions). Each region can be merged with each other for further optimization, and one filter is assigned for each merged region.

      - Filtering control map

Filtering control map consists of LCU-level on/off flags to decide whether ALF shall be applied on LCUs or not.filte After designing initial filter coefficients, filtering on/off decisions for each LCU is made to generate filtering control map. Filtering on/off is decided according to the distortion reduction of filtered results in one LCU, where the based on LCU boundarynumber of available pixels in one LCU is determined in encoding process.  Filter re-design process is performed 1 time for high efficiency mode using filtering on regions, and no re-design process is performed for low latency mode.

  - Wavefront parallel processing

WPP produces a bitstream that can be processed using one or more cores running in parallel. When WPP is used, the following operations can be performed by the encoder.

When the encoding of the second CTU in a CTU row is finished, the CABAC probabilities are stored in a buffer.

When starting the encoding of the first CTU in a CTU row, the following process shall be applied:

if the last CU of the second CTU of the row above is available, the CABAC probabilities are set to the values stored in the buffer.

if not, the CABAC probabilities are reset to the default values

When the encoding of the last CTU in a CTU row is finished, and the end of a slice has not been reached, CABAC is flushed and a byte alignment is performed.

Furthermore, entry point offsets shall be written in the slice header. Each CTU row in the slice shall have an entry point offset that indicates where the corresponding data starts in the slice data. Offsets are in byte units. When WPP is used, a slice that does not start at the beginning of a CTU row shall not finish after the last CTU in the same row. When a slice starts at the beginning of a CTB row, there is no constraint on where it finishes.

# Description of encoding methods

## Cost Functions

[Ed. note: Definitions of all cost functions used in the HM software and remaining sections should refer to relevant section number below that specifies cost function to be used]

Various cost functions are used in the HM software encoder to perform non-normative mode/parameter decisions. In this section, the cost functions actually used in the encoding process of the HM software are specified for reference in the remaining sections of this document.

### Sum of Square Error (SSE)

The difference between two blocks with the same block size is produced using

Diff(i,j) = BlockA(i,j) - BlockB(i,j)		(4-)

SEE is computed using the following equation:

SSE=i,jDiff(i,j)2		(4-)

### Sum of Absolute Difference (SAD)

SAD is computed using the following equation:

SAD=i,jDiff(i,j)		(4-)

### Hadamard transformed SAD (SATD)

Since the transformed coefficients are coded, an improved estimation of the cost of each mode can be obtained by estimating DCT with the Hadamard transform.

SATD is computed using:

SATD=(i,jDiffT(i,j))2		(4-)

The Hadamard transform flag can be turned on or off. SA(T)D refers to either SAD or SATD depending on the status of the Hadamard transform flag.

SAD is used when computing full-pel motion estimation while SA(T)D is used for sub-pel motion estimation.

### RD cost functions

#### Lagrangian constant values

In the HM encoder, lambda values that are used for cost computation are defined as

λmode=α∗Wk∗2((QP−12)3.0)		(4-)

λpred  =λmode																		(4-)

α=1.0−Clip3(0.0,0.5,0.05∗numberof)1.0						 		(4-)

, for referenced pictures

, for non-referenced pictures

Wkrepresents weighting factor dependent to encoding configuration and QP offset hierarchy level of current picture within a GOP, as specified in Table 4-9. Note that the value of Wkderived from Table 4-9 is further modified by multiplying 0.95 when SATD based motion estimation is used.

Table 4-9 – Derivation of Wk

| k | QP offset hierarchy level | Slice type | Referenced | Wk |
| --- | --- | --- | --- | --- |
| 0 | 0 | I | - | 0.57 |
| 1 | 0 | GPB | 1 | RA: 0.442 LD: 0.578 |
| 2 | 1, 2 | B or GPB | 1 | RA: 0.3536 * Clip3( 2.0, 4.0, (QP-12)/6.0 ) LD: 0.4624 * Clip3( 2.0, 4.0, (QP-12)/6.0 ) |
| 4 | 3 | B | 0 | RA: 0.68 * Clip3( 2.0, 4.0, (QP-12)/6.0 ) |

#### Weighting factor for chroma component

The following weighting parameter wchroma is used to derive lambda value λchromato be used for chroma-specific decisions in RDOQ, SAO and ALF process.

wchroma=2QP−QPchroma3														(4-)

With this parameter, λchromais obtained by

λchroma=λmodewchroma														(4-)

Note that the parameter wchroma is also used to define cost function to be used for mode decision in order to weight chroma part of SSE.

#### SAD based cost function for prediction parameter decision

The cost for prediction parameter decision Jpred,SAD is specified by the following formula.

Jpred,SAD =SAD + λpred * Bpred, 														(4-)

where Bpred specifies bit cost to be considered for making decision, which depends on each decision case. λpred and SAD are defined in the section 4.1.4.1 and 4.1.2, respectively.

#### SATD based cost function for prediction parameter decision

The cost for motion parameter decision Jpred,SATD is specified by the following formula.

Jpred,SATD =SATD + λpred * Bpred, 													(4-)

where Bpred specifies bit cost to be considered for making decision, which depends on each decision case. λpred and SATD are defined in the section 4.1.4.1 and 4.1.3, respectively.

#### Cost function for mode decision

The cost for mode decision Jmode is specified by the following formula.

Jmode =(SSEluma+ wchroma *SSEchroma)+ λmode * Bmode,									(4-)

where Bmode specifies bit cost to be considered for mode decision, which depends on each decision case. λmode and SSE are defined in the section 4.1.4.1 and 4.1.1, respectively.

## Encoder configurations

### Overview of Encoder Configurations

The HM encoder works with two sets of encoder configurations, designated High Efficiency (HE) and Low Complexity (LC), as defined in the JVC-VC test condition document [9].

### High Efficiency (HE) coding

Coding tools for HE coding configuration are chosen to obtain high compression performance as its primary target. It supports a bit depth increase up to 10 bit, full capability of loop-filtering process including ALF and CABAC as entropy coder.

### Low Complexity (LC) coding

Coding tools for LC coding configuration are chosen to obtain reasonably high compression performance while keeping codec complexity to be low. Compared with HE coding tools, main differences are no bit depth increase, no support of ALF and the use of CAVLC as its entropy coding.

## Temporal Prediction Structure

The HM encoder works with three kinds of temporal prediction structures depending on experimental conditions, as defined in the JVC-VC test condition document [9]. Reference picture list management should depend on each temporal configuration.

### Intra-only configuration

In the test case for Intra-only coding, each picture in a video sequence shall be encoded as IDR picture. No temporal reference pictures shall be used. It is not allowed to change QP during a sequence within a picture. Figure 4-22 gives graphical presentation of Intra-only configuration. The number associated with each picture represents encoding order.

Figure 4-22 Graphical presentation of Intra-only configuration

### Low-delay configurations

Two kinds of low-delay coding configurations have been defined for testing coding performance in low-delay mode. For these low-delay coding conditions, only the first picture in a video sequence shall be encoded as IDR picture. In mandatory low-delay test condition, the other successive pictures shall be encoded as Generalized P and B-picture (GPB). The GPB shall be able to use only the reference pictures, each of whose POC is smaller than the current picture (i.e., all reference pictures in RefPicList0 and RefPicList1 shall be temporally previous in display order relative to the current picture). The contents of RefPicList0 and RefPicList1 shall be identical, and they shall be updated with sliding-window management process. Reference picture list combination is used for management and entropy coding of reference picture index. Figure 4-23 shows graphical presentation of Low-delay configuration that is mandatory for performance evaluation in any CEs. The number associated with each picture represents encoding order. QP of each inter coded picture shall be derived by adding offset to QP of Intra coded picture depending on temporal layer. In the additional non-normative low-delay condition, all inter pictures shall be coded as P-picture, where only the content of RefPicList0 is used for inter prediction.

Figure 4-23 Graphical presentation of Low-delay configuration

### Random-access configuration

For the random-access test condition, hierarchical B structure shall be used for coding. Figure 4-24 shows graphical presentation of Random-access configuration. The number associated with each picture represents encoding order. Intra picture shall be inserted cyclically per about one second. The first intra picture of a video sequence shall be encoded as IDR picture and the other intra pictures shall be encoded as non-IDR intra pictures (“Open GOP”). The pictures located between successive intra pictures in display order shall be encoded as B-pictures. The GPB picture shall be used as the lowest temporal layer that can refer to I or GPB picture for inter prediction. The second and third temporal layers consists of referenced B pictures, and the highest temporal layer contains non-referenced B picture only. QP of each inter coded picture shall be derived by adding offset to QP of Intra coded picture depending on temporal layer. Reference picture list combination is used for management and entropy coding of reference picture index.

Figure 4-24 Graphical presentation of Random-access configuration

## Input bit depth modification

When the Main configuration is used for a 10-bit source, each 10-bit source sample x is converted prior to encoding to an 8-bit value (x+2) / 4 clipped to the [0,255] range. Similarly when the HE10 configuration is used for an 8-bit source, each 8-bit source sample x is converted prior to encoding to a 10-bit value 4*x. This behaviour is built into the reference encoder and no external conversion program is required [9].

## Slice partitioning operation

The HM encoder can configure the depth in an LCU at which slices may begin and end, provided that CUs at the given depth are of size 16x16 or larger. For 64x64 LCUs, this means that the maximum allowed depth is 2 since that gives a CU of 16x16 which is the smallest allowed.

The HM encoder has two ways of determining slice size. One is to specify the maximum number of CUs at the determined depth in a slice. The other is to specify the number of bytes in a slice.

The HM encoder has also an option to enable entropy slice coding. In this case, the size of a slice can be specified as the maximum number of CUs in a slice, or by the number of bytes (CAVLC case) or bins (CABAC case) in a slice.

## Tile partitioning operation

[Ed. note: need description on what kind of tile partitioning operation is available in HM5 encoder.]

## Derivation process for slice-level coding parameters

### Sample Adaptive Offset (SAO) parameters

In the HM encoder, the following process is performed to determine the SAO parameters:

- Decide the maximum partitioning level-L according to picture size.

- Divide a picture into multiple regions with LCU-aligned boundaries according to partitioning rules.

- Collect the statistical information of all SAO types in each region.

- Derive the SAO parameters of all SAO types in each region at level-k.

- Select the SAO type with the lowest rate-distortion (RD) cost for current region at level-k.

- Combine the statistical data of child regions at level-k into one parent region at level-(k-1).

- Decide whether the child regions are merged into parent region according the RD-cost.

- Run step 4, 5, 6, and 7 form level-L to level-0 (level-0 represent that a picture is a sigle partition).

- Update luma DPB when the offsets of Luma component are added.

First, the the maximum partitioning level L is determinated. Second, the encoder divides a picture into regions according to the region partition rules as follows:

Divide the current region into four regions, upper-left, upper-right, down-left, and down-right with following parameters.

  - Wp is the number of LCUs in parent region width.

  - Hp is the number of LCUs in parent region height.

  - Wul is width of the upper-left region which contains floor(Wp/2) LCUs.

  - Hul is height of the upper-left region which contains floor(Hp/2) LCUs.

  - Wur is width of the upper-right region which contains (Wp-floor(Hp/2)) LCUs.

  - Hur height of the upper-right region which contains floor(Hp/2) LCUs.

  - Wdl is width of the down-left region c which contains floor(Wp/2) LCUs.

  - Hdl height of the down-left region which contains (Hp- floor(Hp/2)) LCUs.

  - Wdr is width of the down-left region which contains (Wp- floor(Wp/2)) LCUs.

  - Hdr  is height of the down-left region which contains (Hp- floor(Hp/2)) LCU.

Further divide each region into four regions according to the previous rule until the maxmun level-L is reached.

According the result of region partitioning, the statistical information can be collected and the estimated distortion reduction can be obtained by using the statistical information. The variables N, O, and E are described as follows. Nl,i,t,c  represents the number of pixels in level-l, region i, type t, and category c. Ol,i,t,c represents the corresponding offset value to be added for the categoried pixels. El,i,t,c represents sum of difference between original signal and reconstructed signal. Rl,i,t represents the estimated bits of corresponding offset.The estimated distortion reduction Dl,i,t for level-l, region-i and type-t can be calculated by the following equation

Dl,i,t=c∈tNl,i,t,cO2l,i,t,c−2Ol,i,t,cEl,i,t,c	,												(6-14)

where c is the number of categories in SAO type-t. After the estimated distortion reduction is obtained, we can calculate the estimated RD-cost by using equation (6-14), where λ is the Lagrange multiplier.

Jl,i,t=Dl,i,t+λRl,i,t																	(6-15)

The encoder will choose one SAO type with the lowest estimated RD-cost (6-15) in level-l and region-i according to equation (6-17), where T is a set of SAO type which can be selected for current region. Note that the estimated RD-cost should be less than zero because the D means delta distortion; otherwise, the current region will disable SAO processing.

Jl,i=mint∈T(Jl,i,t)																		(6-16)

tl,i=argmint∈T(Jl,i,t)																	(6-17)

After the best SAO type with the minimum cost for each region is obtained, the encoder will try to merge four sub-regions into one merged region according to equation (6-18) and (6-19), where J'l,i and t'l,i represent the updated RD-cost and the updated SAO type after merging. Note that Ωl,i represents a set of sub-regions in level-l and region-i.

J'l,i=min(k∈Ωl,iJl+1,k,Jl,i)																(6-18)

t'l,i=argmint∈T(k∈Ωl,iJl+1,k,Jl,i)															(6-19)

According the merge result of each region, the spilt flag of each region is determined by (6-20).

sl,i=1,ifk∈Ωl+1,iJl+1,k<Jl,i0,otherwise,															(6-20)

where sl,i  equals to 1 represent that current region will be spiltted into sub-regions, otherwise the current region will be a leaf region without splitting. After all regions in current level are processed, the encoder will process merge in each level for level-L to level-0. Accoding to the split flags, the encoder can decide that current is a leaf region or not. Finally, the SAO parameters of all leaf regions can be obtained and the corresponding offset values can be added to the reconstructed signal to update Luma DPB.

### Adaptive Loop Filter (ALF) parameters

[Ed. Note: Need to describe

- Encoding of ALF parameters that refers to CU split status

- For LC test, this process shall not be invoked and the syntax element for ALF does not exist in coded bitstream.]

[Ed. Note: Need to align expression of cost functions used in ALF encoding process with the one defined in section 6.1.]

In the HM encoder, two kinds of encoding algorithms in ALF are supported. One is high efficiency mode to provide the best coding efficiency and the other is low latency mode to achieve low encoding latency. The former uses the statistics of current frame to do filter coefficient derivation and the iterations of filter coefficients redesign are allowed. The latter uses the statistics of previous frame to perform filter coefficient derivation and there is no filter coefficient redesign to achieve LCU-level encoding latency. Overall encoder decision process is illustrated in . The first stage is filter coefficient derivation as specified in 4.7.2.2 based on the statistics of a previous or current picture. The second stage is picture-level filter selection. In this stage, up to eight previous APS can be reused by current picture, and the best one will be selected to be the final filter coefficients for current picture. Next, according to filter coefficients, picture-level ALF on/off decision is made. And if picture-level ALF is enabled, LCU-level on/off decision is performed to decide LCU-level on/off flags for each LCU and ALF control map is generated. The last one is to re-do picture-level ALF on/off decision again. That is, after LCU-level on/off decision, if ALF cannot improve the coding performance for current picture, then picture-level ALF on/off will be disabled.

In the HM encoder, the ALF parameters for luma/chroma component can be determined by 2 (high efficiency mode) or 1 (low latency mode) filtering passes through the encoder configuration. The following process is performed to determine ALF parameters for luma/chroma components if 2-pass encoding scheme is configured:

- In the high efficiency mode, redesigning filter coefficients is allowed and the number of iterations in filter coefficients re-design is set equal to 3. Moreover, the last stage is enabled in high efficiency mode. However, in order to achieve LCU-level encoding latency, redesigning filter coefficients and re-doing picture-level ALF on/off decision are not allowed in low latency mode.  In the following subsections, the details in the encoding algorithm are introduced. Section 4.7.2.1 introduces region-based adaptation for the luma component. Filter coefficient derivation and coefficient coding are presented in Section 4.7.2.2 and 4.7.2.3, respectively. And the last one is LCU-level on/off decision in Section 4.7.2.4.

.There are two kinds of encoding modes; high efficiency mode and low latency mode. In high efficiency mode, ALF coefficient design and LCU level on/off control can use the current frame data and one additional redesign is applied. In low latency mode, ALF coefficient design and LCU level on/off control use only previous frames’ data

#### Derivation of luma/chroma pixel classifications

The category derivation in the RA mode for one 4x4 block with the top-left pixel position [xC, yC] is shown in the following equations.

fIdx[ xC ][ yC ] = region_tab[ (y_idx << 2) + x_idx ]

where

region_tab[16] = {0, 1, 4, 5, 15, 2, 3, 6, 14, 11, 10, 7, 13, 12,  9,  8}

x_interval = ((((PicWidthInSamplesL + (1<<Log2MaxCUSize) – 1 )) >> Log2MaxCUSize ) + 1) >> 2 )

y_interval = ((((PicHeightInSamplesL + (1<<Log2MaxCUSize) – 1 )) >> Log2MaxCUSize) + 1) >> 2 )

x_idx = (xInterval == 0) ? 3 : min (3, (xC) / (x_interval<< Log2MaxCUSize) )

y_idx = (yInterval == 0) ? 3 : min (3, (yC) / (y_interval<< Log2MaxCUSize) )

where PicWidthInSamplesL and PicHeightInSamplesL are picture width and height in Luma, respectively, and 1<<Log2MaxCUSize is the max size of the coding unit.

Figure 4-25 – Flow-graph of encoder decision for ALF

#### Derivation of filter coefficients

Filter coefficients for each class are derived based on Wiener-based filter design.  A cross-correlation matrix Cj,0 between the original pixels and the reconstructed pixels and an auto-correlation matrix Ai,j of the reconstructed pixels are created for each class for the range of tap length of the filter.  All pixels in a slice or pixels that are marked as filtered (by alf_cu_flag) during the re-design process of filter coefficients are used to calculate the correlation matrix.

Based on the correlation matrix, filter coefficients for each class are derived by solving the following simultaneous equations:

i=0N−1wiAi,j=Cj,0 for j = 0 .. N-1															(4-)

where N (=10) is the number of filter coefficients, wi is the filter coefficients to be derived.

Estimated distortion is calculated for each class according to the following equation:

D=ijwiwjAi,j−2iwiCi,0														(4-)

When deriving luma filter coefficients, class merging is performed by deriving filter coefficients and estimating the distortion (for all pixels in a slice or pixels that are marked as filtered).

      - Encoding of filter coefficients

Filter coefficients for luma/chroma are encoded directly except the center coefficient.  Coefficients of the luma filter are encoded directly by signed exponential golomb code with parameter k.  Coefficients of the chroma filter are encoded directly by signed exponential golomb code.

#### Derivation of ALF control map

ALF control map consists of LCU-level on/off flags which are used to indicate whether ALF shall be applied on thUs or not for mu

The following two SSEs of each LCU are estimated (section 4.7.2.2) or calculated by the followings:

SSErecon = (org(i,j) – recon(i,j))2

SSEalf = (org(i,j) – filt(i,j))2

where org(i,j), recon(i,j) and filt(i,j) correspond to the original pixels, the reconstructed pixels and the filtered pixels.  If SSEalf is smaller than SSErecon, the block is marked as filtered (i.e. ctb_alf_flag[cIdx] for the LCU is set to 1 for component index cIdx).  Otherwise, the block is marked as not-filtered (i.e. ctb_alf_flag[cIdx] for the LCU is set to 0 for component index cIdx).

In this derivation process, the number of re-design process of filter coefficients is based on the encoder configuration mode (high efficiency mode or low latency mode).

#### Rounding control

[Editor’s note: Text of C253 should be summarized here.]

## Derivation process for CU-level and PU-level coding parameters

### Intra prediction mode and parameters

The unified intra coding tool provides up to 34 angular and planar prediction modes for luma component of different PUs. With the PU size of 4x4, 8x8, 16x16, 32x32 and 64x64, there are 17, 34, 34, 34 and 34 angular prediction modes available respectively for luma. The best intra prediction mode for luma component of each PU is derived as follows. Firstly, a rough mode decision process is performed. Prediction cost Jpred,SATD specified in the section 4.1.4.4 is computed for all possible prediction modes and pre-determined number of intermediate candidates are found per each PU size (8 for 4x4-8x8 PU, 3 for other PU sizes) resulting in least prediction costs. In this rough decision process, number of coded bits for intra prediction mode is set to Bpred. Then, RD optimization using the coding cost Jmode specified in the section 4.1.4.5, is applied to the candidate modes selected by the rough mode decision and MostProbableMode. During this RD decision, prediction parameters and coefficients for luma component of the PU are accumulated into Bmode. Concerning chroma mode decision, all possible intra chroma prediction modes are evaluated through RD decision process, where coded bits for intra chroma prediction mode and chroma coefficient are used as Bmode.

### Inter prediction mode and parameters

#### Derivation of motion parameters

In the HM encoder, an inter-coded CU can be segmented into multiple inter PUs, each of which has a set of motion parameters consisting of more than one motion vectors (per each RefPicListX), corresponding reference picture indices(ref_idx_lX) and prediction direction index (inter_pred_flag). Note that the current common test conditions those includes inter prediction coding are adopting reference picture list combination process, which is the case “X=c”. An inter-coded CU can be encoded with one of the following coding modes (“PredMode”): MODE_SKIP, MODE_INTER. For MODE_SKIP case, any sub-partitioning to smaller PUs is not allowed and its motion parameters are assigned to the CU itself, where the PU size is PART_2Nx2N. On the contrary, up to eight types of further partitioning to smaller PUs can be allowed for a CU coded with MODE_INTER. The PredMode and the CU partitioning shape (“PartMode”) are signaled by a CU level syntax element “part_type” as specified in Table 7-10 of the WD. For a MODE_INTER CU other than those having maximum depth, seven PU partitioning patterns (PART_2Nx2N, PART_2NxN, PART_Nx2N, PART_2NxnU, PART_2NxnD, PART_nLx2N and PART_nRx2N) can be selected. PART_NxN can only be chosen at maximum CU depth level but permission to set N to 4 is controlled by a specific flag in SPS(“inter_4x4_enabled_flag”). For each PU, PU-based Motion Merging (merge mode) or normal inter prediction with actually estimated motion parameters (inter mode) can be used. This section describes how luma motion parameters are obtained for each PU. It is noted that chroma motion vector shall be derived from luma motion vector of corresponding PU according to the normative process specified in section 8.4.2.1.10 of the WD, and the same reference picture index and prediction direction index as luma’s one shall be used in chroma components.

##### Motion Vector Prediction

For each PU, the best motion vector predictor is computed with the process specified as follows. Firstly, a set of motion vector predictor candidates for RefPicListX shall be derived with normative process specified in section 8.4.2.1.7 of the WD, by referring to motion parameters of neighbouring PUs. Then, the best one from the candidate set is determined by a criterion that selects a motion vector predictor candidate that minimizes the cost Jpred,SAD specified in the section 4.1.4.3, with setting the bits for an index specifying each motion vector predictor candidate to Bpred. The index corresponding to the selected best candidate is assigned to the mvp_idx_lX.

##### CU coding with MODE_SKIP

In the case of skip mode (i.e., PredMode == “MODE_SKIP”), motion parameters for the current CU(i.e., PART_2Nx2N PU) are derived by using merge mode. In this case, the motion parameters are determined by checking all possible merge candidates derived by the normative process specified in section 8.4.2.1.1 to 8.4.2.1.5 of the WD, and selecting the best set of motion parameters that minimizes the cost Jmode specified in the section 4.1.4.5. In this case, Bmode includes coded bits for skip_flag and merge_idx that signals position of the PU having the best motion parameters to be used for the current PU. Since prediction residual is not transmitted for skip mode, SSE is obtained by inter prediction samples.

##### CU coding with MODE_INTER

When a CU is coded with MODE_INTER, motion parameter decision for each PU is performed first based on the ME cost Jpred,SATD specified in the section 4.1.4.4.

For merge mode case, the motion parameter decision starts with checking availabilities of all neighbouring PUs to form merge candidates according to the normative process specified in the section 8.4.2.1.1 to 8.4.2.1.5 of the WD. If there is no available merge candidate, the HM encoder simply skips cost computation for merge mode and does not choose merge mode for the current PU. Otherwise(i.e., if there is at least one merge candidate), the ME cost Jpred,SATD specified in the section 4.1.4.4 is computed for all possible PUs as merge candidate and the best one is selected as the best motion parameters for the PU predicted with merge mode. SATD between source and prediction samples is used as distortion factor, and bits for merge_idx is set to Bpred.

For inter mode case, the best motion parameters are derived by invoking motion estimation process specified in the section 6.9.2.2. During the motion estimation process, the best motion parameters are obtained based on the cost function Jpred,SATD specified in the section 4.1.4.4, which is comparable with the cost of motion parameter derivation for merge mode. SATD between source and prediction samples is used as distortion factor, and bits for inter_pred_flag, ref_idx_lX, mvd_lX and mvp_idx_lX are set to Bpred.

After both of the best motion parameters are obtained, the best motion parameters are determined by comparing them and taking the better one that results in lower cost.

#### Motion estimation

In order to get motion vector for each PU, block matching algorithm (BMA) is performed at encoder. Motion vector accuracy supported in HEVC is quarter-pel. To generate half-pel and quarter-pel accuracy samples, interpolation filtering is performed for reference picture samples. Instead of searching all the positions for quarter-pel accuracy motion, integer-pel accuracy motion vector is obtained at first. For half-pel search, only 8 sample points around the motion vector which has the minimum cost are searched. Similarly, for quarter-pel search, 8 sample points around the motion which has the minimum cost so far are searched. The motion vector which has the minimum cost is selected as the motion vector of the PU. To get the cost, SAD is used for integer-pel motion search and SA(T)D is used for half-pel and quarter-pel motion search. The rate for motion vector is obtained by utilizing pre-calculated rate table. In the following sub-sections, algorithms for integer-pel motion search is provided in detail.

##### Integer-pel accuracy motion search

To reduce search points for integer-pel motion, 3 step motion search strategy is used. Figure 4-26  illustrates 3 step approach for integer-pel accuracy motion search.

Figure 4-26 Three step motion search strategy for integer-pel accuracy

At first, start position of the search is selected. As a default, motion vector predictor (PMV) obtained by motion vector predictor derivation process is used. Optionally, motion vectors of neighbouring positions (A, B, and C), and zero motion can be checked. In the common test condition, only PMV is used as the start position of integer-pel search.

Figure 4-27 Start position selection

As a second step, the first search is done using diamond search pattern or square search pattern. Currently, diamond search pattern is default, and square search pattern is used by changing input configuration. Additional raster search is performed when the difference between obtained motion vector and start position is too big. Currently, search range is set by 64 in integer-pel accuracy. Figure 4-28 illustrates 3 search patterns used for the first search. Red circles represent current position and coloured squares represent candidate search positions for each pattern. Same colour means positions having same distance from the start position.

Figure 4-28 Search patterns for the first search

Last step is refinement search. In this step, refinement search is performed by changing the start position to the best position from the second step. Also, diamond or square search is utilized, and refinement is stopped when 2 rounds are passed after best match.

##### Bi-predictive search strategy

In principle, bi-predictive motion search means to search two motion vectors which produce minimum error between original block (O) and predicted block with two prediction (P=P0+P1). In HM, practical strategy is implemented by utilizing iterative uni-predictive search. Bi-predictive search steps are as follows.

- Search P1 which produces minimum error with (2O - P0), where O represents original block and P0 means predictor produced by the first motion vector. P0 is fixed in this step. To get motion vector for P1, uni-predictive motion search is utilized after setting (2O - P0) as reference samples.

- Search P0 which produces minimum error with (2O – P1), where O represents original block and P1 means predictor produced by the second motion vector. P1 is the predictor obtained in step 1) and fixed in this step. To get P0, uni-predictive search is utilized after setting (2O – P1) as reference samples.

- Iterate 1) and 2) until maximum number of iterations is reached. The maximum number of iteration is set by 4 unless the fast search option is enabled.

##### Fast search options

There are two options to accelerate motion estimation. The first one is using sub-sampled SAD for integer motion search. This option is only used for blocks which have larger number of rows than 8. In this method, only samples in odd-number rows are involved for SAD calculations. The second option is reducing number of iterations for bi-predictive motion search. The number of iteration is set by 4 unless the fast search option is enabled.

#### Decision process on AMP mode evaluation procedure

For encoder speed up, additional conditions are checked before doing motion estimation for AMP. If the certain conditions are met, additional motion estimation for AMP can be skipped. Conditions of mode skipping are based on the two values: the best partition mode (PartMode) before AMP modes are evaluated and the PartMode and prediction mode (PredMode) at lower level in CU quad-tree, so called, parent CU, which contains current PU. Conditions and actions are specified in Table 4-2. Related contribution is [12].

Table 4-10 – Conditions and actions for fast AMP mode evaluation

| Conditions | Actions |
| --- | --- |
| The best PartMode is SIZE_2NxN | Try SIZE_2NxnU and SIZE_2NxnD |
| The best PartMode is SIZE_Nx2N | Try SIZE_nLx2N and SIZE_nRx2N |
| The best PartMode is 2Nx2N && !merge mode && ! skip mode | Try all AMP modes |
| PartMode of parent CU is AMP mode | Try merge mode only for all AMP modes |
| PartMode of parent CU is PART_2Nx2N && parent CU is not skipped | Try merge mode only for all AMP modes |
| PredMode of parent CU is intra && the best PartMode is SIZE_2NxN | Try merge mode only for SIZE_2NxnU and SIZE_2NxnD |
| PredMode of parent CU is intra && the best PartMode is SIZE_Nx2N | Try merge mode only for SIZE_nLx2N and SIZE_nRx2N |
| Size of current CU is 64x64 | No AMP modes are evaluated |

### Intra/Inter/PCM mode decision

For inter coded CUs, the following mode decision process is conducted in the HM encoder. Its schematic is also shown in Figure 4-29.  Please refer to contributions about early termination for Early_CU condition, CBF_Fast condition, and Early_SKIP condition [12][13][14].

- Coding costs (Jmode) for MODE_INTER with PART_2Nx2N is computed and Jmode is set to minimum CU coding cost J.

- Check if motion vector difference of MODE_INTER with PART_2Nx2N is equal to (0, 0) and MODE_INTER with PART_2Nx2N contains no non-zero transform coefficients (Early_SKIP condition). If both are true, proceed to 17 with setting the best interim coding mode as MODE_SKIP. Otherwise, proceed to 3.

- Check if MODE_INTER with PART_2Nx2N contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_2Nx2N. Otherwise, proceed to 4.

- Jmode for MODE_SKIP is evaluated and J is set equal to Jmode if Jmode < J.

- Check if the current CU depth is maximum and the current CU size is not 8x8 when inter_4x4_enabled_flag is zero. If the conditions are true, proceed to 6. Otherwise, proceed to 7.

- Jmode for MODE_INTER with PART_NxN is evaluated and J is set equal to Jmode if Jmode < J. After that, check if MODE_INTER with PART_NxN contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_NxN. Otherwise, proceed to 7.

- Jmode for MODE_INTER with PART_Nx2N is evaluated and J is set equal to Jmode if Jmode < J. After that, check if MODE_INTER with PART_Nx2N contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_Nx2N. Otherwise, proceed to 8.

- Jmode  for MODE_INTER with PART_2NxN is evaluated and J is set equal to Jmode if Jmode < J. After that, check if MODE_INTER with PART_2NxN contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_2NxN. Otherwise, proceed to 9.

- Invoke a process to determine AMP mode evaluation procedure specified in 4.8.2.3. Output of this process is assigned to TestAMP_Hor and TestAMP_Ver. TestAMP_Hor specifies whether horizontal AMP modes are tested with specific ME or tested with merge mode or not tested. TestAMP_Ver specifies whether vertical AMP modes are tested with specific ME or tested with merge mode or not tested.

- If TestAMP_Hor indicates that horizontal AMP modes are tested, MODE_INTER with PART_2NxnU is evaluated with procedure suggested by TestAMP_Hor and J is set equal to the resulting coding cost Jmode if Jmode < J. After that, check if MODE_INTER with PART_2NxnU contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_2NxnU. Otherwise, MODE_INTER with PART_2NxnD is evaluated with procedure suggested by TestAMP_Hor and J is set equal to the resulting coding cost Jmode if Jmode < J. After that, check if MODE_INTER with PART_2NxnD contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_2NxnD. Otherwise, proceed to 11.

- If TestAMP_Ver indicates that vertical AMP modes are tested, MODE_INTER with PART_nLx2N is evaluated with procedure suggested by TestAMP_Ver and J is set equal to the resulting coding cost Jmode if Jmode < J. After that, check if MODE_INTER with PART_nLx2N contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_nLx2N. Otherwise, MODE_INTER with PART_nRx2N is evaluated with procedure suggested by TestAMP_Ver and J is set equal to the resulting coding cost Jmode if Jmode < J. After that, check if MODE_INTER with PART_nRx2N contains no non-zero transform coefficients (CBF_Fast condition). If the condition is true, proceed to 17 with setting the best interim coding mode as MODE_INTER with PART_nRx2N. Otherwise, proceed to 12.

- MODE_INTRA with PART_2Nx2N is evaluated by invoking the process specified in 4.8.1, only when at least one or more non-zero transform coefficients can be found by using the best interim coding mode. J is set equal to the resulting coding cost Jmode if Jmode < J.

- Check if the current CU depth is maximum, If the condition is true, proceed to 14. Otherwise, proceed to 15.

- MODE_INTRA with PART_NxN is evaluated by invoking the process specified in 4.8.1, only when the current CU size is larger than minimum TU size. The resulting coding cost Jmode is set to J if Jmode < J.

- Check if the current CU size is greater than or equal to the minimum PCM mode size specified by the log2_min_pcm_coding_block_size_minus3 value of SPS parameter. If the condition is true, proceed to 16. Otherwise, proceed to 17.

- Check if any of the following conditions are true. If the condition is true, PCM mode is evaluated and J is set equal to the resulting coding cost Jmode if Jmode < J.

- Bit cost of J is greater than that of the PCM sample data of the input image block.

- J is greater than bit cost of the PCM sample data of the input image block multiplied by λmode.

- Update bit cost Bmode by adding bits for CU split flag and re-compute minimum coding cost J.

- Check if the best interim coding mode is MODE_SKIP (Early_CU condition). If the condition is true, do not proceed to the recursive mode decision at next CU level. Otherwise, go to next CU level of recursive mode decision if the current CU depth is not maximum.

Figure 4-29 The schematic of Intra/Inter/PCM mode decision

For the computation of Jmode except for PCM mode, residual signal is obtained by subtracting intra or inter prediction samples from source samples and is coded with transform and quantization with quad-tree TU partitioning as specified in the section 4.9. Bits for side information (skip_flag, merge_flag, merge_idx, pred_type, pcm_flag, inter_pred_flag, reference picture indices, motion vector(s), mvp_idx, intra prediction mode signaling) and residual coded data are considered as Bmode. SSEluma and SSEchroma are obtained by using local decoded samples, except for MODE_SKIP case where prediction sample is used as local decoded samples.

For the computation of Jmode for PCM mode, bits for side information (skip_flag, pred_type, pcm_flag, pcm_alignment_zero_bit) and PCM sample data are considered as Bmode. SSEluma and SSEchroma are set to 0. (Note that in current test conditions, the PCM mode decision processes in (15) and (16) are skipped since the minimum PCM mode size is 128.)

This CU level mode decision is recursively performed for each CU depth and final distribution of CU coding modes is determined at LCU level.

## Derivation process for TU-level coding parameters

### Residual Quad-tree partitioning

[Editor’s note: Non-normative description of C311 & C319 should be summarized here.]

### Rate-Distortion Optimized Quantization

[Editor’s note: This section described RDOQ applied to PIPE. Additional description for RDOQ applied to VLC may be considered. Similarly this section does not describe RDOQ applied to chroma.]

[Editor’s note: PIPE related description needs to be removed. Does the current description fit to software implementation for other aspects?]

The basic idea behind trellis-based rate distortion optimized quantization (RDOQ) is to perform a soft decision quantization for a given coefficient given both its impact on the bitrate and quality. In the HM, RDOQ is applied to PIPE in a similar manner as it was applied to CABAC in H264/AVC.

To estimate the number of bits required to code a coefficients, we use the tabularized values of entropy of the probabilities corresponding to states in PIPE coding engine. But, the PIPE algorithm uses the context modeling (PIPE context model is currently the same as in CABAC). That is, the coding of current coefficient in a block is related to the state of previous coded coefficients.

The RDO-Q is closely related to the context modeling for residual coding. Residual coding by PIPE includes two parts, i.e., coding a so-called significance map and coding non-zero coefficients. Given a zigzag ordered sequence of transform coefficients, its significance map is a binary sequence which indicates the occurrence and location of the non-zero coefficients. The context modeling for coding the significance map is associated with the zig-zag order, and is easy to be included in RDO-Q. The context modeling for coding non-zero coefficients, however, is complicated. For a given sequence, there are in total 10 contexts for coding non-zero coefficients, with 5 of them for coding the first bit of a binary representation and the other 5 dedicated to coding the second to 14th bits. Briefly, contexts are selected as follows,

1. Scan the sequence in the inverse order to initiate two parameters as NumLg1 and NumEq1. NumLg1 is the number of coefficients that are greater than 1 while NumEq1 accords to those equal to one.

2. The context for the first bit is determined by

(12)

3. The context for the 2-14th bits is selected by

(13)

There is also a bypass mode with a fixed distribution. Other bits in the binary representation use the bypass mode.

Figure 4-30 – Graph structure for RDO quantization based on CABAC in H.264/AVC

In order to solve the minimization problem, a graph-based algorithm is used to address the computation of the rate function R(.) of PIPE. As shown in Figure 4-30, the graph is constructed based on coding features of PIPE. Basically, states are defined based on the context model selection. Thus, states are named by values of NumEq1and NumLg1, as of NumEq1_NumLg1 , e.g., 2_0 accords to NumEq1 = 2 and NumLg1 = 0. When NumLg1 > 0, the context is irrelevant to NumEq1. Thus, there are three states as X_1, X_2, and X_3. The context is fixed for allNumLg1 ≥4. Accordingly, one state X_X is defined. For a 4×4 luma block, there are 16 columns, each of them corresponding to one coefficient. In each column there are up to 8 states. Transitions are established between states according to the increase of NumEq1 and NumLg1 , e.g., the state 1_0 is connected to 1_0, 2_0 and X_1according to a quantization output of 0, 1, or greater than 1, respectively. In case that the quantization output is greater than 1, parallel transitions are established so that each accords to a unique value. In practice, because the distortion is a quadratic function with respect to the quantization output, it is sufficient to investigate only a few parallel transitions. Thus the complexity is greatly reduced without sacrificing the optimality. Finally, a graph structure as shown in Figure 4-30 is obtained.

The optimal RDO quantization design now becomes a problem to search for a path in the graph for the minimal RD cost. It is not hard to see that the above graph design allows an element-wise additive computation of the RD cost. The Viterbi algorithm is then used to do the search, which leads to the solution of the minimization problem.

To avoid searching within a full graph, a simplified RD optimized quantization (RDO-Q) scheme is applied:

In RDO-Q, assuming the transform coefficients before quantization areci, (i=0,…,M-1),  then the quantized coefficients/levels li (i=0,…,M-1) are calculated as follows:

For a given coefficient position k, k=M-1,…,0, assume that coefficientck is the last significant coefficient in the block:

For each coefficientci, i=k-1,…,0, calculate its Lagrangian cost when the quantized value li is equal to 0, lfloor and lceil. The Lagrangian cost Jk,iλ,li when coefficient ci is quantized to li is calculated as:

Jk,iλ,li=errci,li+λ⋅bitsli

where errci,li is the quantization error if the coefficient ci is quantized to valueli and bitsli is the number of bits needed to code li. The value of lfloor and lceilare defined as (4x4 block as an example):

lfloor=floor(|ci|⋅QQP%6,i215+QP6)

lceil=lfloor+1.

Let the final quantized level li,opt=argminliJk,iλ,li and update Lagrangian cost Jkλ using Jk,iλ,li,opt.

The final quantized vector of quantized coefficients lopt=argminkJk(λ).

Figure 4-31 – Possible quantized values in RDO-Q

To speed up the algorithm the following simplifications are made:

For each coefficient cij at most 3 possible values of level lij are tested: 0, lijfloor and lijceil. For 4x4 block values lijfloorand lijceil are calculated as follows:

lijfloat=|cij|⋅QQP%6,i,j215+QP6,

lijfloor=floorlijfloat,

lijceil=lijfloor+1.

To reduce complexity if coefficient cij is closer (as measured by for example absolute distance between lijfloat and lijflooror lijceil) to lijfloor than to lijceil only value lijfloor is considered.  If cij is closer to level 0 than 1 it is assigned level 0 without any further analysis.

Let us assume that the coefficients in a given block are ordered using zigzag scanning, resulting in a one-dimensional ordered coefficient vector. We will denote the ordered coefficients asci, where i=0,...,M .

The quantized coefficients liare obtained in two passes. In the first pass it is determined which coefficient should be the last non-zero coefficient; this coefficient will be denoted as ck. In the second pass we find final values of quantized coefficients assuming that coefficientck is the last nonzero coefficient in the block.

Only coefficients ci0,…,ci1 are considered in the first pass, where

i1 is the largest value of i for which lifloat>0.5.

i0 is the largest value of i for which lifloat>1. If such index does not exist i0=0.

To reduce complexity value of Jsum is pre-calculated as sum of quantization errors errci,li when coefficientsci,i=i0,...,i1, are quantized to li=0:

Jsum=i=i0i1errci,0.

The first pass consists of following steps:

If for coefficient ci0, li0float1.5:

Update value of Jsum:

Jsum=Jsum−errci0,0.

The approximation of RD Cost Ji0λ of the block when ci0is the last nonzero coefficient is calculated as:

Ji0λ=Jsum+λ⋅bitslast=1−bitslast=0.

bitslast=1 is the approximate number of bits required to indicate that ci0 is the last non-zero coefficient (i.e., number of bits to encode last_significant_coeff_flag equal to 1). bitslast=0 is approximate the number of bits required to indicate that ci0 is not the last non-zero coefficient (i.e., number of bits to encode last_significant_coeff_flag equal to 0).

The starting index i0 is updated to i0=i0+1.

For coefficients ciwhere i=i0,...,i1:

Update value of Jsum:

Jsum=Jsum−errci,0.

Find RD Cost Jλ,ci,0 when ci is quantized to 0.

If coefficients ciis closer to 1 than to 0 calculate two additional values of RD Cost:

Jlast=0λ,ci,1 - ci is quantized to 1 and is not the last nonzero coefficient,

Jlast=1λ,ci,1 - ciis quantized to 1 and is the last nonzero coefficient.

The approximation of RD Cost Jiλ when ci is the last nonzero coefficient is:

Jiλ=Jsum+Jlast=1λ,ci,1

When calculating bits needed to code value of equal to 1 we fix context for greater_than_1 symbol to be one of the 5 possible values for all coefficients.

Update value of Jsum:

Jsum=Jsum+minJλ,ci,0,Jlast=0λ,ci,1

The coefficient ck with the smallest corresponding value of Jkλ is assumed to be the last nonzero coefficient in the block.

In the second pass we find final values of quantized coefficients assuming that coefficient ck selected in the first pass is the last nonzero coefficient in the block. For each coefficient ci, i=k,…,0  we find value of level li for which RD Cost J(λ, ci, li) is minimized. As described previously at most 3 different values of level li are considered: 0, lifloor and liceil. To calculate context when encoding value of level li , already selected values for levels lj, j=k,…,i-1 are used.

# References

- JCT-VC, “Encoder-side description of Test Model under Consideration”, JCTVC-B204, JCT-VC Meeting, Geneva, July 2010

- JCT-VC, “High Efficiency Video Coding (HEVC) Test Model 1 (HM 1) Encoder Description”, JCTVC-C402, October 2010

- JCT-VC, “High Efficiency Video Coding (HEVC) Test Model 2 (HM 2) Encoder Description”, JCTVC-D502, January 2011

- JCT-VC, “High Efficiency Video Coding (HEVC) Test Model 3 (HM 3) Encoder Description”, JCTVC-E602, March 2011

- JCT-VC, “High Efficiency Video Coding (HEVC) Test Model 4 (HM 4) Encoder Description”, JCTVC-F802, July 2011

- JCT-VC, “High Efficiency Video Coding (HEVC) Test Model 5 (HM 5) Encoder Description”, JCTVC-G1102, November  2011

- JCT-VC, “High Efficiency Video Coding (HEVC) Test Model 6 (HM 6) Encoder Description”, JCTVC-H1002, February 2012

- JCT-VC, “High Efficiency Video Coding (HEVC) text specification draft 6”, JCTVC-H1003, February 2012

- JCT-VC, “Common test conditions”, JCTVC-H1100, February  2012

- ITU-T Recommendation H.264 / ISO/IEC 14496-10: "Information technology - Coding of audio-visual objects- Part 10: Advanced Video Coding"

- JCT-VC, “CE2: Test result of asymmetric motion partition (AMP)”, JCTVC-F379, July 2011

- JCT-VC, "Early Termination of CU Encoding to Reduce HEVC Complexity", JCTVC-F045, July 2011

- JCT-VC, "Coding tree pruning based CU early termination", JCTVC-F092, July 2011

- JCT-VC," Early skip detection for HEVC ", JCTVC-G543, November 2011