| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0390 |
| --- | --- |

| Title: | AHG6: Further cleanups and simplifications of the ALF in JCTVC-J0048 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Tomoo Yamakage, Takayuki Itoh, Takeshi Chujoh TOSHIBA 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai, chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Tomoo Yamakage +81-44-549-2152 {tomoo.yamakage, takayuki2.itoh, takeshi.chujoh} @toshiba.co.jp Marta Karczewicz +1-858-658-5673 {ichong, martak} @qualcomm.com |
| Source: | MediaTek Inc., TOSHIBA Corporation, Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, further cleanups and simplifications of the non-normatively improved ALF in JCTVC-J0048 are developed according to suggested directions on the JCT-VC email reflector. Three software packages are provided.

In JCTVC-J0048, compared with ALF-off, the ALF coding gain is 2.9% (Y) / 4.0% (Cb) / 4.0% (Cr) for common test condition (CTC) sequences when Main-AI/RA/LB/LP and HE10-AI/RA/LB/LP are averaged and class F is excluded. For key technical area (KTA) sequences (i.e., 15 720p and 10 1080p sequences that were commonly seen during the KTA study period), the ALF coding gain is 5.2% (Y) / 5.4% (Cb) /5.3% (Cr).

The first software is on top of the JCTVC-J0048 software. Reduction of the filter coefficient precision from 9-bit to 7-bit, reduction of filter shape from cross9x7+square3x3 to cross7x7+square3x3, a fix of RDO for considering a previous APS, and code cleanups are added. The ALF coding gain is 2.8% (Y) / 4.0% (Cb) / 4.0% (Cr) for CTC sequences and 5.0% (Y) / 5.0% (Cb) / 4.6% (Cr) for KTA sequences.

The second software is on top of the first software. When samples are 8-bit, for achieving 16-bit accumulation, filter coefficients are normatively constrained on the encoder side as follows: Sum of positive filter coefficients times 255 shall be in the range of [0, 215-1-32), and sum of negative filter coefficients times 255 shall be in the range of [-215, 0). The ALF coding gain is 2.8% (Y) / 4.0% (Cb) / 4.0% (Cr) for CTC sequences and 5.0% (Y) / 5.0% (Cb) / 4.7% (Cr) for KTA sequences. For CTC sequences, the encoding time increase is 1%, while the decoding time increase is 7%. With typical SIMD instructions, the decoding time increase can be reduced to 3% for Main conditions.

The third software further includes cleanups and fixes for APS in JCTVC-J0047, changing the filter coefficient codewords from leading ones to leading zeros to unify exponential golomb coding of ALF with other parts, and a missing adoption in the meeting notes of the 9th JCT-VC meeting without any contribution number (i.e., to apply virtual boundary processing for the last LCU row in order to unify the ALF process for the last LCU row and the non-last LCU rows). Moreover, it is based on HM-7.1 and can be easily reused for developing HM-8.0.

# Introduction

JCTVC-J0048 [1], non-normative encoder-only improvements and the bug-fix of ticket #574 are added on top of the ALF in HM-7.0. After releasing JCTVC-J0048 results on the JCT-VC main reflector, we received suggestions of further cleanups and simplifications. In this contribution, we would like to show results of the suggested cleanups and simplifications as a response and thank the people for guiding us to significantly reduce ALF complexity.

# Proposed methods

We developed three software packages, which are described in the following subsections.

## Software package 1

On top of the JCTVC-J0048 software, four modifications are added as follows.

- Reduction of the filter coefficient precision from 9-bit to 7-bit

- Reduction of filter shape from cross9x7+square3x3 to cross7x7+square3x3

- Fix of RDO for considering a previous APS

- Code cleanups

In item (1), the center coefficient is unsigned 7-bit, and each of the non-center coefficients has 1-bit sign and 6-bit magnitude. The benefit of item (1) is that the multiplier output of ALF (i.e., product of a center coefficient and a center sample, or product of a non-center coefficient and a sum of two samples) can be represented within 16 bits. In item (2), the number of multiplications is reduced from ten to nine. In item (3), the rate calculation is more accurate when a previous APS is considered. In item (4), code cleanups that do not change ALF results are done.

## Software package 2

On top of the software package 1, filter coefficients are normatively constrained on the encoder side as follows.

- Sum of positive filter coefficients times 255 shall be in the range of [0, 215-1-32).

- Sum of negative filter coefficients times 255 shall be in the range of [-215, 0).

In this way, 16-bit accumulation can be achieved for filtering 8-bit samples in Main conditions.

## Software package 3

All the cleanups, fixes, non-normative improvements, and simplifications in JCTVC-J0048 and the previous two software packages are integrated in the software package 3. In addition, the followings are also included.

- Cleanups and fixes for APS in JCTVC-J0047

- Unifying exponential golomb coding of ALF with other parts by using leading zeros

- Applying virtual boundary processing for the last LCU row (a missing adoption in software and text)

What is more, the software package 3 is based on HM-7.1, where the ALF part can be easily reused for developing HM-8.0.

# Simulation results

Simulations were conducted according to the common test conditions (CTC) defined in JCTVC-I1100 [2] with ALF-off as the anchor. Results of using the current picture to derive filter coefficients are shown.

Table 1 shows the BD-rates of different ALF software packages using CTC sequences. In JCTVC-0048, the ALF coding gain is 2.9% (Y) / 4.0% (Cb) / 4.0% (Cr). In the software package 1, the ALF coding gain is 2.8% (Y) / 4.0% (Cb) / 4.0% (Cr). In the software package 2, the ALF coding gain is 2.8% (Y) / 4.0% (Cb) / 4.0% (Cr). Please note the software package 2 does not change results of the software package 1 in Main conditions. In the software package 3, the ALF coding gain is x.x% (Y) / x.x% (Cb) / x.x% (Cr).

Table 1.  BD-rates of different ALF software packages using CTC sequences

Table 2 shows the BD-rates of different ALF software packages using KTA sequences. In JCTVC-0048, the ALF coding gain is 5.2% (Y) / 5.4% (Cb) / 5.3% (Cr). In the software package 1, the ALF coding gain is 5.0% (Y) / 5.0% (Cb) / 4.6% (Cr). In the software package 2, the ALF coding gain is 5.0% (Y) / 5.0% (Cb) / 4.6% (Cr). In the software package 3, the ALF coding gain is 5.0% (Y) / 5.0% (Cb) / 4.7% (Cr).

Table 2.  BD-rates of different ALF software packages using KTA sequences

Accurate run times are also measured for software package 2. For CTC sequences, the encoding time increase is 1%, while the decoding time increase is 7%. With typical SIMD instructions, the decoding time increase can be reduced to 3% for Main conditions (1.9%/1.8%/3.8%/3.7% for Main-AI/RA/LB/LP). For KTA sequences, the encoding time increase is 1%, while the decoding time increase is 10%.

# Conclusions

In this contribution, further cleanups and simplifications of the non-normatively improved ALF in JCTVC-J0048 are proposed. Additional suggested normative changes include reduction of the filter coefficient precision from 9-bit to 7-bit, reduction of filter shape from cross9x7+square3x3 to cross7x7+square3x3, a constraint on sum of positive filter coefficients, and a constraint on sum of negative filter coefficients. With these changes, the multiplier output can be represented within 16 bits, and the 16-bit accumulation is also doable. According to the simulation results, these changes cause very minor coding efficiency loss, while they are very helpful for SIMD instructions to speed up ALF.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, T. Yamakage, T. Itoh, T. Chujoh, I. S. Chong, M. Karczewicz, “AHG6: ALF with non-normative encoder-only improvements,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0048, July, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.